DWHost v2.0

(c) Copyright 1986 - 1991 by J.C. Kilday Associates

Developed at the Northern Lights BBS

207-766-2467   (v.32bis, v.32, HST)

Table of Contents

-----------------

Introduction ....................  1

Description .....................  1

Advanced Answering Modes ........  2

Installation ....................  3

Additional Setup Notes ..........  4

Operating DW Host ...............  6

Using New Features in v2.0 ......  7

Support .........................  8

DW Host v2.0                        Page 1

Introduction

------------

DW Host  is  a  specialized  host  communications  program   primarily

intended to support operations at the  DOS level on the host.   It may

be interfaced  with other  communications packages  to implement  more

complex functions.   It is intended for operation with Doorway  (which

allows nearly  any non-graphics  program to be run from remote).  File

transfer  operations are  supported directly  from DOS by DWXFER as is

mentioned  in  the  next  section.    Popular  applications of DW Host

include  home-to-office  (and  vice  versa)  communications as well as

operations  between  a  company  home  office  and  its  remote sites.

Software configuration  and file  maintenance on  the DW Host-equipped

host are frequent uses.  DW Host  is designed for operation on an  IBM

PC, XT, AT, PS/2 or a close compatible connected to a Hayes-compatible

modem or a US Robotics HST.

The complete DW Host package includes the following files:

DWHost.DOC   - this file

DWHost.COM   - executable module

DWHost.OVR   - DW Host "overlay" file

CONFIG.HLP   - context-sensitive help file

HOST.BAT     - optional controlling batch file

Description

-----------

DW Host normally monitors the modem  and upon detecting a ring  answers

the call while adjusting to the  caller's baud rate.  Speeds of  19,200

bps down to 300 bps  are supported with the following  comm parameters:

8 data bits, no parity, 1  stop bit.  Upon successful connect,  DW Host

greets the caller with a one-line message of your choosing and requests

entry of a password, also of your choosing.  If the correct password is

supplied and the callback mode is not in effect, DW Host will  transfer

control  of  the  next  part  of  the  communications  session to other

software.  Depending on the configuration option chosen, the caller may

be dropped  to the  DOS level  (with a DOORWAY command line having been

invoked to support continued communications).  Or a caller may next see

another communications application,  such as a  BBS-like package, or  a

menu of programs that might be run (using the Doorway munu capability).

If  the correct  password is  not given  to DW Host  in three attempts,

DW Host hangs up and recycles.

For enhanced security against unauthorized access the callback feature

is available.   If  the callback  mode is  in effect  when the correct

password is given, rather than  give the caller immediate access,  the

program  disconnects  the  caller  and  dials  the  telephone   number

specified in the configuration data.  Access is granted only if  there

is  a  successful  connect  at  that  number.    DW Host makes up to 3

attempts  to  establish  a  connect  (it  calls  a 2nd and 3rd time at

intervals  of  1  minute  if  no  answer,  a  busy signal, or no modem

connect).  If no  connection is made on  the 3rd attempt, the  program

DW Host     	                        Page 2

recycles and sets up to await another call.

If access is  granted to the  DOS level on  the host machine  (through

DOORWAY), virtually all DOS commands  may be executed and so  may most

other programs which do not use bit mapped graphics.  A program called

DWXFER  supports file  transfers while in DOS.  It is included in this

ZIP.  You might also like to try Chuck Forsberg's DSZ  for Zmodem file

transfers, rather than the older protocol provided by DWXFER (although

DWXFER is simpler to use).

A log of call activity (a file called DW.LOG) is created by DW Host so

that any attempts of unauthorized access to your system may be  noted.

Calls  accepted  (and   normal  logoffs),  rejected   calls  (password

failures), and other  connects not resulting  in access to  the system

(hang  ups  at  the  password  prompt  and  3-minute timeouts awaiting

password entry) are all recorded in this text file.  The file includes

the date and time of each call  and how each ended.  It can  be viewed

using the the DOS TYPE command.  It may be deleted at any time  (using

DOS ERASE or DEL)  and the program will  start a new one  whenever the

next entry is to be made.

When DW Host  is awaiting  a call,  a display  including the  date and

time, some key setup information, and present DW Host status is shown.

A screen blanker turns off the screen if there is no keyboard activity

within a 2-minute span.  Striking  any key restores the display.   The

display  also  is  toggled  on  and  off  by hitting F9.  F10 presents

information about the program.  F3 allows access to the  configuration

screen where various DW Host setup options may be specified.  F1 or F2

cause DW Host to terminate.   F1 leaves the  telephone line in a  BUSY

state (modem offhook) while F2 leaves the modem onhook.

Advanced Answering Modes

------------------------

DW Host normally is set  up to answer a  call on the first  ring.  But

where there is need  for the host computer  to share a phone  line and

not intrude on voice calls, special answering modes may be configured.

"Long Ring" may be configured such that DW Host answers only after the

specified number  of rings  have been  received.  Alternatively, "Ring

Back" may be chosen.  This mode requires a caller to dial twice before

DW Host answers.   The first  call consists  of a  specified number of

rings (or DW Host can be configured for a non-specific number of rings

on the first call).  DW Host answers a second call if received  within

one minute of the first that lasted the proscribed number of rings.

Still another  answering mode,  Triggered Callback,  may be  specified

where DW Host never answers but, instead, places a call to the  number

with which it had been  configured.  Almost a combination of  callback

and Ring Back, Triggered Callback is set up by two calls much the same

as Ring Back.  The first call  must be of a specified number  of rings

(or a non-specific number) and  a second call must be  received within

DW Host v2.0                        Page 3

Advanced Answering Modes (Continued)

------------------------------------

one minute of the first.  If so "triggered", DW Host goes offhook  and

dials out.   Upon connecting  to the  called computer,  a password  is

required before the  called party is  allowed access.  Just  as in the

callback process described earlier, 3 attempts at a successful connect

are made.

Installation

------------

DW Host is designed  to operate  under DOS  3.0, and  up, on IBM PC's,

XT's, AT's and compatibles equipped with Hayes 1200-compatible modems.

If  DW Host has been  configured to  answer on true  ring detect,  the

modem cable used must have pin 22 (ring indicate) connected.  Also pin

20  (Data Transmitter  Ready signal)  must be  connected and the modem

must be capable of providing  result messages indicating baud rate  at

connect time (e.g., CONNECT 1200).   Recommended Hayes switch settings

are:

SW 1 - Up, DTR normal (pin 20 controlled by program)

SW 2 - Up, Verbal result messages sent (not numeric result codes)

SW 3 - Dn, Result messages sent (not quiet mode)

SW 4 - Dn, Commands not echoed back

SW 5 - Dn, Auto Answer suppressed (controlled by program)

SW 6 - Up, Carrier detect indicated by modem (not always on)

SW 7 - Up, Single phone line connection

SW 8 - Dn, Normal "AT" command set enabled

If you are using a modem other than the Hayes standard, make sure that

its basic  configurable functions  are set  up the  same as above.  If

your  modem  does  not  have  switches  for  one  or more of the above

functions, there should be appropriate AT commands to achieve the same

state.  Please consult your modem owner's manual.

DW Host  has  been  improved  so  that  the  complication  of   having

controlling batch  files is  no longer  a requirement.   There  may be

circumstances, however, that may require you to use a batch file (like

HOST.BAT,  included  in  the  distribution  archive), but more on that

later.    To  install  the  software  you  need only copy  DWHost.COM,

DWHost .OVR, and CONFIG.HLP to a directory from which it will be  run.

The directory you choose should be the default directory when  DW Host

is invoked.  This will already be done if you run the INSTALL program.

To configure DW Host, run it for the first time in LOCAL mode so  that

it won't look for your modem and attempt to initialize it.  Enter  the

command:    DWHost L . The  L command  line parameter specifies LOCAL

mode.    Initially,  the  file  DWCONFIG  cannot  be found, you are so

informed, and the configuration/setup screen is presented with default

settings.  If you take some time  with this screen and use the F1  key

to display the available help for each configuration item, you'll have

a good understanding of DW Host capabilities when you finish.  Hit the

F10 key to save the configuration information.  After DW Host  returns

to its status screen, hit F1 or F2 to quit operations in LOCAL mode.

DW Host v2.0                        Page 4

Installation (Continued)

------------------------

If  all  necessary  configuration  information  has been specified and

you've set up whatever batch  files and/or other programs that  are to

be accessed from remote through DW Host, your're ready to try bringing

up  DW Host  for  its  first  test  run.   The command DWHost  with no

command line parameters should be tried next.  If the code displays  a

status screen  and "Waiting  for a  call" in  the status field, you're

ready to try calling in.

As indicated on the screen, you may exit DW Host by pressing F1 or  F2

(choose F1 if you wish to have the line ring BUSY to anyone calling).

DWCONFIG is a straight ASCII file and can be modified using any  ASCII

editor so long as its lines are not interchanged and the limits of the

variables (as specified by the original prompts for the data) are  not

exceeded.  It  may be necessary  to modify this  file from remote,  in

which case the DOS EDLIN, or your favorite line editor may be used.

Additional Setup Notes

----------------------

Near the bottom of the  configuration screen is a parameter  headed by:

"Program name  of executable  to be  run ...".   If  this item  is left

empty, DW Host exits  to DOS upon  successful entry of  a password from

remote.  Therefore, proper operation  of DW Host requires that a  batch

file such  as HOST.BAT  be used  as a  controlling batch  file when the

"program name to  execute" is left  blank on the  configuration screen.

If you study HOST.BAT,  you will see how  DW Host is controlled and  is

not permitted to drop to DOS without invoking a communications  program

or DOORWAY.

Another way  to set up  DW Host is  to  enable it to  "shell out" to a

another program to be executed  when a caller successfully enters  the

password.  This can be done by specifying the program name to execute,

perhaps as in the following example:

doorway COM1 /g:on /m:100 /O: /S:* /v:d^U /R:^X /c:dos

(causes Doorway to operate using COM1, allowing the caller

100 minutes of time after dropping to DOS.  Additionally it

would allow the running of full-screen, direct screen-writing

applications from remote, such as Lotus, word processors, Kedit,

Brief, Vern Buerg's LIST, etc. )

The above setup allows you to  invoke DW Host on the host computer  by

merely the command DWHost.  When a caller enters the correct password,

a shell  to do_drwy.bat is invoked  and the simple  batch file invokes

DOORWAY,  which in turn invokes  COMMAND.COM.  Operations  at the  DOS

level  from remote  are  sustained  until the  caller gives  the  EXIT

command.   Upon the EXIT,  control is returned directly to DW Host and

the code recycles internally to set up for another call.

DW Host v2.0                        Page 5

Additional Setup Notes (Continued)

----------------------------------

This setup also allows DW Host to remain resident during the  "shell."

This increases memory usage,  since both DOORWAY  and DW Host  will be

resident  at  the  same time.  For maximum  memory availability  it is

recommended to use the DROP to BATCH FILE method.  If INSTALL  was run

then the DROP to BATC?H FILE method will be used.

The above shows how DW Host  may be set  up to operate in  conjunction

with  other software  without using  batch files.  The advantages  are

simplicity and speed of recycling after a call.  The only disadvantage

to this kind of setup is that, in the worst case,  about 35K of memory

is  required  for DW Host during the shell.  If  you cannot afford the

memory  utilization,  a controlling  batch file  such as  HOST.BAT  is

recommended.

Operation within HOST.BAT leaves  more of system memory  available for

applications after DW Host has answered the call and verified  correct

password entry.  For this  reason, operation with a controlling  batch

file such as HOST.BAT remains an available option.

If you  use the  batch file  approach to  control of  DW Host, include

HOST.BAT in the directory where you have installed DW Host.  Modify it

to suit your environment and  setup as appropriate.  For  example, you

may wish  to change  "DOORWAY COM1"  to  "DOORWAY COM2"  if  you  have

specified use of COM2 on the DW Host configuration screen.

DW Host v2.0                        Page 6

Operating DW Host

-----------------

If the DW Host drive/directory is the current default, and your  modem

is turned on, simply enter the following command to run:

DWHost    (or HOST if employing batch file)

The complete syntax of DW Host is as follows:

DWHost  [ L|[ C|T ]   [ S ] ]  [ O ]

The C and T are optional parameters which invoke the callback mode  or

triggered  callback  mode  (but  only  if  the  callback mode had been

configured during the setup of DWCONFIG).  If the S parm is  specified

alone or  appended to  another parm  such as  C or  T, the "slow" BIOS

screen  write  method  will  be  used  for  writing to the host system

screens.  Otherwise, fast screen writing methods are used.  If running

under a multi-tasker such as Omniview or DoubleDOS, some bleed through

may be noted if  DW Host is operating in  a background partition.   In

such cases, specifying the S parm clears up that problem.  Examples:

DWHost  T   or   DWHost  CS   or DWHost  S

(or  HOST C   or   HOST TS  or  HOST S  if employing batch file)

The L  parm specifies  local mode  (no communication  with a modem) as

mentioned  in  an  earlier  section.    The  O  parm is used only when

employing  the  batch  file  option  to  recycle  the code externally.

DW Host invoked with  the O disconnects  the caller and  exits back to

the batch file.  When used the O parm must be the only command line

parm.

To terminate DW Host operation from the host console, hit F1 or F2. If

a caller is logged on to the system and currently operating at the DOS

level, DOORWAY COM1 or DOORWAY COM2 in effect, a more drastic means is

necessary to terminate DW Host (i.e., the power switch).

As the caller who  has successfully logged on  to the system, you  may

log off  in an  orderly manner  by entering EXIT from any directory.

If  you  are  using  either  of  the  callback  modes and awaiting the

callback, you  may answer  DW Host's call  to your  location in one of

several different ways:

- Set up to receive a call per your modem and/or communications

program documentation, or

DW Host v2.0                        Page 7

Operating DW Host (Continued)

-----------------------------

- If using one of the communications programs which, in terminal mode

before a connect leaves the Hayes compatible modem in command state

(e.g., PibTerm,  Qmodem, or ProComm),  you may type  the command

ATA when the onscreen report of RING is received from your modem,

or

- You may setup your modem before or during the session to

Auto-Answer mode by entering ATS0=1 or including ATS0=1 in the

modem initialization string. If your modem has indicator lights,

the AA light will be lit after that command takes effect.

When operating in the callback  security mode, DW Host makes the  call

using the baud rate established during the initial call into  DW Host.

Upon successful connect at your location, you are immediately  granted

access to the host machine (just as though you had entered the correct

password in normal mode).

When calling out under  "Triggered Callback" operation, the  baud rate

specified in the configuration file DWCONFIG is used.  Upon successful

connect  you  are  presented  the  configurable one-line greeting that

normally is presented to callers  when DW Host answers a call.   Entry

of the correct password is required before dropping to DOS on the host

machine.

Using New Features in v2.0

--------------------------

This section describes some of the new features in DW Host v2.0.

o Extended  Comm Port  Specification -  DW Host now  allows you to use

comm ports  other than  COM1 and  COM2.   If you  specify a  comm port

greater than 2 on the setup screen (F3), you are prompted for the port

address and interrupt (IRQ) number.  This technical detail is required

due to the lack of standardization on COM3 and COM4.

o No  Password Option  - Some  people are  using DW Host  to front-end

applications which provide their own  security.  When used with  these

applications,   the   DW Host   password   entry  requirement  becomes

redundant.   If you  are using  DW Host v2.0  and delete  the password

entry on the setup screen, then  as soon as a connect is  established,

DW Host exits (or invokes the application named on the setup  screen).

The  caller  sees  no  password  entry  prompt  when  no  password  is

configured in the setup screen.

o Modem Activity  Display - As  an aid in  setup trouble-shooting, all

strings sent to the modem AND all modem responses are displayed on the

status  screen  adjacent  to  the  new  label  of  "Modem:".    As new

characters are sent or received from the modem while it is in  command

state,  they  are  inserted  into  the  display at the right, shifting

earlier data to the left in order to keep the same length display.

o F1 or F2 to Quit in Modem Reset Error loop - You may now quit out of

of DW Host by hitting F1 or F2 when the system is re-cycling due to  a

modem  reset  error  (earlier  versions  required  hitting the big red

switch).

DW Host v6.0                        Page 8

New Features in v2.0 (Cont'd)

-----------------------------

o Other enhancements  to DW Host include:   a longer  wait for carrier

detect (allowing some v.32 modems  to achieve a connect); the  initial

modem init string is sent at  low speed to correct problems with  some

modems; proper initialization of the comm base locations is  performed

by DW Host  if the  system's BIOS  has not  done so  according to  the

installed comm ports found; plus other small bug fixes.

DW Host v2.0

DW Host is made available to you as a fully functional package.  It is

distributed as SHAREWARE.  When you register DOORWAY, DWHOST will also

be registered atuomatically.  Check  the  DOORWAY.DOC, or REGISTER.FRM

for information on registering this package.

Support

-------

Support is available to registered  users of DOORWAY through the  main

message  base  of DATA WORLD BBS  which can  be reached  at the number

given below.   Please note  that support  is provided  only for use of

DW Host on equipment for which it was designed (described earlier).

DATA WORLD BBS - 615-966-3574   Hayes ULTRA

PRIVATE REGISTRATION LINE - 615-675-3282   V.32-9600/2400/1200

Note: the private line becomes a support line when you register.

Please note that the triggered  callback does not work on  some phone

exchanges due to  differences in  ring timing if a specific number of

rings is specified.  If this is the case then set the number of rings

to not specified.