The BGI256 driver obtains the basic bit-mapped font by reading in

the system font. The system font is actually obtained from two

locations. The lower 128 characters are read from the BIOS ROM,

which is the font the graphics mode uses when the 8x8 font is

selected. The lower font table is located at the real address

$F000:$FA6E, and is arranged as an array[0..127,0..7] of byte.

Each of the eight bytes in a table entry make up a character

definition, the first byte representing the first scan line of

the character. The system font table is located in ROM, so you

can't change it.

The upper 128 character set is derived by reading the extended

system font, which can be found at the location indicated by the

vector found at interrupt $1F. If the vector at interrupt $1F is

zero, the extended character set is not available. On these

systems you can load the character set by running the

GRAFTABL.COM program that comes with MS-DOS. GRAFTABL.COM loads

the extended character set into memory and points the $1F

interrupt at the table.

Normally on MCGA and VGA display systems the character set is

automatically loaded. Actually, the character set isn't really

"loaded" -- the interrupt vector at location $1F simply points at

the display BIOS ROM where the extended character table is

stored. Once again, the interrupt points at a ROM location, which

means it can't be changed.

Or can it? Remember that you can load the extended character set

as a TSR into RAM and point the $1F interrupt to the character

set using the GRAFTABL.COM program. Having the character set in

RAM means we can change it. When you do, it will displace the

ROM based table that the display card provides with the one that

GRAFTABL provides.)

The BGI256 driver provides hooks which allow a full character

font set to be loaded into memory. It does this by looking at a

font ID in character 127 of the upper font table. If it finds the

ID there, it knows that the modified table has been loaded into

the interrupt vector at $1F, and a full font table can be used.

Note: The BITFNT.ZIP file found in the Compuserve Borland Pascal

library contains modified BGI drivers and a full description on

how they were modified to support the user loadable bit-mapped

fonts.

-----------------------------------------------------------------

The MAKEFONT program shown below will create the needed full font

table. As before, it assumes that you have loaded the extended

character font using the GRAFTABL program where needed.

{Program to build a font definition file}

program MAKEFONT;

uses  Dos;

type  FontChar = array [0..7] of byte;

FontArray = array [0..127] OF FontChar;

const FontFilename = 'FNTIMG.DAT';

FontID : FontChar = ($4D,$49,$4B,$45,$20,$44,$41,$59);

var   LoFont : FontArray absolute $F000:$FA6E;

UpFont : FontArray;

SysFont : ^FontArray;

f : file of FontArray;

begin

assign(f,FontFilename);                   { Create font file }

rewrite(f);

GetIntVec($1F,pointer(SysFont));    { Get upper font address }

UpFont := SysFont^;                 { copy font to local mem }

UpFont^[127] := FontID;            { Insert our font ID mark }

write(f,LoFont);            { Write system font to the disk, }

write(f,UpFont^);               { Followed by the upper font }

close(f);                                   { Close the file }

end.

Within a program you can load the modified font table with the

following procedure:

{procedure for loading the modified character font}

const FontFilename = 'FNTIMG.DAT';

type  FontArray = array [0..255, 0..7] of byte;

var   ExtFont : FontArray;

f : file of FontArray;

procedure LoadFont;

begin

assign(f,FontFilename);  { Read the font table into memory }

reset(f);

read(f,ExtFont);

close(f);

SetIntVec($1F,@ExtFont[128]);   { set the interrupt vector }

end;

Alternately, you can load the font as a TSR with the following

program:

{TSR program for loading the modified character font to memory}

{$M 1000,0,0}

program MGRAFTBL;

const FontFilename = 'FNTIMG.DAT';

type  FontArray = array [0..255, 0..7] of byte;

var   ExtFont : FontArray;

f : file of FontArray;

begin

assign(f,FontFilename);   { Read the font table into memory }

reset(f);

read(f,ExtFont);

close(f);

SetIntVec($1F,@ExtFont[128]);    { set the interrupt vector }

Keep(0);

end;

-----------------------------------------------------------------

The BGI256 driver tests if the vector found at $1F is valid (non-

zero). Next a test is made to see if it is our own modified table

or some other table.  It determines whether it is the modified

table by looking for a signature (4D,49,4B,45) in the first half

of the last font character (#127). If this is a standard system

font, the character would have zeros in it. If the font table is

not our custom table, the driver uses the system font as normal.

This means it uses the lower font found at address $F000:$FA6E,

and assumes the font being pointed to by interrupt $1F is the

upper (extended) font.

If the driver finds the custom font signature, it assumes the

font table being addressed by $1F is the custom font and that the

normal lower system font is to be replaced by the font found in

the half of the extended font table located below the interrupt

vector. Be sure to restore the old interrupt vector when you are

done with it so that other programs will be happy.

A note about loading your own font:

With the older V2.00 BGI drivers, you could reload the font table

address at Int $1f anytime you wanted and the BGI driver would

automatically reflect the new font on the next usage.

In the V3.00 drivers, this is no longer true. To deal with

protected mode, the BGI driver must pre-load the address at the

time the BGI driver is registered. That means that if you change

the address found at INT $1f after the driver has been

registered, the change will not be reflected in the BGI driver.

There are two ways in which a BGI driver can be registered, the

normal method is to call the BGI InitGraph function which

automatically attempts to register the indicated driver. (If no

driver is indicated it will attempt to determine the proper

driver to use.)

Alternately, you can use the RegisterBGIdriver function to

specifically register a driver that you have loaded into memory

either via Linking it into the code, or via loading it on the

heap (or elsewhere in memory).

The act of registering the driver updates the various registers

in the driver. This is true for both protected mode and real mode

operation.

Keep in mind that while the memory address is fixed, the contents

of the memory can still be changed. Thus to change the font, you

only need to change the contents in the memory where you loaded

the font table. The only limitation is that you cannot move the

table to a different memory address once the driver has been

registered.

Also keep in mind that the BGI driver expects the font to appear

in real memory. This means that if you are writing a protected

mode application, you must use the GlobalDOSAlloc system call to

allocate memory that can be shared by both protected and real

mode in which to place the font table if you intend to load it

inside your application. If you use a TSR to load the font before

you run your application rather than loading it inside your

application, you don't need to worry about that.

Additionally, remember that when in protected mode, you are in a

virtual environment. That means that the $1f interrupt vector

that you set with SetIntVec is not the real mode interrupt

vector, but rather a simulated vector that is isolated from the

real mode interrupt. To change the real $1f interrupt vector

inside your program, you must place a call to the DPMI server to

perform that action.

The program TESTFONT.PAS shows an example of working with the

modified BGI in real and protected mode operation.

<eof>