KSP-HOST (tm)

An Inbound Telnet Server

for

MS/DOS Bulletin Board Systems

Version 1.2

Copyright (C) 1995-96

All Rights Reserved

by

KEY SOFTWARE PRODUCTS

440 Ninth Avenue

Menlo Park, California 94025-1802

BBS/FAX: 415-364-9847

KSP-HOST, KSP-SRVR, KSP-CALL, and KSP-NODE

are trademarks of Key Software Products.

WATTCP is a trademark of Erick Engelke.

Power C and Multi-C are trademarks of Mix Software.

Lantastic is a trademark of Artisoft, Inc.

Novell is a trademark of Novell Corp.

Banyan Vines is a trademark of Banyan Inc.

DESQview is a trademark of Quarterdeck Office Systems.

TABLE OF CONTENTS

CHAPTER 1 - INTRODUCTION        ...........................    1

1.1  Other KSP Software      ...........................    2

1.1.1  KSP Telnet       .............................    2

1.1.2  KSP FTP       ................................    3

1.1.3  KSP SLIP       ...............................    3

1.1.4  KSP Mail       ...............................    3

1.1.5  So Many CD's      ............................    3

CHAPTER 2 - INSTALLING THE NETWORK CONNECTION FIRST    ....    4

2.1  Packet Driver Shim for Novell    ..................    4

2.2  Packet Driver Shim for Novell w/Token-Ring SNAP  ..    5

2.3  Packet Driver Shim for Lantastic    ...............    5

2.3.1  Changes to CONFIG.SYS      ...................    5

2.3.2  Changes to PROTOCOL.INI      .................    6

2.4  Packet Driver Shim for Banyan Vines   .............    7

CHAPTER 3 - THE WATTCP CONFIGURATION FILE     .............    8

3.1  Network Parameters       ..........................    9

3.1.1  The PC's Host Name     .......................    9

3.1.2  The PC's Domain Name     .....................    9

3.1.3  The PC's IP Address     ......................    9

3.1.4  The Name Server's IP Address    ..............   10

3.1.5  The Router's IP Address     ..................   10

3.1.6  The PC's Network Mask     ....................   10

3.2  TCP/IP Parameters (optional)      .................   11

3.2.1  Timeouts        ..............................   11

3.2.2  Maximum Segment Size (MSS)     ...............   11

3.3  Parameters Specific to KSP Host    ................   11

3.3.1  The Log File Directory     ...................   11

3.3.2  Retrieving Hostname of Remote Client    ......   12

3.3.3  Specifying the Fossil Ports     ..............   12

3.3.4  Specifying the Buffer Size     ...............   13

3.3.5  Maximum Number of Rings     ..................   13

3.3.6  Sending a "CONNECT" string to the BBS  .......   13

3.3.7  Specifying Modem Delay      ..................   14

3.3.8  Disabling Automatic Load High     ............   14

CHAPTER 4 - Programs in the KSP Host Package   ............   15

4.1  KSP-NODE (Installed on each BBS Machine)   ........   15

4.1.1  Command Line Parameters      .................   15

4.2  KSP-CALL (Used to Test KSP-NODE)    ...............   16

4.2.1  Command Line Parameters      .................   16

4.3  KSP-SRVR (The Inbound Telnet Server)    ...........   16

4.3.1  Command Line Parameters      .................   16

CHAPTER 5 - SPECIAL CONFIGURATION CONSIDERATIONS      .....   18

5.1  PCBoard        ....................................   18

5.2  DESQview        ...................................   18

TABLE OF CONTENTS

5.3  Novell        .....................................   18

APPENDIX 1 - HOW TO REACH US     ..........................   20

APPENDIX 2 - GETTING UPDATES VIA THE INTERNET    ..........   21

APPENDIX 3 - LEGAL STUFF       ............................   22

Feb 11, 1996           KSP-HOST (tm) v1.2                       1

CHAPTER 1 - INTRODUCTION

KSP-HOST  allows  MS/DOS Bulletin Board Systems to accept inbound

Telnet  connections.   In  addition to being able to contact your

BBS  by  modem,  users  can  now  connect  to  your  BBS over the

Internet using Telnet.

The  name  "KSP-HOST"  actualy  refers  to  a pair of programs: A

program  called  KSP-SRVR  handles  reception  of  inbound Telnet

sessions  and  communicates  with another program called KSP-NODE

running on each of your BBS nodes:

����������ͻ

������������������������������Ķ KSP-SRVR �

�                            �Ķ          �

�      (NetBios)             �Ķ          ������ Internet

�                              �  Telnet  �      (TCP/IP)

�                   ����������Ķ   Host   �

����������ͻ        ����������ͻ     ����������ͼ

� KSP-NODE �  ...   � KSP-NODE �

����������ͼ        ����������ͼ

����������Ŀ        ����������Ŀ

� BBS Prog �  ...   � BBS Prog �

� (Node 1) �        � (Node N) �

������������        ������������

This  arrangement allows Telnet users to use a single Internet IP

address  or  hostname  (that  of the machine running KSP-SRVR) to

contact  your  BBS.  KSP-SRVR then determines which BBS nodes are

available,  and  routes  the  session  to one of those nodes.  As

implied,  KSP-SRVR  can handle multiple telnet connections at the

same time.

The   above   diagram   illustrates   how  things  are  organized

logically;   the   actual   physical   organization  is  somewhat

different:

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                       2

Local Ethernet (Novell, Artisoft, etc.)

����������������������������������������������������������������

�                   �         �             �

����������Ŀ  �     ����������Ŀ  �   ����������Ŀ   ���������Ŀ

� BBS Prog �  �     � BBS Prog �  �   �  Packet  �   � Router  �

� (Node 1) �  �     � (Node N) �  �   �  Driver  �   � E.g.,   �

������������  �     ������������  �   ������������   � KA9Q NET�

�       �           �       �         �        �����������

����������ͻ  �     ����������ͻ  �         �             �

� KSP-NODE ����     � KSP-NODE ����   ����������ͻ        �

����������ͼ        ����������ͼ      � KSP-SRVR �        �

�                   �           �  Telnet  �     Internet

����������Ŀ        ����������Ŀ      �   Host   �    Connection

�  Fossil  �  ...   �  Fossil  �      ����������ͼ     (TCP/IP)

�  Driver  �  ...   �  Driver  �

������������        ������������

�                   �

Modem               Modem

As  indicated,  the  small TSR program KSP-NODE, acts as a switch

so  that  each BBS node can accept either Telnet calls or regular

modem  calls.   This  requires  every  BBS  node  to use a Fossil

driver as shown.

Communication  between KSP-SRVR and KSP-NODE uses NetBios.  I.e.,

KSP-SRVR  is a protocol converter that translates Internet TCP/IP

packets   into  NetBios  packets,  and  vice-versa.   NetBios  is

commonly  supported  by  most local area network software such as

Lantastic or Novell.

The  packet  driver  (or  packet driver shim) is explained in the

next  chapter;  detailed  information  regarding the installation

and  configuration of KSP-SRVR and KSP-NODE appears in subsequent

chapters.

The  router  shown  above  connects  your  local  network  to the

Internet;  a  shareware  program  called  KA9Q NET can serve this

purpose.

1.1 Other KSP Software

Key  Software  Products  offers  a  number  of other products for

BBS's:

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                       3

1.1.1 KSP Telnet

A   door  program  that  allows  callers  to  connect  to  remote

computers  anywhere  on the Internet via your BBS.  Available now

on our BBS.

1.1.2 KSP FTP

A  door  program  that  allows  callers to transfer files to/from

remote computers anywhere on the Internet via your BBS.

1.1.3 KSP SLIP

A  door  program  that  allows callers to run any TCP/IP software

from  home,  including using Mosaic to browse the World Wide Web.

Available now on our BBS.

1.1.4 KSP Mail

A  Multi-Threaded  Server  for  SMTP  Mail  and NNTP Usenet news.

Replaces  UUCP,  its  monthly  fees,  and slow transfer rates! No

more  unwanted  newsgroups!  Instant  mail  without  waiting  for

scheduled  events!   Works  with  any BBS software that presently

uses  UUCP.   Requires a 24hr TCP/IP Internet connection. Can now

receive NNTP news feeds!

1.1.5 So Many CD's

A  PCBoard PPE to handle off-line CD-Roms.  Seamlessly integrated

into  PCBoard.   Users  post requests for off-line files and have

then  returned  as  attachments to messages. Configurable message

pack-out  dates  automatically  keep  your hard disk from getting

cluttered.  Available now on our BBS.

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                       4

CHAPTER 2 - INSTALLING THE NETWORK CONNECTION FIRST

Before installing KSP-SRVR, you must first install:

1.  The network interface hardware.

2.  A corresponding packet driver.

3.  A packet driver shim (if needed).

Detailed  directions for these preliminary steps are available in

separate   documentation   that   comes  with  the  corresponding

hardware or software.

It's  most  common  that multi-node BBS's are interconnected with

Ethernet  and  either  Lantastic or Novell.  Unfortunately, these

two  network  operating  systems  were  designed  using their own

proprietary  protocols  rather than the TCP/IP protocol and their

own  proprietary  software  rather than packet drivers to talk to

their  Ethernet  interface  cards.   However, a piece of software

called  a packet driver "shim" can be used to let both TCP/IP and

their proprietary protocol coexist.

2.1 Packet Driver Shim for Novell

Novell's  network  software is installed in layers as TSRs in the

order  shown  below.   These commands are usually found either in

the  AUTOEXEC.BAT  file  or  in another batch file in a directory

typically called C:\NWCLIENT.

LSL

NE2000  }-- specific to your interface card

IPXODI

VLM

The  packet driver shim (ODIPKT) logically sits on top of IPXODI,

providing   a  packet  driver  interface  for  software  such  as

KSP-SRVR:

LSL

NE2000 +--- Frame Type (0-3)

IPXODI |

ODIPKT 2 97  }--- The packet driver shim

VLM       |

+----- Packet Vector Interrupt (96-127)

(See comment below about hex vs. decimal)

The  ODIPKT  command  line parameters may vary according to which

version  of  the  software  you  have  and  how  your hardware is

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                       5

configured.   The "Frame Type" parameter should correspond to the

position  of  ETHERNET_II  among  the  frame  types  specified in

NET.CFG;  zero  (0)  selects  the  first  frame type, one (1) the

second,  and  so on.  The "Packet Vector Interrupt" number should

correspond  to  an  unused  interrupt  vector.   Note  that older

versions  of  ODIPKT  insist that this number be given in decimal

(96-127)  rather  than  in hex (0x60-0x7F).  The necessary packet

driver  shim can be downloaded from the Key Software Products BBS

as file ODI-SHIM.ZIP.

2.2 Packet Driver Shim for Novell w/Token-Ring SNAP

Another  shim  called  ODITRPKT  exists for Novell that should be

used  if the underlying network is Token-Ring_SNAP.  Installation

is  similar  to  ODIPKT as described above, except that the first

command  line  parameter  must  correspond to the Token-Ring_SNAP

frame  type  in NET.CFG, and starts at "1" rather than "0".  This

shim  can  be  downloaded  from  the Key Software Products BBS as

file TKN-SHIM.ZIP.

2.3 Packet Driver Shim for Lantastic

Using   a   packet  driver  shim  with  Lantastic  requires  that

Lantastic  be  installed  using  NDIS  (Network  Driver Interface

Specification)  Support.  The necessary packet driver shim can be

downloaded   from   the   Key   Software  Products  BBS  as  file

DIS-SHIM.ZIP.

NDIS   allows  you  to  stack  multiple  protocols  on  a  single

adapter.   This  lets  you use multiple protocol drivers (such as

LANtastic  and  TCP/IP)  on  the  same adapter.  You can also use

NDIS  to  include  third-party adapters that have NDIS drivers in

your   LANtastic   network.    Supported  adapter  types  include

Ethernet,  Token-Ring  and ARCNET (R) adapters.  The software and

documentation  necessary  to  add  NDIS  support  to  an existing

Lantastic network is available free of charge from Artisoft.

Once  you  have NDIS installed and working with Lantastic, adding

the  shim is a simple matter of editing PROTOCOL.INI (part of the

NDIS support) and CONFIG.SYS.

2.3.1 Changes to CONFIG.SYS

With  NDIS  installed,  there  will be two device driver lines in

your CONFIG.SYS file that look something like the following:

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                       6

DEVICE=C:\LANTASTI\PROTMAN.DOS /I:C:\LANTASTI

DEVICE=C:\LANTASTI\AEXNDIS.DOS

The  file  listed  in  the  second line may differ if you are not

using  Artisoft's  interface  card; in that case, this file would

typically  be  replaced  by  a  NDIS  driver supplied by the card

manufacturer.

The  packet  driver  shim  itself  is installed as a third device

driver after the first two, as in:

DEVICE=C:\LANTASTI\PROTMAN.DOS /I:C:\LANTASTI

DEVICE=C:\LANTASTI\AEXNDIS.DOS

DEVICE=C:\DRIVERS\DIS_PKT.DOS  }--- The packet driver shim

2.3.2 Changes to PROTOCOL.INI

The  PROTOCOL.INI  file  is  a  text file created (usually in the

C:\LANTASTI  directory) as part of the NDIS installation.  Before

adding  the  packet  driver  shim,  it  typically  looks like the

following,  but  with  the  "iobase"  and  "interrupt" parameters

changed   according   to   your  hardware,  or  with  the  entire

"[AEXNDIS_NIF]"   section  replaced  if  you  are  not  using  an

Artisoft interface card.

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

[AEXNDIS_NIF]

DRIVERNAME = AEXNDS$

IOBASE = 0x300

INTERRUPT = 15

Adding  the packet driver shim requires adding another section to

the PROTOCOL.INI file:

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

[AEXNDIS_NIF]               <---+

DRIVERNAME = AEXNDS$          |

IOBASE = 0x300                |

INTERRUPT = 15                | These names must match!

|

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                       7

[PKTDRV]                        |

DRIVERNAME = PKTDRV$          |

BINDINGS = AEXNDIS_NIF    <---+

INTVEC = 0x61

CHAINVEC = 0x66

NOVELL = Y

Note  that the name "AEXNDIS_NIF" must exactly match the spelling

used  as  the  title of the previous section, "[AEXNDIS_NIF]"; if

you  are not using Artisoft interface cards, then both occurences

will  use  some  other identifier.  The "INTVEC" parameter may be

anything  from  0x60  to 0x80; you may have to experiment to find

an unused interrupt number.

2.4 Packet Driver Shim for Banyan Vines

Although  Key  Software  Products  has  never  used  it, and thus

cannot  offer help on its installation, a packet driver shim does

exist  for  Banyan  Vines  and  can  be  downloaded  from the Key

Software Products BBS as file BAN-SHIM.ZIP.

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                       8

CHAPTER 3 - THE WATTCP CONFIGURATION FILE

In  order  to  run,  KSP-SRVR  and  KSP-NODE  need  to  know some

information  about  your network and BBS configuration, and tries

to   find   this  in  a  configuration  file  called  WATTCP.CFG.

KSP-SRVR  and  KSP-NODE  look in three directories to locate this

file.   First,  they  check  for  an  environment variable called

WATTCP.CFG  that  specifies  the directory.  Second, they look in

the  current  (default)  directory.   Third,  if still not found,

they   look   in  the  directory  that  contains  the  executable

(KSP-SRVR.EXE or KSP-NODE.EXE).

The  following  example  may  be  helpful  for  those  using  the

environment  variable  approach:  If you place WATTCP.CFG in your

PCB  directory,  then  your  AUTOEXEC.BAT file should contain the

following command:

set WATTCP.CFG=C:\PCB

Note that there is no trailing "\" after the directory name!

If  KSP-SRVR or KSP-NODE still can't find the configuration file,

they  will  attempt  to  automatically configure by looking for a

"BOOTP"  server on your network.  Most BBSs will not have a BOOTP

server,  so  we  do not recommend this approach.  We only mention

it  here  because  it  explains  why  you'll get a message saying

"Configuring  through  BOOTP" if it can't find your configuration

file.

The   WATTCP.CFG   configuration  file  is  a  normal  text  file

containing  one  entry  per line.  A sample configuration file is

included  in  this  distribution, but the values MUST be modified

to   suit  your  particular  environment  or  else  KSP-SRVR  and

KSP-NODE will not work!

The syntax of every entry follows the following format:

[ directive = [ "data" | data ] ] [ # comment | ; comment ]

I.e.,  if  a  directive is not followed by data, the directive is

ignored.   Similary,  lines  without directives are ignored.  The

directive is NOT case sensitive; the data IS case sensitive.

e.g.,   netmask=255.255.252.0

domainslist=ksp.com    ; Our domain

Whitespace  is normally removed from data; data containing blanks

must  be  surrounded by quotes.  An unquoted '#' or ';' marks the

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                       9

beginning of a comment.

3.1 Network Parameters

The  following  parameters control how KSP-SRVR communicates with

the rest of the TCP/IP network.

3.1.1 The PC's Host Name

This is the network name of the PC that runs KSP-SRVR.

Example: hostname=bbs

Note  that  the  host  name  does  not  include  the  domain name

suffix.   For  example, the hostname of machine '"bbs.ksp.com" is

simply "bbs".

3.1.2 The PC's Domain Name

This  is  the  network  name  of the subnet to which your PC (and

possibly others) are connected.

Example: domainslist=ksp.com

Note  that  the  domain  name  does  not  include  the  host name

prefix.   For  example, the domain name of machine '"bbs.ksp.com"

is "ksp.com".

3.1.3 The PC's IP Address

This   is  the  unique  IP  address  assigned  to  your  KSP-SRVR

machine.   It  is  the  one  Telnet users will use when trying to

connect  to  your  BBS  through the Internet.  (They may also use

the hostname.domainname format if supported by your nameserver.)

Example: my_ip=100.2.37.4

Note:  As  an  alternative, you may also set the IP address using

an environment variable, as in:

set ksp-ip=100.2.37.4

NOTE:  An  "IP  address"  is  a logical addressing scheme used on

TCP/IP  networks  such  as the Internet.  Each computer connected

to  the  Internet  is  assigned  a unique IP address.  Your local

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      10

network  "guru"  or access provider should be able to provide you

with the IP addresses you need.

IMPORTANT:  The  IP  addresses  given  in  this document are only

examples.   Do  NOT  attempt to use them - they will NOT work and

your network administrator will probably get VERY upset!

3.1.4 The Name Server's IP Address

This  is  the  unique  IP  address  assigned  to  a  network name

nerver.   You  may  specify more than on nameserver by using more

than one "nameserver" line.

Example: nameserver=111.21.108.110

Your  local  network  "guru" or access provider should be able to

provide  you  with the proper IP addresses of appropriate network

name servers.

3.1.5 The Router's IP Address

This is the unique IP address assigned to the network router.

Syntax: gateway = ipaddr [ , subnet [ , subnet_mask ] ]

Examples: gateway=129.97.176.1

gateway=129.97.176.2,129.97.0.0

gateway=129.97.176.2,129.97.0.0,255.255.0.0

Usually  the  (destination)  subnet  and  subnet mask need not be

specified,  and  is  used to create a "default".  The other forms

are  used  to  specify  one or more other gateways for particular

subnets.

Your  local  network  "guru" or access provider should be able to

provide you with the proper IP address of the network router.

3.1.6 The PC's Network Mask

Network  masks  are  used to distinguish destination IP addresses

that  are  on  the  local  subnet  from those that are not.  This

option may not be required, depending on your network topology.

Example: netmask=255.255.254.0

Your  local  network  "guru" or access provider should be able to

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      11

provide you with the proper netmask if needed.

3.2 TCP/IP Parameters (optional)

KSP-SRVR  will  work  without using the following parameters, but

they are provided if you wish to change them.

3.2.1 Timeouts

Most  network  operations (such as establishing a connection to a

remote  host)  have a maximum time before a timeout error occurs.

The  default  value is 30 seconds; a smaller value is unwise, but

larger   values   may   be   necessary   for   particularly   bad

connections.

Example: sockdelay=40

3.2.2 Maximum Segment Size (MSS)

The  default  value  of  MSS  is  1400.  If you know what maximum

segment  size  means  and know what size you want, you can change

it:

Example: mss=512

Note:  Some  Internet  access  providers  configure their

dial-up  slip  and ppp accounts with a very small segment

size.   You  may  need  to  set mss as low as 212 if your

Internet connection is through such a connection.

3.3 Parameters Specific to KSP Host

Two  parameters in the WATTCP.CFG configuration file are specific

to the operation of KSP-SRVR.  Each follows the syntax:

ksp-srvr.<parameter_name>=<value>

3.3.1 The Log File Directory

Syntax: ksp-srvr.log_directory=<directory_spec>

Example: ksp-srvr.log_directory=c:\ksp

Purpose: Specifies  the  directory where the file KSP-SRVR.LOG is

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      12

stored.  Each entry is made using the following format:

<date>,<time>,<client>,<minutes>,<node_id>

Comment: This  parameter  is  optional;  if not specified, no log

file will be created.

3.3.2 Retrieving Hostname of Remote Client

Syntax: ksp-srvr.retrieve_hostnames=<option>

Example: ksp-srvr.retrieve_hostnames=disabled

Purpose: If  left enabled, IP addresses of remote clients will be

translated  into  hostnames using the domain name server

to  provide  a reliable identification.  Otherwise, only

IP  addresses  will  be  used.  Affects both the display

and the log files.

Comment: Default is enabled.

All  remaining  parameters  in  the WATTCP.CFG configuration file

are BBS node-specific.  These parameters follow the syntax:

ksp-node.<parameter_name>=<value>

or:     ksp-node[<id_range>].<parameter_name>=<value>

where  "<id_range>"  is either a single ID number or a pair of ID

numbers  separated  by a dash.  Parameters specified by the first

form  apply  to all BBS nodes; parameters specified by the second

form  apply only to those BBS nodes whose IDs match the specified

range.

Note  that when more than one entry applies to the same node, the

last  entry  prevails; thus the first form can be used to specify

a  default  for all BBS nodes, followed by use of the second form

to specify exceptions.

3.3.3 Specifying the Fossil Ports

Syntax: ksp-node[<node_range>].fossil_ports=<port_range>

Example: ksp-node[11-20].fossil_ports=1

or: ksp-node[5].fossil_ports=0-3

Purpose: Specifies  the  range of Fossil port numbers used by one

or  more BBS nodes.  There should be one fossil port for

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      13

each BBS node running on the same machine.

Comment: Fossil port numbers typically start at 0 (for COM1).

3.3.4 Specifying the Buffer Size

Syntax: ksp-node[<node_range>].buffer_size=<bytes>

Example: ksp-node[11-20].buffer_size=512

Purpose: Specifies  the  buffer size provided by KSP-NODE for one

or more BBS nodes.

Comment: This  parameter is optional.  If not specified, KSP-NODE

will  use  the  same  buffer  size (not "buffer") as the

Fossil driver.

3.3.5 Maximum Number of Rings

Syntax: ksp-node[<id_range>].maxrings=<number>

Example: ksp-node.maxrings=3

ksp-node[5-10].maxrings=0

Purpose: Specifies  the  maximum  number  of rings to send to the

BBS when trying to establish a connection.

Comment: Set  this  value to zero for those nodes which configure

their modems for auto-answer.

3.3.6 Sending a "CONNECT" string to the BBS

Syntax: ksp-node[<id_range>].connect_string=<response>

Example: ksp-node.connect_string="CONNECT 14400/V32/LAPM.V42BIS^M^M"

ksp-node[5-10].connect_string="+FDM^M^MCONNECT 14400^M^M"

Purpose: Specifies  the  connect  string  sent  to the BBS when a

telnet connection has been established.

Note: The  quotation  marks  are  required  immediately after the

equals  sign  and  at the end of the line to enclose the

string   pair   since   the  string  includes  a  blank.

Carriage  return followed by line feed is represented by

the pair of characters, "^M".

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      14

3.3.7 Specifying Modem Delay

Syntax: ksp-node[<node_range>].modem_delay_seconds=<seconds>

Example: ksp-node[11-20].modem_delay_seconds=60

Purpose: It  usually takes several seconds from the moment that a

modem  detects  a  ring before the carrier is negotiated

and  a  connection  is established.  During this period,

the  "carrier  detect"  signal  from  the modem is still

inactive  and  cannot  be  used  to cause inbound Telnet

connections  to  be  refused by that BBS node.  To solve

this  problem,  KSP-NODE ignores all connection requests

from  KSP-SRVR  for  a specified number of seconds after

any  modem  activity  (such  as a ring).  This parameter

specifies that time period for one or more BBS nodes.

Comment: This  parameter is optional.  Default is 30 seconds.  If

the  modem is configured for auto-answer, set this value

for that node to zero.

3.3.8 Disabling Automatic Load High

Syntax: ksp-node[<node_range>].loadhigh=disabled

Example: ksp-node[11-20].loadhigh=disabled

Purpose: KSP-NODE  will  try  to  load into an upper memory block

(UMB)  if possible.  This parameter disables that option

for one or more BBS nodes.

Comment: This parameter is optional.  Default is enabled.

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      15

CHAPTER 4 - Programs in the KSP Host Package

There  are  three  programs  in  the  KSP Host package: KSP-NODE,

KSP-SRVR,  and KSP-CALL.  The purpose and installation of each is

described below.

4.1 KSP-NODE (Installed on each BBS Machine)

KSP-NODE  is  a  small  Terminate and Stay Resident (TSR) program

installed  on one or more BBS machines.  It provides an extension

to  the  services  provided  by  a  Fossil driver, and so must be

installed  after  the  Fossil  driver  and before running the BBS

program.

For  the  duration  of a telnet session, KSP-NODE keeps the modem

off-hook so that node will look busy to modem callers.

KSP-NODE  creates  a  small buffer for each serial port supported

by  the  Fossil  driver.   In  addition  to the buffers, KSP-NODE

needs  about  2,000 bytes of code space.  The size of the buffers

can be specified as desired, but is typically around 512 bytes.

4.1.1 Command Line Parameters

KSP-NODE   requires   one   (and  optionally  two)  command  line

parameters.   The  first  is  a unique ID number assigned to that

machine.   If  you are multitasking several nodes on one machine,

then  a  second  parameter  is  required,  and  together  the two

specify  a  range  of  ID  numbers for all nodes on that machine.

For example:

KSP-NODE  4  8

�  �

�  ��� Last ID#

������ First ID#

You  may  use your regular BBS node numbers for these ID numbers,

although  that  is  not  required.   The only real requirement is

that  no  two  BBS  nodes  share the same ID.  The ID numbers are

used  in the WATTCP.CFG configuration file (see above) to specify

certain  configuration  parameters  that  apply  to  specific BBS

nodes.

If  you are multi-tasking under DESQview, then you should add the

"/DV"  command  line  parameter  so  that KSP-NODE will let other

processes run while it is waiting on the network:

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      16

KSP-NODE  /DV 4  8

�  �

�  ��� Last ID#

������ First ID#

4.2 KSP-CALL (Used to Test KSP-NODE)

KSP-CALL  is  a  small  utility  program  intended  to be used to

verify  proper  installation of KSP-NODE.  Basically, KSP-CALL is

sort  of  a NetBios version of a Telnet client program.  That is,

you  may  run it to log into any of your BBS nodes where KSP-NODE

has been installed.

KSP-CALL  and  KSP-NODE  communicate  via  NetBios, and thus work

only  within  the  limits  of  your own local area network (i.e.,

within the context of your Novell or Lantastic network).

4.2.1 Command Line Parameters

If  no  command  line parameter is given, KSP-CALL will select an

idle  BBS  node  and  establish a connection with it.  Otherwise,

you  may  specify  a BBS node's ID number on the command line and

KSP-CALL will try to establish a connection with that node.

4.3 KSP-SRVR (The Inbound Telnet Server)

KSP-SRVR  acts  like a telephone switchboard.  It accepts inbound

Telnet  calls  from  the  Internet and then uses NetBios to route

them  to  KSP-NODE  on  an  idle  BBS  node.  KSP-SRVR can handle

several  Telnet  sessions  at  the  same time.  The actual number

depends  on  memory requirements (see below), but in practice the

usual limit is about sixteen simultaneous sessions.

4.3.1 Command Line Parameters

KSP-SRVR  has two command line parameters; both are optional.  If

specified,  the  first  is  the  maximum  number  of simultaneous

sessions  and  the second is the per-session buffer size.  Memory

requirements  increase  as  the  product  of  sessions and buffer

size;  the  larger  the  buffer  size,  the  fewer  the number of

simultaneous  sessions  that can be supported.  The default is 16

sessions and a buffer size of 512 bytes.

Note:  Unlicensed  (evaluation) copies of KSP-SRVR are limited to

a  single  session,  five  minutes per session, and a total of 15

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      17

minutes  running  time.   These restrictions are removed with the

purchase of an "access key" (see ORDERFRM.DOC).

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      18

CHAPTER 5 - SPECIAL CONFIGURATION CONSIDERATIONS

There  are  two common software environments that deserve special

mention.   When running any of the KSP-HOST software under one of

these  environments,  there  are  some  additional  configuration

steps required.

5.1 PCBoard

There  are  two  versions  of  PCBoard  that  run  under  MS/DOS:

PCBOARD.EXE  and  PCBOARDM.EXE.   The  latter is the "Multi-Port"

version,  and  is  the  only  one  that can be used with a Fossil

driver (required by KSP-NODE).

You  can  tell  whether  or  not PCBoard is configured to use the

fossil  driver because it will display "PORT1F" instead of "COM1"

(for example) on the waiting for calls screen.

Don't  forget  that  PCBoard  uses  port  numbers  which  are one

greater  than  those used by Fossil drivers.  For example, Fossil

designates  COM1 and COM2 as port 0 and port 1 respectively.  Yet

PCBoard thinks of these as port 1 and port 2.

5.2 DESQview

To  run  any  software based on the Waterloo TCP/IP library under

DESQview,   it's   absolutely   necessary  to  install  a  packet

multiplexer  on  top  of  the packet driver or packet driver shim

before  starting  DESQview.   This  is necessary whenever running

more  than  one  program on the same machine, even if only one of

them   is   a   Waterloo   TCP/IP  product.   A  freeware  packet

multiplexer  is  available  from  Key  Software  Products as file

KSPMUX*.ZIP, where the "*" is a two-digit version number.

To  run  the  KSP-HOST  software  under  DESQview,  you must also

enable  NETBIOS  support.  To do so, run DVSETUP inside DESQview.

Select  "Advanced  Setup",  and  press  "N"  for "Network".  Then

enter  "Y"  under  "NETBIOS"  and  press return twice to save and

exit.

5.3 Novell

NetBios  support  on  Novell networks is provided by a TSR called

"NETBIOS".  You must install this TSR before running any KSP-HOST

software.   (Lantastic  networks  already  have  NetBios  support

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      19

built into the basic system.)

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      20

APPENDIX 1 - HOW TO REACH US

The  Key Software Products BBS/FAX number (415-364-9847) operates

24   hours   a   day,  7  days  a  week.   Software  at  our  end

automatically  determines whether an incoming call is data or FAX

and will operate accordingly.

If  you have access to electronic mail, you can send us a message

via any of the following:

On COMPUSERVE, send mail to:

>Internet:tech.support@ksp.com

On PRODIGY, send mail to: VGDC59A

On Internet, UUCP, or Bitnet, send mail to:

tech.support@ksp.com

On Fidonet, address mail to "UUCP" at nearest fidonet site which

provides a gateway to Internet, such as 1:105/42.

1st line of message: To: tech.support@ksp.com

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      21

APPENDIX 2 - GETTING UPDATES VIA THE INTERNET

The  main  distribution  file  is KSPHST??.ZIP, where "??" is the

version  number.  You can retrieve this file via anonymous ftp at

"scizzl.scu.edu",  directory "ksp".  Please note that there is no

"e" at the end of "scizzl".

This  file  is also available from the KSP BBS, either by calling

in   and   downloading,   or  via  e-mail  using  the  "LISTSERV"

capabilities of the BBS.

To  use  LISTSERV,  you  must  send  a  message  addressed to the

Internet  address  listserv@ksp.com.  The message must contain an

LISTSERV  command  entered  as a single line of text, and it must

be  the first line in the message. The LISTSERV commands that are

supported include:

Command: HELP tlbmm-l

Purpose: Returns an text file with help information about LISTSERV.

Command: DIR tlbmm-l

Purpose: Returns an e-mail message listing all of the file

directories on the BBS by number and description.

Command: LIST tlbmm-l 1

Purpose: Returns an e-mail message listing all of the files in

directory #1 of the BBS.

Command: GET tlbmm-l KSPHST12.ZIP

Purpose: Returns an e-mail message containing a uuencoded version of

the file KSPHST12.ZIP.  To extract the file, you must save

the message as a file on your system, edit out the e-mail

header at the beginning of the file, and then use a program

called UUDECODE (commonly available on Unix) to convert the

uuencoded version back to the original ZIP format, and then

(finally) use PKUNZIP to extract the contents of the file.

Copyright (C) 1995-96, Key Software Products. All Rights Reserved

Feb 11, 1996           KSP-HOST (tm) v1.2                      22

APPENDIX 3 - LEGAL STUFF

LIMITED WARRANTY

This  software  is provided 'as is' without warranty of any kind,

either  expressed  or  implied, including, but not limited to the

implied   warranties   of   merchantability  and  fitness  for  a

particular  purpose.   The  entire  risk  as  to  the quality and

performance of the program is with you.

Some  states do not allow the exclusion of implied warranties, so

the  above  exclusions may not apply to you.  This warranty gives

you  specific  legal  rights  and  you may also have other rights

which vary from state to state.

Key  Software  Products  has  taken  due  care  in  preparing the

documentation  and  software  included  in   to  ascertain  their

correctness  and  effectiveness.   However, Key Software Products

does  not  warrant  that  operation  of  this  software  will  be

uninterrupted  or  error  free.   In  no event shall Key Software

Products  be  liable  for  incidental or consequential damages in

connection  with  or  arising out of the furnishing, performance,

or use of this software.

LICENSE

You  MAY  use  this software on any computer or computers in your

possession.   The licensed version is registered for use on up to

a  fixed  number of BBS nodes running on multiple machines and/or

multiple multi-tasking processes.

You  MAY  copy this software into any machine readable or printed

form  for  backup or modification purposes in support of your use

of the software.

You  MAY  distribute  the original unmodified, unlicensed version

of  this  software,  but you may not charge a fee exceeding $5.00

to  cover  the  cost of duplicating, shipping, and handling.  You

may NOT distribute a licensed version of this software.

You  may  NOT  use,  copy, modify, sublicense, assign or transfer

this  software  and  its license, or any copy or modification, in

whole  or  in  part,  except  as  expressly  provided for in this

license.

Copyright (C) 1995-96, Key Software Products. All Rights Reserved