DCSCBV v1.0�, (c) 1996, Data Communication Systems

INTRODUCTION

============

DCSCBV is a Call Back Verifier for Maximus-CBCS v3.00.  This

program was written with Borland Pascal v7.0.  Because this is a

Beta Release, the documentation is very limited.  Every important

aspect for installation has tried to have been covered but no

guarantees will be made.

If you have any problems, comments or suggestions, please feel

free to contact Data Communication Systems at the address listed

at the end of this documentation.  All suggestions will be taken

into consideration for future releases of this software.

LIMITATIONS AND REQUIREMENTS

============================

In order to use this software, you must meet the following

requirements.

1.             You must be running Maximus v3.x BBS software.  It

has been tested on both v3.00 and v3.01, but will

not run on any version below v3.00.

2.             You cannot allow your users to enter or modify

information in the Data Phone field of the user

database.  This is where DCSCBV stores the

telephone numbers that has been dialed.

QUICK INSTALLATION

==================

1.             Create a directory (suggested \MAX\DCSCBV\) and

unzip the distribution archive into that

directory.

2.             Unzip the archive SAMPLES.ZIP (contained in the

original archive) into the same directory.

3.             Using a standard ASCII text editor, edit

DCSCBV.CFG and uncomment and modify the LOCAL_AREA

token to contain the three digit number of your

local calling area.

4.             Using a standard ASCII text editor, create a file

named TRNSLATE.CFG containing a list of

translation strings for each prefix in your local

calling area.  For more information, see the

section on Translation Strings.

5.             Modify your MENUS.CTL file to contain the

following line.

Display_File  DCSCBV\DCSCBV  Demoted  "Register"

6.             Run SILT.EXE and test your Call Back Verifier.

TRANSLATION STRINGS

===================

DCSCBV converts the phone number entered by the user into a

standardized form (1-AAA-BBB-CCCC) using the LOCAL_AREA token in

the configuration file.  It then scans the file TRNSLATE.CFG and

translates the standardized phone number into a number that can

be dialed.  Here's how it works ...

Each line in TRNSLATE.CFG should contain two columns.  If the

string contained in the first column is found in the standardized

phone number, then that string is replaced with the string found

in the second column of the file.

For an example, let assume that the user enters "5551212" and

your LOCAL_AREA token is "504".  The standardized form of this

number will be "1-504-555-1212".  Now, lets assume that the

following line can be found in TRNSLATE.CFG.

1-504-555- 555-

The first column is compared to the standardized phone number and

"1-504-555-" is replaced with "555-" resulting in "555-1212".

This is a dialable number and can now be compared for toll

status, duplicate and unwanted numbers.

TOLL STATUS

===========

To determine the toll status of a number, each line of TOLL.CFG

is compared to the dialable number.  If the dialable number

starts with a line that is read, then that number is considered

to be a long distance call.

For example, if "1-" is contained in TOLL.CFG, and the dialable

number is "555-1212", then the number would not be considered to

be long distance.  If the dialable number read "1-504-555-1212"

(which starts with "1-") then that number would be considered to

be long distance.

This file could be used to compare numbers down to individual

prefixes to consider them long distance.  For example, consider

the following TOLL.CFG file.

1-

486-

478-

If a number translated to "486-7890", then that number would be

dialed and would be treated as a long distance call.

DUPLICATE NUMBERS

=================

When a user is verified, their dialable number is placed in the

Data Phone field of the user database.  If the token CHECK_DUPES

is uncommented in the configuration file, then DCSCBV will scan

the user database to determine if another user has been verified

using that number.  If another user has been verified using that

number, then the DUPE.SCR file is displayed and the program exits

back to the BBS.  The users access is not modified.

UNWANTED NUMBERS

================

The process of checking unwanted numbers is identical to checking

the toll status.  To determine if a number is unwanted, each line

of UNWANTED.CFG is compared to the dialable number.  If the

dialable number starts with a line that is read, then DCSCBV will

display the DENY.SCR file and will exit back to the BBS.  The

users access is not modified.

A sample UNWANTED.CFG would be:

1-900-     ; Toll calls

1-818-     ; Hawaii

454-6578   ; Your BBS or home number

COMMAND LINE PARAMETERS

=======================

DCSCBV will accept the following command line parameters.

-P             Communication port (1 = COM1)

-B             Current baud rate

-S             Baud rate if port is locked

-N             Current task

-C             Specify configuration file

CONFIGURATION FILE

==================

DCSCBV's configuration file is a standard ASCII text file that

contains a list of tokens to customize the way DCSCBV will

operate.  In this file, a semicolon (";") can be used as a

comment.  All text after a semicolon and all blank lines will be

ignored.

For tokens concerning modem commands, use the following list of

translation characters.

|              Transmits a Carriage Return (ASCII

code 13)

~              Creates a 1 second delay

`              Creates a 0.5 second delay

^              Raises DTR on port

v              Lowers DTR on port

The following chart is a list of all recognized tokens along with

their default values and a brief description.

NOTE: The only token that is required is LOCAL_AREA.

Token Name     Default        Description

----------------------------------------------------------------

LOCAL_AREA                    The three digit number of your

local area code.  This number will

be used to create a standardized

telephone number before translating

to a dialable number.

CHECK_DUPES                   Will tell DCSCBV not to dial a

number if another user has already

been verified with it.  If this

token is not present, DCSCBV will

allow multiple users to be verified

at the same number.

ADD_NONCONNECT                Will tell DCSCBV to add a number to

the unwanted list if it failed to

reconnect after the specified

number of attempts.  If this token

is not present, DCSCBV will allow a

user to try again if it failed to

reconnect.

LOGFILE        DCSCBV.LOG     Where sessions will be logged.  The

file specified must contain the

full path and filename.  The

default extension .LOG will be

added if one does not exist.  For

tasks greater than zero, the last

two characters of the filename

(extension excluded) will be

translated into the two digit

hexadecimal value of the task

number.

LOG_RESPONSE                  Will tell DCSCBV to add response

messages from the modem to the log

file after dialing.  If this token

is not present, DCSCBV will not log

modem responses.

MAX_DIR        \MAX\          Where DCSCBV can find the Maximus

user database and LASTUS??.BBS.

USERFILE       USER.BBS       The name of the Maximus user

database.  This file will be

searched for in the directory

specified by MAX_DIR.

PASS_PRIV      30             The privilege level to give a user

after a successful verification.

PASS_XPFLAG    0              Expiry flags to set after a

successful verification.

PASS_XPMINS    0              Expiry minutes to set after a

successful verification.

PASS_XPDATE                   Expiry date to set after a

successful verification.

PASS_XPPRIV    30             Expiry privilege to set after a

successful verification.

VERIFY_TIME    60             The number of seconds to allow a

user to enter their verification

password.

PW_ATTEMPTS    3              The number of times to allow a user

to enter their verification

password.

LD_ATTEMPTS    1              The maximum number of times to dial

a number to re-connect to a user if

the toll status determines it to be

outside of the local calling area.

LOCAL_ATTEMPTS 3              The maximum number of times to dial

a number to re-connect to a user if

the toll status determines it to be

in the local calling area.

DIAL_DELAY     45             The maximum number of seconds to

wait for a connect after dialing.

DIAL_PREFIX    ATDT           The string to send to the modem

before the number to tell the modem

to dial.

DIAL_SUFFIX    |              The string to send to the modem

after the number to tell the modem

to dial.

DIAL_HANGUP    v`^~+++~ATH0|  The string to send to the modem to

hangup on the caller.

CBV_DIR        \MAX\DCSCBV\   Where DCSCBV can find all necessary

files to run.  This includes all of

the configuration and screen files

which can be redefined here.

================================================================

NOTE: All of the files specified below should contain the

filename and extension only.  They will be searched for in the

directory specified by CBV_DIR.

================================================================

TRANSLATE      TRNSLATE.CFG   The name of the file that contains

a list of translate strings to

convert a standardized telephone

number to a number that can be

dialed from your system.

UNWANTED       UNWANTED.CFG   The name of the file that contains

a list of numbers that you do not

wish to be dialed.

TOLL           TOLL.CFG       The name of the file that contains

a list of strings to compare a

translated number to determine if

it is in the local calling area.

ALREADY        ALREADY.SCR    The name of the file to display to

the user if he/she already has a

privilege level greater than or

equal to PASS_PRIV.

INTRO          INTRO.SCR      The name of the file to display to

the user just before prompting for

their phone number.

DUPE           DUPE.SCR       The name of the file to display to

the user if the number he/she

entered was used to verify another

user.

DENY           DENY.SCR       The name of the file to display to

the user if the number he/she

entered was found in the unwanted

numbers file.

PASSCODE       PASSCODE.SCR   The name of the file to display to

the user just before giving them a

verification password.

READY          READY.SCR      The name of the file to display to

the user just before hanging up and

just before displaying the LOCAL or

LD file.

LOCAL          LOCAL.SCR      The name of the file to display to

the user if the toll status

determines the caller to be in the

local calling area.  DCSCBV will

display this file just before

hanging up and just after

displaying the READY file.

LD             LD.SCR         The name of the file to display to

the user if the toll status

determines the caller to be outside

of the local calling area.  DCSCBV

will display this file just before

hanging up and just after

displaying the READY file.

PASS           PASS.SCR       The name of the file to display to

the user after a successful

verification.

FAIL           FAIL.SCR       The name of the file to display to

the user after entering the wrong

verification password with the

specified number PW_ATTEMPTS.

REGISTRATION

============

DCSCBV is distributed as ShareWare.  You are entitled to 30 days

to evalueate this software.  To register DCSCBV, fill out the

form and send $10 to the address below.  All registered users of

this software will be notified of future releases and will

receive a discount on all upgrades.  There will be no cost for

minor changes and bug fixes.  This beta release is distributed

without any restrictions or limitations.

COMPANY:       _____________________________________________

NAME:          _____________________________________________

ADDDRESS:      _____________________________________________

_____________________________________________

CITY/ST/ZIP:   _____________________________________________

BBS NAME:      _____________________________________________

# OF LINES:    _____________________________________________

PRIMARY PHONE: _____________________________________________

MODEM TYPE:    _____________________________________________

FIDO ADDRESS:  _____________________________________________

WHERE DID YOU HEAR ABOUT THIS SOFTWARE? ____________________

____________________________________________________________

Send $10 check or money order along with this registration form

to:

Data Communication Systems

2221 Houma Blvd, Apt 102

Metairie, LA  70001

DISCLAIMER

==========

Neither Data Communication Systems nor any of its authors,

programmers, employees or owners will be held liable for any

special, direct, indirect, incidental, consequential, exemplary

or punitive damages or any inconvenience arising out of the

provision or use of this software.

The latest version of this software may be obtained from the

following BBS:

The Third Millennium

Located in Metairie, Louisiana

Operating 24 hours a day, 7 days a week

A service of Data Communication Systems

1:396/85  @ FidoNet

8:3007/2  @ FamilyNet

24:101/3  @ CCHN (Christian Concert Hotline Network)

97:2001/1 @ SubSpaceNet

(504) 454-6578 : 300 - 28,800 b.p.s. (V.34)

FTSC file requests can be made using the name "CBV".  In the

future case of relocation, questions or suggestions, the authors

of this software can be reached through the FidoNet MUFFIN, MECCA

or PASCAL echo.  If your message does not adhere to the rules or

topic of the echo's then please address them via NetMail or in a

locally through the BBS.  To guarantee reception of the message,

please address it to "David Dunson".