Zmax v3.20

Copyright 1990, 91, 92 Mike Bryeans

All Rights Reserved

Zmax  is designed to replace Zmodem. It uses 32 bit CRC's  on

file data blocks, the same as Zmodem, and 32 bit CRC's on its

information blocks, Zmodem uses 16 bit.

An  information  block  is  a block  of  data  used  to  send

information about the name of the file, the receivers  system

setup, about the next block of actual File Data, etc.

Zmax  is setup to allow almost unlimited future expansion  to

the protocol while allowing it to remain backwards compatible

with older versions.

In  stream mode Zmax sends blocks, the size being set by  the

receiver, of data but doesn't require ACKS from the receiver.

Zmax   should   (depending   on   equipment,   phone   lines,

implementation, etc.) allow  about 98.5 percent efficiency at

2400 baud on a 30K file compared to Zmodems 95 percent.

And  unlike Zmodem, Zmax reaches full speed  on  considerably

smaller  files.

In  Non-Stream mode (which can be set by the receiver OR  the

sender), Zmax is a super Batch Xmodem or Ymodem depending  on

the block size. Super because it uses 32 bit CRC's instead of

16.

Even  using  Ymax  mode (Zmax running  1K  blocks  with  Acks

required), you'll get faster transfers than under Zmodem.  In

fact,  there  wasn't any measurable difference  between  Ymax

mode and Zmax mode on a 30K transfer. At least none that  the

efficiency  computation  routine showed.

This  is at 2400 baud of course. Non-Stream  protocols  don't

fair  near  as  well (speed wise) as  Streamed  protocols  at

highspeeds.

We'll call Zmax running 128 byte blocks with ACKS Xmax  mode.

Running  in this mode, you'll get about (maybe a tad  better)

Xmodem speed, but with the additional safty of 32bit CRCs.

Is Zmax compatible with Zmodem?

The  answer  to  that  one  is no,  it  is  not.  This  is  a

PREPLACEMENT protocol, not just a reworked hack of zmodem. It

has better error detection and it is faster than zmodem.

Zmax.exe

Zmax.exe requires a number of command line switches; -B,  -P,

{-R or -F}.

Full Command Line Switch Run Down

-O  (Reserved For Use with Osiris)

-V  (Reserved For Use With Osiris)

-R  Receive files

-K  Keep  interrupted transfers (Do not use if running  on  a

BBS)

-B, supported baud rates 300 - 115,200 Baud.

-P, Supported Com Ports, COM 1, COM 2, COM 3, and COM 4.

Supports COM 5-36 on Osiris systems.

-L, Supports Locked Baud Rates, 300 - 115200 Baud.

-F,  As  many as DOS allows. Full drive, path,  and  filename

supported.

-A, Run in NON-STREAM mode.

-S  xxxx,  Set block size to XXXX. Valid ranges;  1  -  2048.

Going over 1024 at 2400 bps is NOT recommended. If

this parameter is used, the -B MUST be used  first

because -B will reset the block size according  to

baud rate.

-@Filename,  Filename is a text file listing the names of the

files to send. The contents of filename  is  one

filename per line, unlimited number of lines.

e.g.

Tmodem.zip

Zmodem.Zip

C:\Osiris\Files\MISC\PK110.EXE

You use -@ in place of -F, do NOT intermix them!

Sample Command line

Zmax -B2400 -P1 -@C:\RyBBS\Send.Ctl

NOTE:   -R  and  -F  MUST  be  the   last

parameter on the command line. As soon as

Zmax  encounters  either one of these two

switches, it will begin to either Receive

or Send.

The -L switch is only for those that do not have COMx= set in

the  environment. If your running Osiris then Zmax will   use

the COMx= automatically.

Sample command line for a 3/12/2400 modem.

Send    : -B%1 -P%2 -F %3

Recevie : -B%1 -P%2 -R

NOTE:  DOS  batch  parameters, %1, %2,  %3,  etc.  will  vary

depending on the term or BBS program being used.

Sample command line for highspeed modems, locked at 19200  or

38400.

Send    : -L38400 -B%1 -P%2 -F %3

Recevie : -L38400 -B%1 -P%2 -R