Invariably some bugs will pop up.  Here's some tip on what to do.

Runtime error xxx at xxxx:xxxx

This error is being reported by Turbo Pascal.  You can look up the

error number in MESSAGES.DOS to figure out what is going wrong.

Runtine error 100 at xxxx:xxxx.

If this happens at startup, your MSG.BB file is probably bad.

See if you have a BBOLDMSG.BB hanging around from the last GM command

and try renaming it to MSG.BB.

Runtime error 201 at xxxx:xxxx.

You found a bug.  See REPORTING ERRORS below

Runtime error 202 at xxxx:xxxx.

This a stack overflow and is probably a bug.

Runtime error 203 at xxxx:xxxx.

You are out of memory.  One usual problem is that you scrolled a window

forward and then left it.  New lines get tacked on the bottom but the

old ones can't go away because you have the screen stopped.

Runtime error 209 at xxxx:xxxx.

Can't find/read BB.OVR.

Overrun

The RS-232 line to the TNCs dropped a character.  This happens when

the computer gets so busy, it doesn't pull the characters off fast

enough.  You will probably see one of these every few hours.  This

seems to be normal.  If you get a lot, you need to slow down the

TNC link and/or turn monitor off.

Timeout in block loop

BB did not receive anything from the TNC for 60 seconds.  This can

be caused by RF or other noise getting into the RS-232 cables.  Or

maybe your TNC died

Reporting errors

----------------

Please report errors.  It may be my problem.  Send a message to

AA4RE @ AA4RE.#NOCAL.CA.USA.  Include:

1) The exact error message you got and whatever else weird

happened.

2) Your computer type and speed (example: AT clone 16 Mhz)

3) Your TNC layout (example:  PK232 on COM1)

4) Your BB and MBBIOS version numbers.

5) Your DRSI software version # (if any).

6) Your G8BPQ software version # (if any).

If the BBS created a file called DUMP.BB, sent that along too.

You should at least receive an ACK from me.