G8BPQ Packet Switch for the Kantronics Data Engine.

I have now produced a version of my packet switching software for the

Kantronics Data Engine. The facilities provided are very similar to

those of the standard (PC) version, but there are slight differences,

mainly due to the different hardware environment. The two radio ports

are configured as if they were HDLC cards, but the async port may be

used either as a KISS (or NET/ROM) link to another TNC, or as a 'normal'

TNC link to a PC. Eventually this will provide all the options of my

Virtual TNCs (ie TNC2, PK HOST mode for AA4RE or UFQ), but so far only

the TNC2 version works. Obviously (or perhaps not so obvious really),

as there is only one serial port, you can only define one 'Virtual'

TNC in the TNCPORT params. Also the async port can only be used for

one thing at a time, but you can configure both, and select either

link mode or TNC mode using a switch. This is useful if you are setting

up a multiport node, but want to attach a terminal temporarily whilst

configuring the system. The SYSOP functions are also provided within

the code.

Environment.

The code normally runs from EPROM (one 27512), and I suggest that it

replaces the Kantronics EPROM. A facility has been designed to allow it

to go in the second socket, and be selected using the front panel switch,

but as I have mislaid the chip that Kantronics kindly sent me, I havnt

been able to test this feature. It is also possible to down line load the

code, and run it from RAM, which is useful when experimenting, but not

ideal for a live system. It also requires a 128K byte static ram chip,

which is rather expensive! The PROM based system requires 2 * 32k byte

static RAMs.

The program is distributed in a file KANT.BIN, which must be merged

with the BPQCFG.BIN file produced in the normal way, and with PASSWORD.BPQ,

to produce either a PROM Image (KANT.PRM) or an Intel HEX format file for

down line loading (KANT.HEX). This is done using program KMERGE.COM. Normally

a PROM image is produced - to get the Intel HEX output, type KMERGE HEX. Note

that the current HEX file format is not compatible with the loader in the

Kantronics standard ROM. It may well soon be but in the meantime you must

use the supplied KBOOT program (again blown into a 27512, but this time

in the top 16k of the prom) if you want to use the down line load option.

The SYSOP facility is password protected. The password string is taken from

file PASSWORD.BPQ. To enter SYSOP mode, enter PASSWORD. The system will

respond with 5 numbers. Then enter PASSWORD ABCDE where ABCDE are the

corresponding characters from the password string in the EPROM. The system

should then respond Ok. Note that the password is not case sensitive.

There are a few additional SYSOP commands:

CLOCK displays or sets the battery backed real time clock. To set the clock,

enter CLOCK HH MM. As the clock is only used for timestamping the ROUTES

table, only the hours and minutes registers are used.

COLDSTART reinitialises the RAM, and resets the node. Normally the ROUTES,

NODES and STATS are maintained when power is off.

WARMSTART restart the code without clearing the ROUTES, NODES and STATS.

A digi'ed link may be added to a route entry. Enter:

route call port v digi1 (digi2)

A locked node entry may also be added:

node add alias:call neighbour-call port quality

Or deleted:

node del alias:call

The following SYSOP commands are new in version 4.05e, mainly to save

having to keep blowing new EPROMS when experimenting.

Command			Corresponding BPQCFG parameter

OBSINIT			OBSINIT

OBSMIN			OBSMIN

NODESINT		NODESINTERVAL

L3TTL			L3TIMETOLIVE

L4RETRIES		L4RETRIES

L4TIMEOUT		L4TIMEOUT

T3			T3

IDLETIME		IDLETIME

LINKEDFLAG		ENABLE_LINKED

IDINTERVAL		IDINTERVAL

MINQUAL			MINQUAL

FULLCTEXT		FULL_CTEXT

HIDENODES		HIDENODES

L4DELAY			L4DELAY

L4WINDOW		L4WINDOW

BTINTERVAL		BTINTERVAL

Switches and LEDS.

The switch selects LINK or TNC mode on the ASYNC PORT. (In for LINK mode)

LED1. On in TNC mode, flashes as packets are received on async port

in LINK mode.

LED2. Packet Received on A port.

LED3. DCD on A port.

LED4. Not Used.

LED5. Packet received on B port.

LED6. DCD on B port.

LED7. CSMA Indicator. Shows 'Slottime Wait' timer is running

LED8. Flashes at one second intervals to show system is running

'Features' of current version (4.03)

Operation of the async port in Host Mode is not supported.

Configuration Suggestions.

The following are some ideas on how the Data Engine could be used in

packet networks. (Using my software - it can, of course be used as a

normal TNC with the Kantronics software).

Standalone Data Engine.

This would seem to be a reasonable solution for a small node,

providing one user access and one trunk channel. The main advantages

over a pair of conventional NET/ROM or TheNet TNCs are that only one

Call/Alias pair is required, so the size of NODES lists and broadcasts

are minimised, and the STATS facilites allow better network management

and performance monitoring. Also it will be better able to handle higher

speed links as they become available. The main disadvantage is a rather

higher cost.

Data Engine driving KISS TNCs.

-----------------                    ---------------

|                 |------------------|   'Normal'    |-- Radio 3

|  Data Engine    |                  |               |

|                 |                  |   KISS TNC    |

-----------------                    ---------------

|           |

|           |

Radio 1     Radio 2

-----------------                    ---------------

|                 |------------------|               |-- Radio 3

|  Data Engine    |                  |    KPC4       |

|                 |                  |               |-- Radio 4

-----------------                    ---------------

|           |

|           |

Radio 1     Radio 2

-----------------                    ---------------

|                 |------------|-----|  'BPQKISS'    |

|  Data Engine    |            |     |               |-- Radio 3

|                 |            |     |      TNC      |

-----------------             |      ---------------

|           |               |

|           |               |

Radio 1     Radio 2            |      ---------------

|-----|  'BPQKISS'    |

|     |               |-- Radio 4

|     |      TNC      |

|      ---------------

|

|

|      ---------------

|-----|  'BPQKISS'    |

|     |               |-- Radio 5

|     |      TNC      |

|      ---------------

etc

With this system, the Data Engine drives two radio ports (the fastest

or busiest) directly, and has a number of KISS TNCs connected to the

async port. A 3 port node could be built using any KISS TNCs, and a

4 port node would be possible with, for example, a KPC4 (or even another

Data Engine in KISS mode). If you need four or more ports using normal

TNC2's (or clones), you need to run my 'Polled KISS' software, which

allows several TNCs to be driven from one async port, using a simple

diode matrix, and a polling protocol to prevent collisions on the

matrix. The thoretical maximum number of ports is 16, but this might

cause performance problems.

This has the same advantages of reduced NODES size and improved Stats

as the single Data Engine, but also get over the rather nasty problem

of internal routing loops which can occur with multiport NET/ROM

clusters. Although solutions to this problem have been suggested (either

by using a diode maxtrix which does not fully interconnect all the TNCs,

or by careful manipulations of qualities), none seem to be totally

successful. The main disadvantage is that failure of the Data Engine

will disable the entire node complex.

Linked to a PC, which is also running BPQ.

-----------------                    ---------------

|                 |                  |      PC       |

|  Data Engine    |      KISS        |---------------|

|                 |------------------| G8BPQ | BBS   |

-----------------                    ---------------

|           |                        |

|           |                        |

Radio 1     Radio 2                ? Radio 3 (or more)

The DE async link is defined as a KISS (or NETROM) link, as is one of

the async ports on the PC.

This might make sense if you want to run a node and a BBS, where the node

is carrying so much non-BBS traffic that the PC would not be able to cope

on its own - maybe if the DE was supporting high speed trunk links. The

disadvantages are that the BBS (and SYSOP) cant see the monitored stuff

from the DE radio ports, and that you may end up with more nodes tables

entries than you really need - especially if the PC has other radio ports

attached, and is also running as a switch as well as a BBS. It would

normally make more sense to run the Data Engine in KISS mode (using the

Kantronics EPROM), and let the PC do all the hard work.

John Wiseman, G8BPQ

13/3/91

Updated 5/1/92 and 19/7/92

##################################