ProModem RS232 Interrupt Driven Serial Communication Library  v1.0

Written By Adrian J. Michaud

(C) Interactive Telecommunication Systems 1992-1993

Table Of Contents

-----------------

Page

Introduction                                                    1

Features/Supported Compilers                                    2

Registration                                                    3

Ording Form                                                     4

SetupControlBlock()                                             5

OpenCom()                                                       6

CloseCom()                                                      7

SendCharacter()                                                 8

CheckQueue()                                                    9

GetCharacter()                                                  10

ClearQueue()                                                    11

GetCarrierDetect()                                              12

SetBaudRate()                                                   13

SetDTR()                                                        14

DropDTR()                                                       15

SetDataFormat()                                                 16

SendString()                                                    17

SetFIFOMode()                                                   18

SetFIFOTriggerLevel()                                           19

WaitForCharacter()                                              20

DisableIRQ()                                                    21

EnableIRQ()                                                     22

TERMINAL.C - Example Terminal Program source file               23

BBS.C      - Example Bulletin Board source file                 24

Page 1 - ProModem Users Manual         Interactive Telecommunication Systems

Introduction

------------

ProModem is a High Performance Interrupt Driven RS232 serial

communication library that can be used for creating a Computerized

Bulletin Board System, a Terminal program, a RS232 Networking Driver,

or for RS232 Host to Host communications. ProModem supports the following

UARTS: PC16550C, NS16550AF, PC16550CF, 16450, and the 8250. ProModem takes

full advantage of a 16550 by enabling the 16 character FIFO to relieve

the CPU of excessive software overhead. When using a 16550, internal

FIFOs are activated allowing 16 bytes (plus 3 bits of error data per

byte in the RCVR FIFO) to be stored in both receive and transmit modes.

All the logic is on chip to minimize system overhead and maximize

system efficiency.

Page 2 - ProModem Users Manual         Interactive Telecommunication Systems

Features

--------

ProModem contains the following features:

o Interrupt driven INPUT directly VIA 8259 Peripheral Interrupt Controller.

o Full 16550 FIFO support with selectable trigger levels.

o Supports user definable Base Addresses using IRQ3 or IRQ4.

o Automatic CTS/RTS Hardware Handshaking for Asynchronous Devices.

o Baud rates up to 115k bits per second.

o Handles unlimited Open Com Ports using IRQ3 and IRQ4.

(overruns might occur with slow CPU's or High Baud Rates of course).

o Supports both ISA (PC AT) and MCA (PS 2) machines

o Small, (Large, and Huge memory models available to registered users).

'C' Compilers Supported

-----------------------

Microsoft C     - v5.1, v6.0, QuickC v2.5, and C/C++ v7.0

Borland TurboC  - v1.0, v2.0, v3.0, v3.1

Page 3 - ProModem Users Manual         Interactive Telecommunication Systems

Registration

------------

ProModem is not a public domain program and is not free.  ProModem is

Copyright (C) 1993 by Interactive Telecommunication Systems. Non-registered

users of this library are granted a limited one month license to ProModem

to evaluate the libraries suitability for their requirements.  Any usage of

ProModem beyond evaluation time period requires registration of each copy of

the library used.  Use of non-registered copies of ProModem beyond the

original evaluation period is prohibited.

The Shareware version of ProModem has been built with a small model.

When you register ProModem you will receive FULL printed documentation

along with a LARGE and a HUGE library for all compilers supported.

Source code to all memory models for all supported compilers is

also part of the registration package.

There are two ways to register. The first is by sending $25 to

Interactive Telecommunication Systems along with a registration form.

Non U.S. residents, please send checks drawn on U.S. funds. We will accept

non U.S. currency, but you must include an additional $5 (US) to cover

exchange handling. Also add $5 overseas shipping/handling. See the page 4

for the ordering form and ordering information.

Page 4 - ProModem Users Manual         Interactive Telecommunication Systems

ProModem Ordering Form

----------------------

Please Remit To:      Eutecnics

C/O Adrian J. Michaud - ProModem!

30 Nagog Pk., Suite 105

Acton, Ma 01720

Qty.    Description                                   Each     Total

____    ProModem Registration with 3.5" diskette    $25.00   ________

Includes ALL memory models with Source

Code and a printed Manual. MSC & TC.

____    ProModem Registration with 5.25" diskette   $25.00   ________

Includes ALL memory models with Source

Code and a printed Manual. MSC & TC.

(U.S. orders add $2 shipping & handling)        Shipping   ________

(Foreign orders add $5 shipping)

(Add $5 for Foreign Currency Exchange)     Misc. Charges   ________

(MA residents please add 7.25% sales tax)            Tax   ________

TOTAL  $________

Name: _________________________________________________________________

Company: _________________________________________________________________

Address: _________________________________________________________________

_________________________________________________________________

_________________________________________________________________

Where did you obtain ProModem? ( ) Friend   ( ) Shareware House   ( ) Other

( ) BBS      ( ) User Group       __________

All checks or money orders must be drawn on U.S. Funds in U.S. Dollars.

Sorry, no C.O.D. orders will be accepted.

Make All Checks Payable to:  Interactive Telecommunication Systems

Page 5 - ProModem Users Manual         Interactive Telecommunication Systems

Function: int SetupControlBlock(AJMS *controlBlock,

unsigned int baseAddress,

int irq,

unsigned int bufferSize);

Description: Sets up a control block for a specific com port and IRQ

number. This is the first function that is always used when

preparing to communicate to a COM port. Each com port that

will be opened requires a different control block.

Return Value: SUCCESSFUL    - Setup was successful.

TOO_MANY_IRQS - Too many mounted com ports.

Example Usage:

#include "promodem.h"

main()

{

AJMS controlBlock;

/* This sets up a control block for COM1 (0x3f8), IRQ4, and a

4096 Byte receive Buffer.

*/

SetupControlBlock(&controlBlock , 0x3f8 , 4 , 4096);/*Init Control Block*/

OpenCom(&controlBlock);                             /*Open COM port     */

}

Page 6 - ProModem Users Manual         Interactive Telecommunication Systems

Function: int OpenCom(AJMS *controlBlock);

Description: Opens a COM port for communication. This function allocates

an IRQ buffer and sets up the UART for communication.

Return Value: BUFFER_ALLOCATION_ERROR - Could not allocate IRQ buffer.

COM_ALREADY_OPEN        - Port is already open and ready.

SUCCESSFUL              - COM port successfully opened.

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously  */

/* been set up.                            */

OpenCom(&controlBlock);   /* Open COM port */

}

Page 7 - ProModem Users Manual         Interactive Telecommunication Systems

Function: int CloseCom(AJMS *controlBlock);

Description: Closes a previously opened COM port. This de-allocates the

IRQ buffer that was allocated by OpenCom() and restores

the UART to its previous state before it was opened.

Return Value: COM_PORT_NOT_OPEN - COM port was not previously Opened.

SUCCESSFUL        - COM port successfully closed and IRQ

buffer has been de-allocated.

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously  */

/* been set up and the COM port is OPEN    */

CloseCom(&controlBlock);   /*Close COM port */

}

Page 8 - ProModem Users Manual         Interactive Telecommunication Systems

Function: int SendCharacter(AJMS *controlBlock, unsigned char ch);

Description: Sends a character to the COM port.

Return Value: CARRIER    - If there is a carrier.

NO_CARRIER - If there isn't a carrier.

Also see: SendString()

Example Usage:

#include "promodem.h"

example()

{

unsigned char ch;         /* Allocate ch on the stack */

extern AJMS controlBlock; /* Assumes a control block has previously  */

/* been set up and the COM port is OPEN    */

ch = (unsigned char)getch();      /* Get a character from the keyboard */

SendCharacter(&controlBlock, ch); /* Send character to COM port        */

}

Page 9 - ProModem Users Manual         Interactive Telecommunication Systems

Function: int CheckQueue(AJMS *controlBlock);

Description: Checks the status of the IRQ receive buffer.

Return Value: CHARACTERS_WAITING - If there are characters in queue.

QUEUE_EMPTY        - If queue is empty.

Also see: GetCharacter(), ClearQueue(), WaitForCharacter()

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN   */

if (CheckQueue(&controlBlock) == CHARACTERS_WAITING) /* Check Queue     */

printf("%c", GetCharacter(&controlBlock)); /* Print one byte from Queue */

}

Page 10 - ProModem Users Manual        Interactive Telecommunication Systems

Function: int GetCharacter(AJMS *controlBlock);

Description: Gets a single character from Receive IRQ Queue.

Return Value: First Character that was placed in the receive Queue.

The receive queue expands as more characters are

received. GetCharacter() takes the first character

from the queue and returns it as an (int) with the

high byte masked.

Also see: CheckQueue()

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN   */

if (CheckQueue(&controlBlock) == CHARACTERS_WAITING) /* Check Queue     */

printf("%c", GetCharacter(&controlBlock)); /* Print one byte from Queue */

}

Page 11 - ProModem Users Manual        Interactive Telecommunication Systems

Function: void ClearQueue(AJMS *controlBlock);

Description: Clears the contents of the Receive IRQ Queue.

This insures that there are no pending characters

in the Receive IRQ Queue.

Return Value: None.

Also see: CheckQueue()

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN   */

ClearQueue(&controlBlock); /* Clears the Receive IRQ Queue */

}

Page 12 - ProModem Users Manual        Interactive Telecommunication Systems

Function: int GetCarrierDetect(AJMS *controlBlock);

Description: Checks the status of the DCD (Data Carrier Detect) line

on the serial port. This is used to detect if a modem

has a carrier or not.

Return Value: CARRIER    - If there is a carrier.

NO_CARRIER - If there isn't a carrier.

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN   */

if (GetCarrierDetect(&controlBlock) == NO_CARRIER) /* Check DCD */

printf("\nThere is no Carrier\n"); /* Display Loss of Carrier.  */

}

Page 13 - ProModem Users Manual        Interactive Telecommunication Systems

Function: void SetBaudRate(AJMS *controlBlock, unsigned long baudRate);

Description: Sets up the baud rate for the serial port.

Return Value: None.

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN   */

SetBaudRate(&controlBlock, (unsigned long)38400); /* set to 38.4k Baud */

}

Page 14 - ProModem Users Manual        Interactive Telecommunication Systems

Function: void SetDTR(AJMS *controlBlock);

Description: Turns ON the DTR (Data Terminal Ready) line on the

serial port. OpenCom() automaticly turns on DTR

when a COM port is initially opened. DTR needs to

be turned on to talk to a modem.

Return Value: None.

Also see: DropDTR()

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN   */

SetDTR(&controlBlock); /* Turns on DTR (Data Terminal Ready) */

}

Page 15 - ProModem Users Manual        Interactive Telecommunication Systems

Function: void DropDTR(AJMS *controlBlock);

Description: Turns OFF the DTR (Data Terminal Ready) line on the

serial port. CloseCom() does NOT automatically turn

OFF DTR. Some modems when set up correctly will

drop carrier when DTR is toggled from ON->OFF->ON.

Your modem manual will explain more about this

and how to set up your modem to respond to DTR.

Return Value: None.

Also see: SetDTR()

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN   */

DropDTR(&controlBlock); /* Turns off DTR (Data Terminal Ready) */

}

Page 16 - ProModem Users Manual        Interactive Telecommunication Systems

Function: void SetDataFormat(AJMS *controlBlock, int options);

Description: Sets the Data format for a COM port. OpenCom()

automatically sets the COM port to 8 Bits, 1 stop,

no parity. To override this, use this function.

Return Value: None.

Example Usage:

/* BITS_7       - Sets COM port for 7 bits                */

/* BITS_6       - Sets COM port for 6 bits                */

/* BITS_7       - Sets COM port for 7 bits                */

/* BITS_8       - Sets COM port for 8 bits                */

/* STOP_BITS_1  - Sets COM port for 1 stop bit            */

/* STOP_BITS_2  - Sets COM port for 2 stop bits           */

/* PARITY_ON    - Sets COM port to use parity             */

/* NO_PARITY    - Sets COM port not to use a parity bit   */

/* EVEN_PARITY  - Sets COM port to use even parity        */

/* ODD_PARITY   - Sets COM port to use odd parity         */

/* STICK_PARITY - Sets COM port to use stick parity       */

/* SET_BREAK    - Sets COM port to set break              */

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously  */

/* been set up and the COM port is OPEN    */

/* Sets COM port for 8-N-1, simply Bitwise OR anything you want. */

SetDataFormat(&controlBlock, BIT_8 | NO_PARITY | STOP_BITS1);

}

Page 17 - ProModem Users Manual        Interactive Telecommunication Systems

Function: int SendString(AJMS *controlBlock, char *string);

Description: Sends a NULL terminated string to the COM port.

Return Value: CARRIER    - If there was a carrier while sending string.

NO_CARRIER - If there wasn't a carrier while sending string.

Also see: SendCharacter()

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously  */

/* been set up and the COM port is OPEN    */

SendString(&controlBlock, "\nHello World!"); /* Send string to COM port */

}

Page 18 - ProModem Users Manual        Interactive Telecommunication Systems

Function: SetFIFOMode(AJMS *controlBlock);

Description: Turns on the FIFO buffer for a COM port. This will only

work with a UART 16550.

Return Value: FIFO_ENABLED    - The FIFO has been enabled.

NO_FIFO_PRESENT - No FIFO present (16550 Uarts ONLY)

Also see: SetFIFOTriggerLevel()

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously  */

/* been set up and the COM port is OPEN    */

/* Try to enable the FIFO */

if (SetFIFOMode(&controlBlock)==FIFO_ENABLED)

printf("\nFIFO has been enabled.");

else

printf("\nNo FIFO Present, not a 16550.");

}

Page 19 - ProModem Users Manual        Interactive Telecommunication Systems

Function: SetFIFOTriggerLevel(AJMS *controlBlock, unsigned char mode);

Description: Sets the FIFO trigger level for the RCVR FIFO Interrupt.

This only works with a UART 16550.

Return Value: None.

Also see: SetFIFOMode()

Example Usage:

/* FIFO_1_TRIGGER  - IRQ will trigger with 1 byte in FIFO buffer   */

/* FIFO_4_TRIGGER  - IRQ will trigger with 4 bytes in FIFO buffer  */

/* FIFO_8_TRIGGER  - IRQ will trigger with 8 bytes in FIFO buffer  */

/* FIFO_16_TRIGGER - IRQ will trigger with 16 bytes in FIFO buffer */

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously  */

/* been set up and the COM port is OPEN    */

/* Sets FIFO to trigger on 16 characters in RCVR buffer */

SetFIFOTriggerLevel(&controlBlock, FIFO_16_TRIGGER);

}

Page 20 - ProModem Users Manual        Interactive Telecommunication Systems

Function: int WaitForCharacter(AJMS *controlBlock, double seconds);

Description: Waits for a character to be received in the IRQ receive

queue for a specified number of seconds.

Return Value: (int)         - Character received (high byte masked)

WAIT_TIME_OUT - Time out, no character received.

Also see: GetCharacter()

Example Usage:

#include "promodem.h"

example()

{

unsigned char ch;

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN   */

/* wait 3.5 seconds for a character, or timeout */

ch = WaitForCharacter(&controlBlock, 3.50);

if (ch==WAIT_TIME_OUT)

printf("\nTimeout, no character received.\n");

else

printf("\nHere is the character '%c'\n", ch);

}

Page 21 - ProModem Users Manual        Interactive Telecommunication Systems

Function: DisableIRQ(AJMS *controlBlock);

Description: Turns OFF the IRQ vector and restores it to the

original vector before the com port was opened.

This should ALWAYS be done if you are doing any spawning

that swaps your program to Disk or EMS.

Return Value: None.

Also see: EnableIRQ()

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN   */

DisableIRQ(&controlBlock); /* Restores IRQ vector to original  */

}

Page 22 - ProModem Users Manual        Interactive Telecommunication Systems

Function: EnableIRQ(AJMS *controlBlock);

Description: Sets up the IRQ vector to point to ProModem's serial

receive IRQ internal routine. This should only be used

if DisableIRQ() was previously called. OpenCom()

automatically sets up the IRQ vector when the com port

is originally opened.

Return Value: None.

Also see: DisableIRQ()

Example Usage:

#include "promodem.h"

example()

{

extern AJMS controlBlock; /* Assumes a control block has previously */

/* been set up and the COM port is OPEN    */

EnableIRQ(&controlBlock); /* Sets IRQ vector to point to ProModem's IRQ */

}

Page 23 - ProModem Users Manual        Interactive Telecommunication Systems

TERMINAL.C - Example terminal program using ProModem.

TERMINAL.C is a demo program written using the routines in the

ProModem library to demonstrate the library functions.

To build the program you need to have the PROMODEM.H file that came in

the archive and the PROM_TC.LIB or PROM_MSC.LIB to link with.

Once you have the correct LIBs and .H files in your INCLUDE and

LIB directories, compile and link:

nmake PROMODEM.MAK                   ;Microsoft QuickC

cl terminal.c /link PROM_MSC.LIB     ;Microsoft C

tcc terminal.c PROM_TC.LIB           ;Turbo C

TERMINAL.C is yours to do what you want with.

NOTE: With Microsoft compilers, you must <<TURN OFF>> the DEBUG flag

if it is set.

Page 24 - ProModem Users Manual        Interactive Telecommunication Systems

BBS.C - Example Bulletin Board System using ProModem.

BBS.C is a demo BBS program written using the routines in the

ProModem library to demonstrate the library functions.

To build the program you need to have the PROMODEM.H file that came in

the archive and the PROM_TC.LIB or PROM_MSC.LIB to link with.

Once you have the correct LIBs and .H files in your INCLUDE and

LIB directories, compile and link:

nmake BBS.MAK                   ;Microsoft QuickC

cl bbs.c /link PROM_MSC.LIB     ;Microsoft C

tcc bbs.c PROM_TC.LIB           ;Turbo C

BBS.C is yours to do what you want with.

NOTE: With Microsoft compilers, you must <<TURN OFF>> the DEBUG flag

if it is set.