P O W E R   S U P P L Y   D E S I G N   A S S I T A N T

from

Roy G. Caldwell

Roy's Super Toy Shop

P.O. Box 461942

Escondido CA, 92046-1942

P O W E R   S U P P L Y   D E S I G N   A S S I T A N T

R O Y ' S   S U P E R   T O Y   S H O P

This program will calculate the voltage divider required for

any power supply that use a 2.5 Volt reference, TL431 or equal.

to VS+                    This program disregards the current flowing

�                     into the TL431 ref. (5 to 10 �A.).

��                    It also disregards the voltage drop across

R9  � �                    any resistor in series with the

� �                    TL431(10 - 20 mVolts) and the

���                    resistor, if any, in series with

�����  to TL431       the TL431 ref. input ( 1.5K = 7.5 mV.).

��                    The TOTAL error introduced by disregarding both

R11  � �                    of the above resistors is less than 50 mV.

� �                    Also the 10 � VS resistors are shorted

���                    in normal use, so the voltage drop across

�                     these resistors is zero.

to VS-

This program requires a Voltage output greater than 2.74 Volts.

Enter output Voltage desired ?

Enter value for R11 [470 � ] ?

P O W E R   S U P P L Y   D E S I G N   A S S I T A N T

R O Y ' S   S U P E R   T O Y   S H O P

to VS+  actual:   5.000 desired: 5.000  Difference: 0.000 (0.00%)

�

��

R9  � �  470

� �

���

�����  to TL431

��

R11  � �

� �  470

���

�

to VS- (gnd)

You may change the value of R9 or R11 and view the results.

or you may add a resistor in parallel with either resistor.

Which Resistor do you wish to change R9 enter 9 or R11 enter 11

or enter P to parallel a resistor or Q to quit ?

If you select 9 or 11 the computer will calculate the new output

voltage and display the above screen with the new values.

If you select P you will be asked which resistor to parallel R9

or R11 and the value of the resistor to connect in parallel.

The following screen will be displayed.

P O W E R   S U P P L Y   D E S I G N   A S S I T A N T

R O Y ' S   S U P E R   T O Y   S H O P

to VS+ actual: 4.924 desired: 5.000 Difference: -0.076 (-1.25%)

������������������Ŀ

��                ��

R9  � �  470       R9P � �  15000

� �                � �

���                ���

������������������������  to TL431

��                ��

R11  � �           R11P � �

� �  470           � �   Open

���                ���

��������������������

to VS- (gnd)

You will again be asked which resistor to parallel R9 or R11

and the value of the resistor to connect in parallel.  The

screen will be re-displayed with the new values.

If you enter -1 for the value the resistor in parallel will

be removed and the value displayed will be 'Open'.

P O W E R   S U P P L Y   D E S I G N   A S S I T A N T

from

Roy G. Caldwell

Roy's Super Toy Shop

P.O. Box 461942

Escondido CA, 92046-1942

Suggestions are welcome.

Shareware version 1.0  4-24-94  1:00 pm

To register and receive the latest version send the REGISTRATION

FEE of $20.00 to the above address.

If you use this program to help design a power supply that will

be sold or that a company will put to any use, You MUST register

the program.

If you use this program to help design a power supply that

will be used by you in your home, You do not need to register

the program, But you could.

DISCLAIMER and WARRANTY:

The   programmer, Roy G. Caldwell, and Roy's Super Toy  Shop

makes no  warranty,  expressed  or implied as to the  fitness  of

this program for any purpose.

In   no  event  shall the Author be liable to  you  for  any

damages,  including  any  lost profits, lost  savings,  or  other

incidental or consequential damages arising out of the use of  or

inability   to  use  this program, even if the  Author  has  been

advised  of the  possibility of such damages, or for any claim by

any  other party.

By  using  this  program you agree to  the  above   Warranty  and

conditions.

LATEST VERSION

If you need the latest version or you want to get your program

direct from the author, please send a self addressed label in a

disk  mailer  (please write VOLTDIVP and disk size (360K,  720K,

1.2M,  1.44),  I do not support 2.88, on the inside flap  of  the

mailer) with stamps for return postage plus $3.00 handling to

Roy's Super Toy Shop

P.O.Box 461942

Escondido CA. 92046-1942

If you send check or money order make payable to Roy G. Caldwell

PERMISSION TO COPY

Individuals, BBS's, clubs and other organizations are granted

permission  by  the  author to freely copy  and  distribute  this

program and its documentation as long as:

1:  There is no charge for the software or documentation.  Howev-

er, you may charge a service fee for disk duplication and distri-

bution, as long as such fee is not more than $5.00, not including

mailing cost, or $10.00 per disk as this is a small program  that

can be included with other programs on a diskette.

BBS's  that  charge fees for use of the BBS may  distribute  this

program  but if a BBS has a charge for downloads, that  BBS  MUST

contact  the  author before placing this program on  the  system.

Please specify the exact method you use to determine the  charges

for  downloading  files  and estimate the cost to  the  user  for

downloading  this program.  You may contact me by writing to  the

above address.

2.  The program or its documentation are not modified in any way.

3.  All  files  are distributed together on  the  same  diskette

(Preferably in a ZIP, ARJ, or ARC file).

4.  No files may be deleted. There are four files:

VOLTDIVP.EXE,  VOLTDIVP.DOC (this file), LONG, SHORT

5.  This program is not bundled with any hardware.

SHAREWARE DISTRIBUTORS

See above info on latest version.  Please adhere to #2-5 above.