***********************************************************

VGAPRTSC.ASM   ver 1.3   February 1989

Hardcopy Utility for VGA-(and EGA-) Graphics Modes

Robert Kaempf

<DB3D@DDATHD21>

Institut fuer Kernphysik

Technische Hochschule Darmstadt

6100 Darmstadt, West Germany

***********************************************************

VGAPRTSC is similar to the MSDOS program GRAPHICS.COM.

The difference is: GRAPHICS supports low resolution

graphics modes (320x200,640x200), VGAPRTSC supports high

resolution graphics modes (640x480,640x350).

Text or low resolution graphics modes cause a call to the

old hardcopy routine. It is possible to install GRAPHICS

before VGAPRTSC. This will support any graphics modes.

Parameter:

/C= followed by 16 digits

The 16 digits correspond to the 16 screen colors.

An odd digit selects the color to be printed,

colors with even digits are not printed.

Example:

VGAPRTSC /C=0111111111111111   ( default )

prints colors 1 to 15.

VGAPRTSC /C=0000222211113333

prints highlighted colors ( 8 to 15 )

You may modify the colortable of an installed VGAPRTSC

version by typing: VGAPRTSC /C=... .

VGAPRTSC will not be installed twice !

---------------------------------------------

ADDITIONAL NOTE BY OH5IY:

I have modified the code to suit the Cannon BJ-10 bubble jet printer.

If you have a problem which can not be solved by chancing printer

setups, use pctoools to edit vgaprtsc.com file.

There is on the 73. byte a 06 hexcode after 1B 41, cgange the value of 06

to smaller if there is a gap between printing lines, larger if there

is compression in the printout. Notice that the printinglines run VERTICALLY

if you look the print sheet and the gaps or compression is also

vertically oriented on the paper.

(I hope this will help you. OH5IY @OH5RBG.FIN.EU)