File:  PPPPKT.DOC

PPPPKT Ver 0.50   Copyright 1994  -  [totally] ������ Software

Usage:

=====

C:\> PPPPKT [/Option Value ][/Option Value ]...

Options [default]  (any order,  / = DOS SwitchChar)

b - Board [1]                   v - Interrupt Vector [60h]

i - IP number [0.0.0.0]         g - Gateway [0.0.0.0]

m - Subnet Mask [func(i,g)]     t - Interface Type [80]

? - Help [off]                  d - Disable nodeaddress door

Numbers (max. 255) specified as:  either  0xNN, NNh (hex)  or  NNN (dec).

Copyright, Licence, Warranty Disclaimer:

=======================================

See the FINE PRINT section at the end of this document.

Document Organization:

=====================

Synopsis.                       PPPPKT is ...

Options.                        Complete summary.

Getting Started.                Overview.

Detailed Comments.              Underview.

Technical Summary.              Oh, is that right ?

Caveats.                        PPPPKT is *not* ...

Acknowledgements.               Without whom ...

FINE PRINT.                     Read this !

Synopsis:

========

PPPPKT is a TSR `shim' or `adapter' providing a Class 1

ethernet packet driver interface over the Novell LAN

WorkPlace for DOS SLIP_PPP driver.  It is suitable for use

over those PPP or SLIP links which carry only IP packets.

PPPPKT lets you use certain applications, which were

designed to be used on an ethernet connection, over a PPP

(or SLIP) connection.  This combines the benefits of the

stable and fully-featured Novell SLIP_PPP driver with a

minimum of memory overhead.  A typical installation of

the necessary TSR's looks like this:

Psp   Size   Name       Description        Hooked Vectors

----  ------ ---------- ------------------- --------------

1670  18,592  LSL       Link Support Layer    08 2F E6

1AC7  23,648  SLIP_PPP  ODI Link Driver       0C E1

208E   1,536  PPPPKT    Adapter               60

for a total memory usage of about 43 Kbytes.

The overall performance varies with the application.  It

may work very well, may work slowly (either passably or

unacceptably), or it may not work at all.  Your mileage

depends on the application; you have to test drive them.

The file APPHINTS.TXT contains performance evaluations,

configuration hints and ftp sources for some TCP/IP

applications that have been tested with PPPPKT.

Why use PPPPKT instead of an ODI-based application?

Some of your favourite applications may not have been

ported to ODI, or the ODI version may be unsuitable.

Options:

=======

The options supported by PPPPKT must appear after the

DOS SwitchChar (represented here by '/' as is customary).

All of the options are followed by one or more numerical

parameters.  These numbers may be specified in either hex

or decimal form.  The hex forms 0xNN, 0NNh and NNh are

all recognized.  A brief summary of the options is

presented first, followed by a more detailed discussion.

(The IP addresses are examples; use your own.)

Option        Meaning                 Example

------        -------                 -------

/b <board#>   network logical Board   /b 1

/v <vector>   software interrupt      /v 60h

Vector

/i <ip#>      IP address of your PC   /i 130.244.17.25

/d            Disable nodeaddress door

/g <gateway>  IP address of your      /g 132.244.12.9

internet Gateway

/m <mask>     subnetting Mask         /m FFh.FFh.0.0

/t <type>     interface Type          /t 14

Board:    This is the logical board number for the PPP frame

provided by the SLIP_PPP driver, in the context of

Novell LAN WorkPlace (LWP) and the NET.CFG file.

Board numbering starts at 1 (not 0), as per the ODI

specification.  Note that PPPPKT does not have a

NET.CFG entry, but if it did this would be the board

that PPPPKT "bind"s to.

DEFAULT:  1.

Vector:   The software interrupt vector to be hooked by PPPPKT.

Must be in the range 60h ... 80h.  Typically interrupts

60h through 67h are reserved for user purposes; it's

a good idea to choose a free one in this range.

DEFAULT:  60h.

IP#:      The IP (Internet Protocol) address for your end of

of the PPP connection.  If you are using a static

address, it will be assigned to you by your network

administrator.  If you use dynamic address negotiation,

the address varies from one connection to the next, and

is assigned to you by the remote software from a pool

of addresses.  The IP# used by PPPPKT is determined by

the following hierarchy, listed from highest to lowest

precedence:

(1)  Command line /i option.

This is useful for dynamic addressing, or for

static addresses once put in a batch file.

(2)  Taken from the NET.CFG file.  [If /i absent.]

You can specify the IP# by adding an appropriate

"node address" line to the "Link Driver SLIP_PPP"

section of your NET.CFG file.  Use:

node address       NNNNNNNN0000

where NNNNNNNN is your IP#, in hex, with no dots.

For example 130.244.17.25 leads to the entry

node address       82F411190000

This is a convenient `back door' if your IP# is

fixed.

(3)  Taken direct from memory image of SLIP_PPP TSR.

[If "node address" is 0 or absent from NET.CFG,

or if "/d" flag is set.]  Currently this method

is a bit of a kluge, but is very useful when it

works.  The dword at offset 5112h in the image

of SLIP_PPP v4.1 (R41-1, 921113) seems to hold

the IP number after a PPP connection is made.

This may not be universal so it is given least

priority.  Try if it works for you (see the

"Detailed Comments" section).

Disable:  This flag turns off the "node address" doorway

(method (2) above) for specifying your IP#.  This is

only of interest if method (3) works.

DEFAULT:  no flag.

Gateway:  The IP address of a local computer that connects

your university, organization, or internet provider

to the rest of the internet.

DEFAULT:  0.0.0.0

Mask:     The subnetting mask, to be used by your ethernet

application over the PPP link.  It is determined

automatically by PPPPKT based on your PC's and

gateway's IP numbers.

DEFAULT:  function of <ip#> and <gateway>.

Type:     Packet driver interface type, within Class 1 of the

FTP Software spec, which PPPPKT reports to applications

in response to a driver_info() function call.  Normally

you need *not* use this option.  Possible types of

interest:

14  -- used by Merit's ETHERPPP

71  -- ODI to packet driver adapter (ODIPKT)

80  -- SLIP simulated ether

This option does not affect the actual operation of

PPPPKT in any way.  It is provided as a convenience in

the unlikely event that an application will configure

itself for improved performance with a particular type.

DEFAULT: 80.

Note that, at a minimum, you *must* provide your IP#

and gateway information to PPPPKT.  It is crucial that

the IP# be correct, and that the IP#, gateway and netmask

parameters used by PPPPKT and to configure applications

be identical.

The space between an /Option and the parameter following

it is optional.  However you must leave a space before

each / delimiter.  The command line is case-insensitive.

Users of DOS 3.3 (truly `the best DOS ever') will be

pleased to know that a user-defined SwitchChar is supported.

Getting Started:

===============

This section is an overview.  Further details appear in

the next section.

In order to use this software, you need:

*    A PPP or SLIP account for internet connection.  For

brevity, this document usually refers just to PPP.

*    If you are using a dial-up connection over a modem,

you'll need a communications, or dialer, program to

initiate, and close, the connection.

*    Two Novell LAN WorkPlace for DOS programs:

LSL.COM       ...     Link Support Layer

SLIP_PPP.COM  ...     Link Driver

For testing, the following are helpful:

TCPIP.EXE     ...     ODI TCP/IP protocol stack

PING.EXE      ...     ODI ping application

*    Properly configured NET.CFG file.

*    TCP/IP application programs that use a Class 1 packet

driver.  They must be properly configured.  They must use

no protocols other than IP, ARP and RARP.

*    A  TERMIN.COM  program is helpful, to terminate/unload

PPPPKT (i.e. remove it from memory).

Once you have obtained all the necessary software, a

simple outline of one way to proceed is the following:

o   Create a directory to hold the Novell programs listed

above, your NET.CFG file, TERMIN and PPPPKT.  Even if

you're a current user of LWP, it is a good idea to keep

these things separate, so as not to `break' your existing

setup.  Let's call this directory C:\PPP here, to be

definite.

o   Prepare a NET.CFG file, specially for PPP, in this

directory.

o   Go through a `dry run.'  From this directory load

LSL

SLIP_PPP

PPPPKT /g <gateway> /i <ip#> [ ... ]

Do not use the /m option here!  For dynamic addresses,

use a typical value of the <ip#>.  Note down the "Mask"

parameter reported by PPPPKT when it loads.  It will

look something like this:  FF.FF.00.00 hex.  (On the

command line, FF would be written FFh, 0xFF or 255.)

Unload the TSR's with

TERMIN 0x60

SLIP_PPP u

LSL u

o   Determine your <netmask> parameter.  The details of

this depend on whether you have a static or dynamically

assigned IP#.

For a static (fixed) IP#, use the value noted above

to configure the netmask in applications.  You don't

need to use the /m option when loading PPPPKT, since it

will always be computed for you.

For dynamic addresses there is a very slight potential for

difficulty because the netmask must be consistent with all

of the IP numbers that might be assigned.  Exactly what

this `consistency' means is discussed in the "Technical

Summary" section.  Here are two simple ways to proceed:

1.  If your PPP provider or remote software tells you what

netmask to use, try using that value.

2.  Use the netmask noted above in the dry run.  It

should work in virtually all circumstances.

If this does not work, applications will not believe they

are on an ethernet, and won't send packets anywhere, etc.

In general the netmask, gateway IP, and your IP# must be

mutually consistent.  Given your gateway, you must pick a

suitable netmask which will work with any IP# that you

could be assigned.  To do this by hand, take the netmask

calculated by PPPPKT, and replace one or more of the

rightmost 1's in its binary representation by 0's.  Use

this value with the /m option when loading PPPPKT, and to

configure applications, until things are consistent and

working.  For example, suppose PPPPKT calculated the

netmask FF.FF.C0.00 hex, and it did not work.  Then you

should try FF.FF.80.00 hex, followed by FF.FF.00.00 hex,

etc.

o   Configure your application program(s).  Depending on

the app, this may involve editing a configuration file,

running the program and going into a Setup menu, etc.

The main point to be made, at this time, is that when

configuring the `subnetting mask' option for the app, you

should use the value determined as indicated above.  Also

be sure the gateway IP and your IP# are the same as used

to load PPPPKT.

o   Either dialing `by hand' or with a script, get your

communications package to dial the phone, log in to the

PPP account, and get the remote host to the point of

negotiating the PPP connection.  Then exit the

communications program without closing the connection.

o   CD to C:\PPP and load LSL, SLIP_PPP and TCPIP in that

order.  Wait 5 - 10 seconds and try to ping a local

machine using its IP address (each NN represents a

number in the range 1..255) :

PING NN.NN.NN.NN

Do not use a domain name here.  If the connection is

working, PING should reply with something like:

NN.NN.NN.NN is alive

Also, running SLIP_PPP again with the "r" flag

SLIP_PPP r

will show some statistics indicating if packets have

been sent and received.

o   Unload the TCPIP stack; it is not needed further:

TCPIP u

o   For users of dynamic IP addressing, determine your

IP# for the session, if necessary by using SLIP_PPP's

"c" flag:

SLIP_PPP c

It's the 8-digit hex number under "Local IP address".

See also the "IP# Specification Strategy" part of the

next section.

o   Load PPPPKT.

PPPPKT  /g <gateway> [/i <ip#> /m <netmask> ...]

o   Run your application(s).

o   To close the session, unload the TSR's as indicated

above and hang up the phone.

Detailed Comments:

=================

* Communications.

You can try whatever package you are familiar with, for

example MS-Kermit, Procomm, Telix, etc.  The advantage of

MS-Kermit is that it is both free, and (Ver. 3.13)

supports TCP/IP in a way that is relatively easy to

configure.  Thus, telnet inside MS-Kermit can be one of

your applications.  To get started quickly, though, stick

to a package you know.

There are also a number of `dialer' programs available

on internet which may do what is needed.  For example,

COMTOOL in:

mvmpc9.ciw.uni-karlsruhe.de:/nos/sliplog/sliplog.zip

* Novell LAN WorkPlace Programs.

The information related to this subsection is provided

merely as a curiosity, and may be incorrect.  It is

entirely the responsibility of the potential user (you)

to determine and meet all legal requirements for the use

of the Novell software.  Novell LWP for DOS is a commercial

networking package, and is copyright by Novell, Inc.

If you are a current LWP user then you might already

have the necessary files listed above, and you may be

interested to know that updated current versions are

available on Novell's ftp site:

ftp.novell.com:/pub/pub2/UNIXCONN/LWDOS42/

The necessary files are also available in a sample

package.  It is apparently for general distribution to

all who are interested.  Try:

sjf-lwp.novell.com:/lwp4dos/ppp_sample/pppsampl.zip

/old/lsl.com

slip_ppp.com

or:

novell.felk.cvut.cz:

/appl/pub/mirrors/netwire/sjf-lwp/lwp4dos/ppp_samp/pppsample.zip

Compatible versions of the Novell programs are:

LWP for DOS Ver.          Sample Ver.

LSL         v1.21 (920130)            v2.00 (920904)

SLIP_PPP    v4.1 (R41-1, 921113)      v4.1 (R41-1, 921113)

TCPIP       v4.12 (R412-2a, 940404)   v4.1 (R41-1h, 930402)

PING        v4.12 (R412-2a, 940404)   v4.1 (R41-1, 921113)

If you use LAN WorkPlace for WINDOWS, you must obtain

the DOS drivers.  Store, and run, them separately from

the Windows drivers.  The latter will not maintain the

PPP connection by themselves, as they require Novell's

Windows dialer support.

If you will use PPPPKT in tandem with TCPIP.EXE, please

read this paragraph.  There are three types of ODI

protocol stacks: prescan, bound and default.  As the names

suggest, a prescan stack is passed all incoming packets

and chooses which if any it will consume, bound stacks

receive specific protocols not eaten by a prescan stack

(if present), and finally a default stack is handed any

packets not used by prescan and bound stacks.  This

version of PPPPKT is a default stack.  It can remain

loaded in memory, with TCPIP.EXE taking control of IP

packets when it is loaded too.  I.e., if both are loaded

simultaneously, only TCPIP.EXE will work.

* NET.CFG.

Here is a relatively minimal example NET.CFG file for

use with PPP and PPPPKT.  A similar file, NET_CFG.SAM,

with most of the comments removed is included with the

PPPPKT package.  Copy it to NET.CFG and edit it with your

own parameters.

;---------

; C:\PPP\NET.CFG:

Link Support

Max Boards      1       ; if only 1 logical board

Max Stacks      3       ; enough for PPPPKT and TCPIP.EXE

MemPool         1024    ; only for TCPIP.EXE, size as needed

Buffers         8 1600  ; *crucial*

Link Driver SLIP_PPP

int     4               ; Hardware IRQ for serial port

port    3E8             ; I/O address for serial port

node address    NNNNNNNN0000    ; <ip#>0000, hex, no dots

baud    38400           ; DTE CONNECT speed

mru     1500

accm    00000000        ; Async Control Character Map

accomp  yes             ; Address&Control field compression

pcomp   yes             ; Protocol field compression

open    active

tcpipcomp vj

ipaddr  NN.NN.NN.NN     ; <ip#> (or 0.0.0.0 for dynam addr)

frame   PPP                     ; board #1

Protocol IP     0800    PPP

Protocol TCPIP

PATH SCRIPT     C:\NETWORK\LWP\SCRIPT   ; For LWP users

PATH PROFILE    C:\NETWORK\LWP\PROFILE  ;      "

PATH LWP_CFG    C:\NETWORK\LWP\HSTACC   ;      "

PATH TCP_CFG    C:\NETWORK\LWP\TCP      ;      "

ip_router       GG.GG.GG.GG             ; <gateway>

ip_netmask      255.MM.MM.0             ; <mask>

ip_address      NN.NN.NN.NN             ; your <ip#>

tcp_window      4096

tcp_sockets     12

udp_sockets     8

raw_sockets     1

bind            SLIP_PPP

;---------

Explanation of some of these entries may be helpful.

o Max Boards:  Normally you will only have one logical

board, so setting this to 1 saves a bit of memory.

o Max Stacks:  3 is enough to run PPPPKT and TCPIP

together.  To load more protocol stacks, increase this.

o MemPool:  This entry is needed only by TCPIP.EXE,

minimum of 1024.  Omit it for just PPPPKT.

o Buffers:  PPPPKT requires overhead space beyond the

size of an ethernet packet's data, in order to work with

ODI.  It is critical to have sufficiently many buffers of

adequate size, say 1600 bytes, *not* 1500.

o int, port:  The example is set for COM3.  Use the

values for the COM port your modem / serial port is on.

The standard COM port assignments are:

COM1    COM2    COM3    COM4

int       4       3       4       3

port    3F8     2F8     3E8     2E8

o node address:  This is the `back door' discussed in

the "Options" section; a way to tell PPPPKT your IP#

without using the command line.  Use your hex IP# without

0x or h, dots removed, and with 0000 appended.  Don't

use this entry for dynamically assigned IP#'s.

o baud:  Speed of the serial port connection, i.e. modem

to PC CONNECT speed.  Don't confuse with the modem-modem

CARRIER speed.  With a buffered UART the CONNECT speed

can be higher than the CARRIER speed.  Typical baud

values: 9600, 14400, 38400, 57600.  In a multi-tasking

environment, be conservative.

o ipaddr:  your IP#, if fixed (static), or set this to

0.0.0.0 if your account uses dynamic address negotiation.

o frame:  This identifies the frame type, PPP or SLIP.  This

line also implies the ODI logical board number, and thus

determines the  /b <board>  parameter for PPPPKT.  Usually

there will be only one frame type and <board> = 1.  If you

are using other frame types, <board> is determined in the

usual way.  Again, board numbering starts at 1, not 0.

o Protocol TCPIP section:  For use with TCPIP.EXE, the NET.CFG

file should also contain this section.  This is used if

toggling between running ethernet app's through PPPPKT and

ODI app's via TCPIP.EXE.  For example, to test the link with

TCPIP.EXE and PING as described above, this section is needed.

Important:  do *not* include any comments in this section.

The first 4 lines (capitalized) are only relevant to LWP

users, who will have an existing NET.CFG file containing

similar lines.

* IP# Specification Strategy.

What is the best of the three ways to specify your IP#

to PPPPKT?  This depends on your situation.  If the direct

method, (3), works for you, it is the most convenient.  To

check if it works, bring up a PPP connection, run

SLIP_PPP c

and note your IP#, then load PPPPKT with only the /d flag

PPPPKT /d

(Alternatively, run the included SHOWIP.COM.)  If the IP#

reported by PPPPKT when it loads (or by SHOWIP) agrees with

the previous one, method (3) is working.  (Be sure to check

this if you change your PPP setup.)  In this case you can

load PPPPKT with no /i option and no "node address" line,

or use the /d flag if you do keep the "node address" line.

If method (3) does not work, then if you use:

- Only a static ip address:  use either the "node address"

doorway (2) or set /i within a batch file.

- Only dynamic addresses:  set the IP# by hand each time

using the /i option.

- Both static and dynamic addresses:  use /i for dynamic

addresses, and a "node address" line for static.

* Application Configuration.

Typically you must supply the application with some of

the configuration information discussed above, as well as

other things.  Remember to use the `Mask' value determined

as discussed above and, used when loading PPPPKT.  This

ensures that the app's TCPIP stack can contact all internet

sites.

If the app supports RARP, that may be used by the app

to determine the IP address.  This is particularly

convenient in the case of dynamic address assignment.

PPPPKT provides replies to RARP requests using the <ip#>

parameter you load it with.  The included program SHOWIP.COM

can also be useful in setting up config files when dynamic

addresses are used.  See SHOWIP.DOC.

Other parameters such as various server addresses must

be obtained from your internet provider or networking

guru.  TCP parameters like max. segment size, max.

transmission unit, and receive window, may be required.

Configuration varies from app to app; see the app's

documentation for help.  Also, see APPHINTS.TXT.

* TERMIN.COM .

A suitable terminate program is part of the Crynwr

Software packet driver collection.  Try:

boombox.micro.umn.edu:/pub/pc/packet-drivers/programs/termin.com

or for the whole collection:

oak.oakland.edu:/SimTel/msdos/pktdrvr/

Technical Summary:

=================

PPPPKT conforms to revision 1.09 of the FTP Software

packet driver spec.  It is a Class 1 driver (Ethernet_II

or DEC/Intel/Xerox `Bluebook' Ethernet), of type 80

(SLIP-simulated ether) by default.  Its functionality

is 5 (basic & high-performance), *however* the only

high-performance function supported is get_parameters().

It supports no extended functions.

From the link driver perspective, PPPPKT is an ODI

default protocol stack.

PPPPKT emulates a local ethernet interface to the

application software, despite running over a PPP link.

The machines deemed to be on this virtual ethernet are

determined by the netmask.  The netmask computed by PPPPKT

is a function of the supplied <ip#> and <gateway> via a two

stage process.  First, a value is obtained based just on

the class (i.e. class A, B or C) of the <ip#>, specifically

class A   ==>   255.0.0.0

class B   ==>   255.255.0.0

class C   ==>   255.255.255.0

If this value is consistent with the gateway's IP number,

that is, if

( <netmask> & <gateway> ) = ( <netmask> & <ip#> )

[where & represents logical AND]

then this value is used as the netmask.  (See RFC 950.)

Otherwise, a different netmask value is chosen:  it is the

largest number which (a) is consistent in the above sense,

and (b) has a binary form of 1's followed by 0's.

In actual fact, the <gateway> IP number can usually be

chosen to be just about anything.  It need not even belong

to a real machine, provided that no application attempts to

send a datagram addressed *at the IP level* to the gateway.

What is important is that the <gateway>, <ip#> and <netmask>

parameters form a consistent set, so that applications

believe they can send and receive packets to and from the

gateway *at the ethernet level*.  For safety and to avoid

confusion, however, it seems best to set <gateway> to a

suitable real machine.

PPPPKT allows applications to register any protocols

having 2-byte ethertypes.  *However*, all protocols

except for IP, ARP and RARP are ignored.  An application

making essential use of other protocols will not function.

For example, you cannot directly receive or transmit IPX

packets.  Because of this 2-byte restriction, some older

versions of TERMIN.COM will not work.

When appropriate, PPPPKT internally generates replies

to ARP and RARP packets sent by the application, while

all IP packets are sent out over the link.  Packets

received from the link are expected only to be IP in the

situations where PPPPKT will be used.  Incoming non-IP

packets are discarded, while IP packets are passed to the

app's receiver.

The application's receive handler for ARP and RARP

packets must be capable of handling replies before the

corresponding send routine has terminated.

Caveats:

=======

-   PPPPKT is not a "PPP class" packet driver (Class 16

or 18 of revision 1.11 of the FTP Software spec.)

-   PPPPKT is not designed to, and will not, function with

ODI MLID's other than SLIP_PPP.

-   The emulation of ethernet over a PPP link inevitably has

shortcomings, some of which are mentioned in the above

section.  Clearly, ethernet-speed connectivity over

dialup lines will also not be achieved with current modem

technology.  If the application is sensitive to such

timing matters, problems can arise.

-   Despite the fact that PPPPKT is not `all things to

all applications' it can be very useful depending on the

circumstances.  It was written because the need for a

program which fills this niche was felt.

Acknowledgements:

================

None of the following people or organizations are in any

way responsible for maintaining and/or supporting PPPPKT.

Their indirect assistance in the form of publicly available

documentation and code is gratefully acknowledged here.

These materials, together with the RFC collection, were

essential to the development of PPPPKT.

(1)    Portions of PPPPKT's code originate from ODIPKT version 2.4.

ODIPKT.ASM is:

; (c) Copyright Daniel D. Lanciani 1991-1993.  All rights reserved.

;

; This unmodified source file and its executable form may be used and

; redistributed freely.  The source may be modified, and the source or

; executable versions built from the modified source may be used and

; redistributed, provided that this notice and the copyright displayed by

; the exectuable remain intact, and provided that the executable displays

; an additional message indicating that it has been modified, and by whom.

;

; Daniel D. Lanciani releases this software "as is", with no express or

; implied warranty, including, but not limited to, the implied warranties

; of merchantability and fitness for a particular purpose.

;

; Please send bug reports to ddl@harvard.harvard.edu or

;

; Dan Lanciani

; 185 Atlantic Road

; Gloucester, MA 01930

; (508) 283-4974

Please do *not* send PPPPKT bug reports to Dan Lanciani.

(2)    The LAN WorkPlace for DOS package and the Open Data-Link

Interface specification were developed and are Copyright by:

Novell, Inc.

122 East 1700 South

Provo, Utah 84606

(3)    The FTP Software packet driver specification was

developed and is Copyright by:

FTP Software, Inc.

2 High Street

North Andover, MA 01845

(508) 685-4000

(4)    NOTE:  PPPPKT is archived using Info-ZIP's compression

utility.  Info-ZIP's software (Zip, UnZip and related

utilities) is free and can be obtained as source code or

executables from various bulletin board services and

anonymous-ftp sites, including CompuServe's IBMPRO forum

and  ftp.uu.net:/pub/archiving/zip/*  and SimTel:

oak.oakland.edu:/SimTel/msdos/zip/* .

(5)    Thanks to the beta testers, for their time and helpful

comments.

(6)    Prof. Balram Bhakar, University of Manitoba, re-kindled

interest in the Novell ODI drivers.

FINE PRINT:

==========

All products mentioned in this documentation which are

patented, copyrighted or are trademarks are the property

of their respective owners.  Novell is a registered

trademark of Novell, Inc.  ODI, LSL and MLID are

trademarks of Novell, Inc.

PPPPKT is Copyright 1994 by Frank Molzahn.

All applicable rights reserved.

PPPPKT is smilley-ware.  If you think this software is

worth using (or maybe you just like the DOC), why not

send the author your smilley?

PPPPKT may be freely distributed provided all the files in

the package are present and unmodified.  This specifically

includes the files

APPHINTS.TXT

NET_CFG.SAM

PPPPKT.COM

PPPPKT.DOC

README.1ST

SHOWIP.COM

SHOWIP.DOC

This copyright, licence and warranty disclaimer notice

applies to the entire PPPPKT package.  If you are unsure

whether or not your PPPPKT is intact, download the current

version from SimTel.

PPPPKT IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND,

either expressed or implied, including, but not limited

to, the implied warranties of merchantability and fitness

for a particular purpose.  The entire risk as to the

quality and performance of the product is with the user.

This documentation explicitly states that PPPPKT will not

work properly in some circumstances; the user assumes the

cost of all necessary servicing, repair or correction.

In no event will Frank Molzahn be liable for damages,

including any general, special, incidental or consequential

damages arising out of the use of, or inability to use,

PPPPKT (including but not limited to loss of data or data

being rendered inaccurate or losses sustained by the user

or third parties or a failure of the product to operate

with any other programs), even if the author has been

advised of the possibility of such damages.

These copyright, license, and disclaimer notices must be

included with all copies of PPPPKT.

You may use PPPPKT if, and only if, you have read,

understood and accepted all of the above conditions.

Please contact me to report bugs or errors in the document

(even minor ones).  If you are an application developer or

ODI/packet programmer and have constructive suggestions for

improving the performance of PPPPKT I'd be glad to hear

from you.

[totally] ������ Software

is:

Frank Molzahn

7 Denton Place

Winnipeg, MB

Canada

R3R 2Z9                         molzahn@cc.umanitoba.ca