MACRO CROSS ASSEMBLERS

Series I

USER'S MANUAL

for the following microprocessors:

8044

8048

8051/52

8080

8085

8096

Z80

MICEPT INSTRUMENTS INC.

Macro Cross Assembler - Series I                               ii

MACRO CROSS ASSEMBLERS

Series I

USER'S MANUAL

Edition 2

MICEPT INSTRUMENTS INC.

377 Julien St.

Cap De La Madeleine, PQ

Canada  G8T 6W6

e-mail: jveillet@aei.ca

Macro Cross Assemblers - Series I                             iii

NOTICE

This version of  the software is  distributed as shareware.  This

version may  be  freely  copied and  distributed  for  evaluation

purposes as  long  as  NO  FEE IS  CHARGE  FOR  USE,  COPYING  OR

DISTRIBUTION (EXCEPT FOR A NOMINAL DISTRIBUTION FEE) and as  long

as THE SOFTWARE PACKAGE IS NOT MODIFIED IN ANY WAY.

USE OF  THIS  SOFTWARE  BEYOND  A  REASONABLE  EVALUATION  PERIOD

REQUIRES REGISTRATION.

The  entire   package,   consisting   of   the   main   programs,

documentation file and  various utility files  are all  copyright

(C) 1991-1996 by Micept Instruments Inc. All rights reserved.

DISCLAIMER

Micept Instruments  Inc.  disclaims  all warranties  as  to  this

software and  the  documentation,  whether  express  or  implied,

including,  without   limitation,  any   implied  warranties   of

merchantability  and   of  fitness   for  any   purpose.   Micept

Instruments Inc.  assumes no  liability  for damages,  direct  or

consequential, which may results from the use of this software or

the documentation.

Information in this document is subject to change without notice.

TRADEMARKS

Intel is a registered trademark of Intel Corporation.

Zilog and Z80 are registered trademarks of Zilog Inc.

MS-DOS is a trademark of Microsoft Corporation.

IBM, PC, XT, AT are trademarks of International Business Machines

Corporation.

All  brand  and  product  names  are  trademarks  or   registered

trademarks of their respective companies.

TABLE OF CONTENTS

TABLE OF CONTENTS..............................................iv

1. INTRODUCTION.................................................1

1.1 ABOUT THIS MANUAL...........................................1

1.2 OVERVIEW....................................................1

1.3 SPECIFICATIONS..............................................1

1.4 NOTATIONAL CONVENTIONS......................................2

1.5 LEARNING MORE ABOUT THE MICROPROCESSORS.....................2

2. GETTING STARTED..............................................3

2.1 INSTALLATION................................................3

2.2 SYSTEM REQUIREMENTS.........................................3

2.3 DISK CONTENTS...............................................3

2.4 SETTING UP THE ENVIRONMENT..................................3

3. ASSEMBLING...................................................4

3.1 RUNNING THE ASSEMBLER.......................................4

3.2 OPTIONS IN MORE DETAILS.....................................4

3.3 ERROR CODES RETURNED TO DOS.................................6

3.4 OBJECT CODE FILE FORMATS....................................6

4. ASSEMBLER LANGUAGE...........................................7

4.1 SOURCE LINE FORMAT..........................................7

4.2 EXPRESSIONS IN THE OPERAND FIELD............................8

4.2.1 Symbols .................................................8

4.2.2 Constants ...............................................8

4.2.3 Operators ...............................................9

4.2.4 How expressions are evaluated ...........................9

4.3 ASSEMBLER DIRECTIVES.......................................11

ACLIST .......................................................13

DB ...........................................................13

DW ...........................................................13

ELSE .........................................................13

END ..........................................................13

ENDIF ........................................................14

ENDM .........................................................14

EQU ..........................................................14

EXITM ........................................................14

EXPAND .......................................................14

FAIL .........................................................15

FLIST ........................................................15

FNOLIST ......................................................15

IF ...........................................................15

IFC ..........................................................15

Macro Cross Assemblers - Series I                               v

IFN ..........................................................16

IFNC .........................................................16

INCLUDE ......................................................16

LIST .........................................................16

LLEN .........................................................17

MACRO ........................................................17

NOACLIST .....................................................17

NOEXPAND .....................................................17

NOLIST .......................................................17

ORG ..........................................................18

PAGE .........................................................18

PLEN .........................................................18

REDEF ........................................................18

RSB ..........................................................18

RSW ..........................................................19

SET ..........................................................19

TITLE ........................................................19

4.4 CONDITIONAL ASSEMBLY.......................................19

4.5 MACRO ASSEMBLY.............................................20

5. ERROR MESSAGES..............................................23

5.1 COMMON ERRORS..............................................23

5.2 WARNINGS...................................................29

5.3 FATAL ERRORS...............................................30

6. SPECIAL SECTION ON THE MICROS...............................31

6.1 INTEL 8044.................................................31

6.2 INTEL 8048.................................................34

6.3 INTEL 8051/52..............................................35

6.4 INTEL 8080.................................................39

6.5 INTEL 8085.................................................39

6.6 INTEL 8096.................................................39

6.7 ZILOG Z80..................................................40

7. BUG REPORTING PROCEDURE.....................................41

8. ORDERING INFORMATION........................................42

INDEX..........................................................45

1. INTRODUCTION

1.1  About this manual

This manual  provides  the  necessary  technical  information  to

install and use the Macro Cross Assemblers in the series I. It is

written for  the individual  end user  but a  certain measure  of

experience in the operation of MS-DOS has been assumed. A certain

knowledge of the assembly language of each microprocessor is also

assumed.

Most of this manual  is intended for  the registered versions  of

the  software  and  therefore  not  everything  in  it  will   be

applicable to  the  unregistered  versions.  Macros,  conditional

assembly and included files are not supported in the unregistered

versions.

1.2  Overview

This software are  high quality  Macro Cross  Assemblers for  the

Intel 8044, 8048, 8051/52,  8080, 8085, 8096,  and the Zilog  Z80

microprocessors. This  software  are  basic  two-pass  assemblers

which supports macros, conditional assembly and included files.

These powerful Macro  Cross Assemblers for  the MS-DOS  operating

system support the complete instruction set and a complete set of

directives. They also  support  the  8051/52  and  8044  special

function registers and their assigned bit names. These assemblers

can produce formatted listing files and  object code in both  the

Intel Hex and the Motorola Hex formats.

The supported assembler directives are the same as the ones  used

by the microprocessor manufacturers, but you can customised  them

to your liking using the REDEF directive.

1.3  Specifications

The Macro Cross Assemblers supports:

up to 16 nested include files

up to 16 nested conditional assembly sections

up to 16 nested macros

unlimited number of include files

unlimited number of macros (only limited by memory)

Macro Cross Assembler - Series I                                2

1.4  Notational conventions

The following rules are used in this manual:

1. Words in capital letters are keywords and must be entered as

shown.  They  may  be  entered  in  uppercase  or  lowercase

characters.

2. Items in square brackets ([]) are optional.

3. Symbolic fields are enclosed in < >.

4. An ellipsis (...) indicates that an item may be repeated  as

many time as you wish.

5. All punctuation, except  the one mentioned  before, must  be

included where shown.

6. xxx is  a two  or three  character string  representing  the

microprocessor identification.

This string is:

44   : for the Intel 8044 microprocessor

48   : for the Intel 8048 microprocessor

51   : for the Intel 8051/52 microprocessor

80   : for the Intel 8080 microprocessor

85   : for the Intel 8085 microprocessor

96   : for the Intel 8096 microprocessor

Z80  : for the Zilog Z80 microprocessor

1.5  Learning more about the microprocessors

You may want  to get more  familiar with  the microprocessor  you

plan to  be  using  by reading  assembly  language  books,  other

handbooks and data books.

2. GETTING STARTED

2.1  Installation

Make a working  copy of  your disk,  using the  COPY or  DISKCOPY

utility supplied with MS-DOS. Save the original disk for backup.

No particular installation  is required except  that you have  to

copy the executable files to the disk you are going to use.

2.2  System requirements

IBM PC/XT/AT or compatible computer

DOS 2.0 or later

256K of RAM required  (more for bigger  symbol table or  many

macros)

2.3  Disk contents

Each assembler package (one for each microprocessor) will contain

the following files:

MAxxx.EXE     The  executable  file  of  the  corresponding

Macro Cross Assembler

EXAMPLEn.xxx  An example of a source file

README.xxx    An  optional   file  containing   the  latest

information

2.4  Setting up the environment

Each assembler environment  can be customised  by the  use of  an

environment variable.  The variable,  if  set, will  specify  the

default command  line  options.  The options  specified  in  that

variable will  not  replace but  will  be added  to  the  options

invoked in the command  line. Options are  specified in the  same

manner as if they were invoked  in the command line. The  setting

of this variable can be automatically done if the set command  is

placed in the AUTOEXEC.BAT file.

The variable is called MAxxxOPT and  can be set by the  following

command:

SET MAxxxOPT=-l -m -r10h

That command would invoked the l,  m and r options each time  the

assembler is invoked.

3. ASSEMBLING

3.1  Running the assembler

The assembler can be invoked as follow:

MAxxx [<options>] <sourcefile> [<objfile> [<listfile>]]

Where <options> can be one or more of the following separated by

spaces:

-l      Produce a listing file

-o      Produce NO object file

-t      Produce NO symbol table in the listing

-m      Produce object in Motorola Hex

-q      Quiet mode

-s      Display software license

-p<n>   Set listing page length (in lines per page)

-w<n>   Set listing width (in characters per line)

-r<n>   Set object record size (in bytes)

-e      Expand macros in the listing

-f      Do not list false conditions

-x      Use special function register set for the 8052

-c      Produce complete object in listing for long constants

<sourcefile>  Source file name. Default extension is ".xxx".

<objfile>     Object file name. Default extension is ".OBJ".

<listfile>    Listing file name. Default extension is ".LST".

If the  assembler is  invoked with  an error  in the  command

line, a help screen is displayed on the console.

For  <objfile> and <listfile>, if no  name is specified,  the

source file name with the default extensions will be used and

the files will be created in the current directory.

3.2  Options in more details

c  Complete object code in listing for long constants. This  is

used to see  all object codes  generated by long  constants,

like strings  and series  of data.  A source  line may  then

generate more than one  line in the  listing because only  a

few object codes can be written on one line of listing.

e  Expand macros in the listing. This option lists the complete

macro text when that macro is invoked. Even when this option

is invoked, the expansion of macros in the listing file will

still be controlled by the EXPAND and NOEXPAND directives.

Macro Cross Assembler - Series I                                5

f  False  conditional   sections   not  listed.   This   option

suppresses the listing of conditional sections that evaluate

as false. Even when this option  is invoked, the listing  of

conditional section  that evaluate  as false  will still  be

controlled by the FLIST and FNOLIST directives.

l  Listing file. This  option enables the  output to a  listing

file. The  listing produced  has  page headers  and  footers

inserted at appropriate intervals.  Even when the option  is

invoked, the  output  to  the listing  file  will  still  be

controlled by the LIST and NOLIST directives.

m  Motorola Hex format. This option  causes the object file  to

be produced  in  the Motorola  Hex  format rather  than  the

default Intel Hex format.

o  Object file disabled. This  option suppresses the output  to

an object file.

p  Page length. This option sets the  number of lines per  page

in the listing file. When a listing file is produced,  since

page headers  and footers  are inserted,  a page  length  is

required. If this option, immediately followed by a  numeric

constant, is invoked, the  page length will  be set to  that

value. The value must  be between 20 and  120 or it will  be

ignored. Even when this option  is invoked, the page  length

may be set again anywhere in the file by the PLEN directive.

The default page length is 66 lines.

q  Quiet mode.  This  option  turns off  the  display  of  line

numbers  during   assembly.   Normally  a   running   count,

incremented by 50s, of the numbers of lines being  assembled

is displayed on the console.

r  Record size in the object file. This option sets the number

of bytes in each object file record. The number selected  is

the number  of counted  bytes in  the selected  format.  The

number must be between 8 and  255 or it will be ignored.  If

the m option is invoked, the  r option must appear after  it

in the command line to have  any effect. The default  record

size in Intel Hex is 24. The default record size in Motorola

Hex is 27.

s  Software license. This option displays an information screen

on the software license.

t  Table of symbols suppressed in the listing file. This option

suppresses the output  of the  symbol table  in the  listing

file. Of course if output to the listing file is disabled at

the end of the listing, the symbol table will not appear  at

the end of the listing file anyway.

w  Width of the listing  file. This option  sets the number  of

characters per line  in the  listing file.  If this  option,

Macro Cross Assembler - Series I                                6

immediately followed by a numeric constant, is invoked,  the

line length will  be set to  that value. The  value must  be

between 60 and  132 or it  will be ignored.  Even when  this

option is invoked, the line length may be set again anywhere

in the file by the LLEN  directive. The default line  length

is 80 characters.

x  Use the extended function register  set for the 8052.  (only

for the 8051 assembler)

Note: Each option must be preceded by a  dash (-) in the command

line, to distinguish it from a file name.

3.3  Error codes returned to DOS

Condition                                    Error code

Normal completion, without errors            0

Normal completion, with assembly errors      1

Abnormal completion, insufficient memory     2

Abnormal completion, file access error       3

3.4  Object code file formats

This assembler supports two object file formats:

1.  Intel Hex

2.  Motorola Hex (S1-S9)

4. ASSEMBLER LANGUAGE

4.1  Source line format

Lines in  the  source file  consist  of  up to  four  fields,  as

follows:

[Label] [Opcode] [Operand] [Comment]

Each line  of the  file may  be  up to  256 characters  long.  An

arbitrary number of  white space (spaces  or tabs) must  separate

each field.

Label

Labels are symbols and follow the  same rules. If a label  starts

in the first  column, it  must be  terminated with  at least  one

white space (space  or tab) or  the end of  the line  but it  may

follow the rules of  a label starting in  any other column. If  a

label starts in any  other column, it must  be terminated with  a

colon (:) followed with at least  one white space (space or  tab)

or the end of  the line. The  ending colon is  never part of  the

symbol.

A line may consist  of a label alone.  With a normal  instruction

mnemonic, the assembler assigns a value to the label equal to the

current location counter.

Opcode

The operand  field  contains  the  instruction  mnemonic  or  the

assembler directive that is to be performed. This field must  not

start in the first column.

Operand

The operand field may  or may not be  required, depending on  the

opcode or directive being assembled. If present, this field  will

contain a certain number of operands, separated by commas.  There

must not  be  any  white  space  (space  or  tab)  between  these

operands.

Comment

The optional comment field is used for annotation purposes.  This

field is  ignored  by  the assembler,  but  is  included  in  the

listing. No character is necessary  to designate a comment  which

Macro Cross Assembler - Series I                                8

does not begin in  the first column. A semicolon (;) is  used by

the assembler to designate the beginning of a comment and it  can

be used anywhere in the line, including in the first column for a

stand-alone comment.

4.2  Expressions in the operand field

Expressions are combinations of constants, symbols, operators and

parentheses that are evaluated as an integer value.

4.2.1 Symbols

Symbols are strings of characters  that contain letters (A-Z,  a-

z), numbers (0-9),  points(.) and underscores  (_) but they  must

begin with a letter. A symbol may be of any length, but only  the

first 8  characters are  significant and  will be  listed in  the

symbol table. Since  all lowercase  letter will  be converted  to

uppercase, symbols which are capitalised differently will be  the

same. A symbol can not be a reserved word.

examples:LOOP

loop_2

key.buffer

The reserved words are the ones representing:

the mnemonics and assembler directives.

the microprocessor's registers.

4.2.2 Constants

Numeric constants

Numeric constants must  always begin  with a  decimal digit.  The

radix is determine by a  letter immediately following the  number

according to the following table:

Format         Suffix

Binary         B

Octal          O or Q

Decimal        D or nothing

Hexadecimal    H

examples: 10010001b

7774Q

18d

255

6fh

0FEH

Macro Cross Assembler - Series I                                9

Character constants

A character constant consists  of one ASCII character  surrounded

by single quote marks (').

Note: To put a single quote mark  into a character constant, two

consecutive single  quotes may  be used  to  represent one

such character.

examples:'a'

'*'

''''

The location counter

The current  value of  the location  counter can  be used  in  an

expression. It is represented by the dollar sign ($).

4.2.3 Operators

The operators allowed in expressions are shown, in the  following

table, in descending order of precedence:

Order Operator  Type            Description

----- --------  --------------  -----------

1.    ()        expression      parentheses

2.    -         unary           unary negative

~                         one's complement

3.    &         binary          bitwise AND

!                         bitwise OR

^                         bitwise EXCLUSIVE OR

4.    *         multiplicative  multiplication

/                         division

%                         remainder

5.    +         additive        addition

-                         subtraction

6.    <<        shift           shift left

>>                        shift right

7.    =         relational      equal

<                         less than

>                         greater than

<=                        less than or equal

>=                        greater than or equal

4.2.4 How expressions are evaluated

In an expression parenthetical  expressions are evaluated  first,

the innermost parentheses before the outer ones. Next, operations

Macro Cross Assembler - Series I                               10

involving higher  precedence  operators  are  done  first.  Where

several operators  have the  same precedence  they are  evaluated

from left to right except for unary operators which are evaluated

from right to left.  All intermediate values  are truncated to  a

16-bit integer value. The result of the expression is also a  16-

bit integer value.

Macro Cross Assembler - Series I                               11

4.3  Assembler directives

Assembly Control

----------------

ORG       Set assembly origin

END       End of source

REDEF     Redefine an assembler directive

Symbol Definition

-----------------

EQU       Equate symbol value

SET       Set symbol value (temporary)

Memory Allocation

-----------------

DB        Define constant byte(s)

DW        Define constant word(s)

RSB       Reserve storage for byte(s)

RSW       Reserve storage for word(s)

Listing Control

---------------

TITLE     Set title of page(s)

LLEN      Set line length (characters per line)

PLEN      Set page length (lines per page)

PAGE      Advance to top of next page

LIST      List the assembly

NOLIST    Do not list the assembly

ACLIST    List all object codes for long constants

NOACLIST  Do not list all object codes for long constants

FLIST     List sections that evaluate as false

FNOLIST   Do not list sections that evaluate as false

EXPAND    Expand macros

NOEXPAND  Do not expand macros

Conditional Assembly

--------------------

IF        Assemble if not equal to zero

IFN       Assemble if equal to zero

IFC       Assemble if string1 equals string2

IFNC      Assemble if string1 not equal to string2

ELSE      Reverse condition

ENDIF     End of conditional assembly

FAIL      Print error message

Include files

-------------

INCLUDE   Include file

Macro Cross Assembler - Series I                               12

Macro Directives

----------------

MACRO     Define a macro

ENDM      End of macro definition

EXITM     Skip to the end of a macro definition

Macro Cross Assembler - Series I                               13

ACLIST

Purpose:  This directive  restores the  listing of  the  complete

object code  for long  constants. If  the object  codes

generated by the line  do not fit on  a single line  of

listing, they will be listed on the following lines.

Format:   ACLIST

Remarks:  NOACLIST is the complementary directive.

DB

Purpose:  This directive allocates  and initialises  one or  more

bytes.

Format:   [<label>] DB <expression>[,<expression>]...

Remarks:  <expression> must be an 8-bit integer value or a string

surrounded by single quote marks (').

DW

Purpose:  This directive allocates  and initialises  one or  more

words.

Format:   [<label>] DW <expression>[,<expression>]...

Remarks:  <expression> must be a 16-bit integer value.

ELSE

Purpose:  This directive allows the alternate section of code  to

be assembled when  the opposite condition  exists in  a

conditional section.

Format:   ELSE

END

Purpose:  This directive designates the end of the source file.

Format:   END

Remarks:  No line  in the  source file  will be  read after  this

directive.

Macro Cross Assembler - Series I                               14

ENDIF

Purpose:  This directive  designates  the end  of  a  conditional

section in the code.

Format:   ENDIF

Remarks:  Each conditional  assembly  section must  end  with  an

ENDIF directive.

ENDM

Purpose:  This  directive   designates  the   end  of   a   macro

definition.

Format:   ENDM

Remarks:  Each macro definition must end with an ENDM directive.

EQU

Purpose:  This directive  assigns the  value of  <expression> to

<label>.

Format:   <label> EQU <expression>

Remarks:  The <label> entry can not be redefined.

EXITM

Purpose:  This directive is used  to skip to the  end of a  macro

definition. When  this  directive is  encountered,  the

expansion  is  exited  immediately  and  any  remaining

statement in the macro is not generated.

Format:   EXITM

Remarks:  An ENDM directive  is still required  when an EXITM  is

encountered.

EXPAND

Purpose:  This directive lists  the complete macro  text for  all

expansions.

Format:   EXPAND

Remarks:  NOEXPAND is the complementary directive.

Macro Cross Assembler - Series I                               15

FAIL

Purpose:  This  directive  sends  the  <message>  string  to  the

console and writes it in the listing file.

Format:   FAIL <message>

FLIST

Purpose:  This directive  restores  the  listing  of  conditional

section that evaluate as false.

Format:   FLIST

Remarks:  FLIST  is  the  default   condition.  FNOLIST  is   the

complementary directive.

FNOLIST

Purpose:  This directive  suppresses the  listing of  conditional

section that evaluate as false.

Format:   FNOLIST

Remarks:  FLIST is the complementary directive.

IF

Purpose:  This directive designates  the start  of a  conditional

section. The conditional section will only be assembled

if <expression> is not equal to zero.

Format:   IF <expression>

IFC

Purpose:  This directive designates  the start  of a  conditional

section. The conditional section will only be assembled

if <string1> is identical to <string2>.

Format:   IFC <string1>,<string2>

Macro Cross Assembler - Series I                               16

IFN

Purpose:  This directive designates  the start  of a  conditional

section. The conditional section will only be assembled

if <expression> is equal to zero.

Format:   IFN <expression>

IFNC

Purpose:  This directive designates  the start  of a  conditional

section. The conditional section will only be assembled

if <string1> is not identical to <string2>.

Format:   IFNC <string1>,<string2>

INCLUDE

Purpose:  This directive assembles the source statements from the

<pathname> file into the current source file.

Format:   INCLUDE <pathname>

Remarks:  The  <pathname>  entry   can   be   any   valid   file

specification  as  determined  by  the  Disk  Operating

System. The  file  is  opened and  assembled  into  the

current source file  immediately following the  INCLUDE

statement. When the end-of-file  is reached, or an  END

directive is  encountered,  assembly resumes  with  the

statement following the INCLUDE. There may be up to  16

nested  INCLUDE  directives.  The  maximum  length   of

<pathname> is 60 characters.

LIST

Purpose:  This directive restores the output in the listing.

Format:   LIST

Remarks:  If a listing  is not  being made,  this directive  will

start one. NOLIST is the complementary directive.

Macro Cross Assembler - Series I                               17

LLEN

Purpose:  This directive sets the number of characters per  line,

in the listing, to the value of <expression>.

Format:   LLEN <expression>

Remarks:  The number of  characters per line  must be between  60

and 132. The default is 80 characters per line.

MACRO

Purpose:  This directive defines a macro called <label>. A macro

is a sequence  of statements that  can be generated  in

various places in the program.

Format:   <label> MACRO

NOACLIST

Purpose:  This directive suppresses the  listing of the  complete

object code  for long  constants. If  the object  codes

generated by the line  do not fit on  a single line  of

listing, they will not all be listed.

Format:   NOACLIST

Remarks:  NOACLIST  is  the  default  condition.  ACLIST  is  the

complementary directive.

NOEXPAND

Purpose:  This directive suppresses listing of the complete macro

text for all expansions.

Format:   NOEXPAND

Remarks:  NOEXPAND  is  the  default  condition.  EXPAND  is  the

complementary directive.

NOLIST

Purpose:  This directive suppresses the output in the listing.

Format:   NOLIST

Remarks:  NOLIST  is   the  default   condition.  LIST   is   the

complementary directive.

Macro Cross Assembler - Series I                               18

ORG

Purpose:  This directive sets the  location counter to the value

of <expression>.

Format:   ORG <expression>

Remarks:  <expression> must not involved any forward references.

PAGE

Purpose:  This directive starts a new page in the listing.

Format:   PAGE

PLEN

Purpose:  This directive sets  the number of  lines per page,  in

the listing, to the value of <expression>.

Format:   PLEN <expression>

Remarks:  The number of  lines per page  must be  between 20  and

120. The default is 66 characters per line. The first 3

lines and the last 3 lines of the page are left  blank.

The fourth and fifth line are  used for the date,  page

number and page title.

REDEF

Purpose:  This  directive  redefines  the  <directive>  assembler

directive to the <directive2> directive.

Format:   REDEF <directive>,<directive2>

Remarks:  This allows you to  change the name  of each and  every

directive to satisfy your particular source file needs.

You can also use it to  make your own set of  universal

directives.

RSB

Purpose:  This directive reserves a block of <expression> byte(s)

for storage.

Format:   [<label>] RSB <expression>

Remarks:  The number  of bytes  reserved must  be between  1  and

32767.

Macro Cross Assembler - Series I                               19

RSW

Purpose:  This directive reserves a block of <expression> word(s)

for storage.

Format:   [<label>] RSW <expression>

Remarks:  The number  of words  reserved must  be between  1  and

16383.

SET

Purpose:  This  directive  temporarily   assigns  the  value   of

<expression> to <label>.

Format:   <label> SET <expression>

Remarks:  The <label> entry can be redefined.

TITLE

Purpose:  This directive  sets  the  title  of  the  current  and

following page(s) to the <text> string.

Format:   TITLE <text>

Remarks:  <text> is truncated to the first 60 characters.

4.4  Conditional assembly

All conditional section of code uses the following format:

IFxx argument

.

.

.

[ELSE]

.

.

.

ENDIF

Note: There may  be up  to 16  nested conditional  sections. Any

argument to a conditional must be known in pass 1.

Macro Cross Assembler - Series I                               20

4.5  Macro assembly

A macro is a collection of  assembler statements that may  appear

several times in a program with some optional modifications  each

time it is used.

To properly use a macro you must:

1. Define  the  macro  using  the  MACRO  and  ENDM   assembler

directives.

2. Invoke the macro by using the  name of the macro just as  if

it were a microprocessor mnemonic. (in the opcode field)

The invocation must be done  after the definition. When  invoked,

the statements in the macro are inserted, starting on the current

line, and assembled.

A macro may contain up to 9 parameters. In the macro  definition,

parameters are called dummy parameters and are represented by two

characters. The  dummy parameter  is formed  of a  backslash  (\)

followed by the parameter identification number (1-9).

In the macro invocation, actual  parameters are specified in  the

operand  field  of  the  invoking  line.  These  parameters   are

separated by commas and  will replaced their corresponding  dummy

parameters when the macro is invoked.

Example:

macro definition

message   MACRO

LDA       #\2       ;dummy parameter no 2

JSR       SET_Y

LDA       #\1       ;dummy parameter no 1

JSR       SET_X

LDX       #\3       ;dummy parameter no 3

JSR       TEXT

ENDM

macro invocation

message   12,5,mssg1     ;print mssg1 at 12,5

In this  example, the  first actual  parameter  is 12  and  would

replace the first dummy parameter \1. The second actual parameter

is 5 and would replace the  second dummy parameter \2. The  third

actual parameter is mssg1 and  would replace the dummy  parameter

\3. The actual parameters are all  strings that will replace  the

dummy parameters.

Macro Cross Assembler - Series I                               21

The code assembled would become:

LDA       #5

JSR       SET_Y

LDA       #12

JSR       SET_X

LDX       #mssg1

JSR       TEXT

Since labels  may  be  required in  a  macro  and  since  macros,

generally, will be invoked more than once, there must be a way to

generate distinct labels each time the macro is invoked. This  is

required to eliminated  multiply defined  symbols. These  symbols

require a special syntax to be  recognise by the assembler.  This

syntax is only recognised in macro definitions.

These symbols must end by an underscore (_) followed by a  dollar

sign ($). Only the  first 4 characters of  these symbols will  be

used and a three character number (in hexadecimal) preceded by an

underscore will follow these 4 characters to make the symbol.

Example:

macro definition

count     MACRO

DECA

wait_$:   BNE       wait_$

DECB

BNE       wait_$

ENDM

code generated the first time the macro is invoked:

DECA

wait_001: BNE       wait_001

DECB

BNE       wait_001

code generated the second time the macro is invoked:

DECA

wait_002: BNE       wait_002

DECB

BNE       wait_002

5. ERROR MESSAGES

5.1  Common errors

These errors are reported with the name of the source file  where

they are detected, and the line number.

Bit number out of range

This occurs  when the  bit number  specified  is too  big  or

negative.

Branch out of range

This occurs when the  relative branch required is  not within

the range -128  +127 of the current instruction.

Branch out of the 2Kbytes page

This occurs when  the branch  required is out  of the  2Kbyte

page.

Data is not an 8-bit value

This occurs when the data is not an 8-bit integer value.

Different size registers

This occurs when two or  more of the specified  registers are

of different sizes when they have to be of the same size.

Directive is reserved word

This occurs on a try to redefine a directive with  a reserved

word.

Division by zero attempted

This occurs  when  a division  by  zero  is attempted  in  an

expression.

Macro Cross Assembler - Series I                               24

Embedded macro not allowed

This occurs  on  an attempt  to  define  a macro  within  the

definition of another macro.

End of conditional assembly section missing

This occurs when a conditional assembly section does  not end

with an ENDIF directive.

End of macro missing

This occurs when a macro definition does not end with an ENDM

directive.

File name too long

This occurs when the specified file path name is more than 60

characters long.

Forward reference is illegal

This occurs when the expression could not be evaluated in the

first pass of the assembly.

Illegal addressing mode

This occurs  when  the addressing  mode  does not  match  any

admissible addressing mode for the current opcode.

Illegal bit addressable location

This occurs  when the  bit addressed  location is  not a  bit

addressable special function  register or a  byte of the  bit

addressable segment of the 8051 microprocessor.

Illegal bit location

This occurs  when  the  accessed location  is  not  a  direct

addressed bit in the internal data RAM or a  special function

register bit.

Macro Cross Assembler - Series I                               25

Illegal block length

This occurs when the  specified length for a  reserved memory

block is out of the specified range.

Illegal character in expression

This occurs when  an illegal character  is encountered in  an

expression constant.

Illegal direct location

This occurs when  the accessed  location is not  part of  the

internal data RAM of the 8051 microprocessor.

Illegal forward macro call

This occurs on an attempt to  invoke a macro that  is forward

defined. All macros must be backward defined.

Illegal interrupt mode

This occurs when an illegal interrupt mode is specified.

Illegal label

This occurs when  an illegal  character is  encountered in  a

label.

Illegal listing line length

This occurs when the specified listing line length is  out of

the allowed range.

Illegal listing page length

This occurs when the specified listing page length is  out of

the allowed range.

Illegal opcode

This occurs when the opcode in  the statement is not  a valid

mnemonic or a valid assembler directive.

Macro Cross Assembler - Series I                               26

Illegal port

This occurs when an illegal port name or number is specified.

Illegal register for indirect access

This occurs when the specified register number is not 0  or 1

in an indirect access.

Illegal shift count

This occurs when the shift count is not between 0 and 15.

Illegal starting address

This occurs when an illegal starting address is specified. It

is either too big or negative.

Illegal word register

This occurs  when  the  specified  register  is  not  a  word

register.

Immediate data is not a 3-bit value

This occurs when the specified immediate data is not  a 3-bit

value. It is probably too big.

Immediate data is not an 8-bit value

This occurs when an  immediate data is  not an 8-bit  integer

value.

Label is reserved word

This occurs when the specified label is a reserved  word. The

mnemonics,  special  function  registers,   special  function

register bits  and registers  of the  microprocessor are  all

reserved words.

Label required with this directive

This occurs when  the directive in  the statement requires  a

label and none is present.

Macro Cross Assembler - Series I                               27

Line too long

This occurs if the line is more than 256 characters long.

Macro already defined

This occurs on an attempt to redefine a macro.

Macro name is reserved word

This occurs when the specified macro name is a reserved word.

The mnemonics, special  function registers, special  function

register bits  and registers  of the  microprocessor are  all

reserved words.

Missing argument in invoked macro

This occurs when  an argument  required in the  macro is  not

present in the invoking line.

Missing file or bad file

This occurs on  an attempt to  include a  file that does  not

exist in the specified directory.

Missing operand

This occurs when the  opcode or assembler directive  requires

an operand and none is present.

Not a byte register

This occurs  when  the  specified  register  is  not  a  byte

register.

Not a long word register

This occurs when the  specified register is  not a long  word

register.

Not a word register

This occurs  when  the  specified register  is  not  a  word

register.

Macro Cross Assembler - Series I                               28

Not aligned at a multiple of 4 boundary

This occurs when the specified address  is not aligned at  a

multiple of 4 boundary.

Not aligned at an even byte boundary

This occurs when the specified  address is not aligned  at an

even byte boundary.

Not an even byte address for a word

This occurs when  the specified  address do not  point to  an

even byte.

Not in conditional section

This occurs  when  an ELSE  or  ENDIF is  encountered  without  a

previous conditional assembly directive active.

Not in macro definition

This occurs when an  EXITM or ENDM  is encountered without  a

previous macro definition active.

Offset is not an 8-bit value

This occurs when the specified offset is not an 8-bit value.

Phase error

This occurs when a label value in the second pass of assembly

differs from its value in the first pass.

Same register specified twice

This occurs when the same register is specified more than one

in a list or registers.

Symbol multiply defined

This occurs on an attempt to  redefine a symbol that  can not

be redefined.

Macro Cross Assembler - Series I                               29

Syntax error

This occurs when the syntax  of the statement does  not match

any recognisable syntax.

Too many nested conditional assembly sections

This occurs  on  an  attempt  to  insert  a  new  conditional

assembly section  within  one  while the  maximum  number  of

nested conditional assembly sections has been reached.

Too many nested files

This occurs on an attempt to include a file within  one while

the maximum number of nested include has been reached.

Too many nested macros

This occurs on an attempt to invoked a macro within one while

the maximum number of nested macros has been reached.

U register and S register together

This occurs  when  the  two  stack pointers  (U  and  S)  are

specified in the push or pull operation.

Undefined symbol

This occurs when a symbol encountered is not defined.

Unknown directive

This occurs when a specified directive is unknown. An unknown

directive can not be redefined.

5.2  Warnings

These warnings are  generated with the  name of  the source  file

where they are detected, and the line number.

Ascii constant is not an 8-bit value

This occurs when an ascii constant is more than one character

long.

Macro Cross Assembler - Series I                               30

End directive missing

This occurs when a source file ends without an END directive.

Label ignored

This occurs when the definition of a label is  attempted with

a directive which does not require (and allow) one.

Numeric constant is not an 16-bit value

This occurs when a numeric  constant is not a  16-bit integer

value.

5.3  Fatal errors

When one of these errors occur, the assembler sends a message  to

the console and aborts, returning and error code to DOS.

Error closing input file:  "error message"

Error closing listing file:  "error message"

Error closing object file:  "error message"

Error opening input file:  "error message"

Error opening listing file:  "error message"

Error opening object file:  "error message"

Error reading input file:  "error message"

Error writing to listing file:  "error message"

Error writing to object file:  "error message"

Out of memory for the symbol table

Out of memory for the macro table

6. SPECIAL SECTION ON THE MICROS

6.1  Intel 8044

The reserved words in this assembler are:

A, AB, C, DPTR, R0, R1, R2, R3, R4, R5, R6, R7

Special Function Registers:

The 8044  has Special  Function  Registers. These  constants  are

words representing the 8044 registers accessible through the  use

of their addresses in the on-chip RAM.

These are predefined in this assembler:

ACC, B, PSW, SP, DPL,  DPH, P0, P1, P2,  P3, IP, IE, TMOD,  TCON,

TH0, TL0, TH1, TL1,  FIFO, TBS, TBL, TCB,  NSNR, STAD, RFL,  RBS,

RBL, RCB, SMD, STS, SMD

Special Function Register Bits:

These Special  Function Registers  have bits  which are  directly

accessible. These  constants  are  words  representing  the  8044

microprocessor register bits accessible through the use of  their

addresses in the on-chip RAM.

These are predefined in the assembler:

SFR bit  Description

P        PSW bit 0

OV       PSW bit 2

RS0      PSW bit 3

RS1      PSW bit 4

F0       PSW bit 5

AC       PSW bit 6

CY       PSW bit 7

EX0      IE bit 0

ET0      IE bit 1

EX1      IE bit 2

ET1      IE bit 3

ES       IE bit 4

ET2      IE bit 5

EA       IE bit 7

PX0      IP bit 0

PT0      IP bit 1

PX1      IP bit 2

PT1      IP bit 3

PS       IP bit 4

PT2      IP bit 5

IT0      TCON bit 0

Macro Cross Assembler - Series I                               32

IE0      TCON bit 1

IT1      TCON bit 2

IE1      TCON bit 3

TR0      TCON bit 4

TF0      TCON bit 5

TR1      TCON bit 6

TF1      TCON bit 7

ACC.0    Accumulator bit 0

ACC.1    Accumulator bit 1

ACC.2    Accumulator bit 2

ACC.3    Accumulator bit 3

ACC.4    Accumulator bit 4

ACC.5    Accumulator bit 5

ACC.6    Accumulator bit 6

ACC.7    Accumulator bit 7

B.0      Register B bit 0

B.1      Register B bit 1

B.2      Register B bit 2

B.3      Register B bit 3

B.4      Register B bit 4

B.5      Register B bit 5

B.6      Register B bit 6

B.7      Register B bit 7

PSW.0    PSW bit 0

PSW.1    PSW bit 1

PSW.2    PSW bit 2

PSW.3    PSW bit 3

PSW.4    PSW bit 4

PSW.5    PSW bit 5

PSW.6    PSW bit 6

PSW.7    PSW bit 7

P0.0     Port 0 bit 0

P0.1     Port 0 bit 1

P0.2     Port 0 bit 2

P0.3     Port 0 bit 3

P0.4     Port 0 bit 4

P0.5     Port 0 bit 5

P0.6     Port 0 bit 6

P0.7     Port 0 bit 7

P1.0     Port 1 bit 0

P1.1     Port 1 bit 1

P1.2     Port 1 bit 2

P1.3     Port 1 bit 3

P1.4     Port 1 bit 4

P1.5     Port 1 bit 5

P1.6     Port 1 bit 6

P1.7     Port 1 bit 7

P2.0     Port 2 bit 0

P2.1     Port 2 bit 1

P2.2     Port 2 bit 2

P2.3     Port 2 bit 3

P2.4     Port 2 bit 4

P2.5     Port 2 bit 5

Macro Cross Assembler - Series I                               33

P2.6     Port 2 bit 6

P2.7     Port 2 bit 7

P3.0     Port 3 bit 0

P3.1     Port 3 bit 1

P3.2     Port 3 bit 2

P3.3     Port 3 bit 3

P3.4     Port 3 bit 4

P3.5     Port 3 bit 5

P3.6     Port 3 bit 6

P3.7     Port 3 bit 7

IP.0     IP bit 0

IP.1     IP bit 1

IP.2     IP bit 2

IP.3     IP bit 3

IP.4     IP bit 4

IP.5     IP bit 5

IP.6     IP bit 6

IP.7     IP bit 7

IE.0     IE bit 0

IE.1     IE bit 1

IE.2     IE bit 2

IE.3     IE bit 3

IE.4     IE bit 4

IE.5     IE bit 5

IE.6     IE bit 6

IE.7     IE bit 7

TCON.0   TCON bit 0

TCON.1   TCON bit 1

TCON.2   TCON bit 2

TCON.3   TCON bit 3

TCON.4   TCON bit 4

TCON.5   TCON bit 5

TCON.6   TCON bit 6

TCON.7   TCON bit 7

RBP      STS bit 0

AM       STS bit 1

CPB      STS bit 2

BV       STS bit 3

SI       STS bit 4

RTS      STS bit 5

RE       STS bit 6

TBF      STS bit 7

SER      NSNR bit 0

NR0      NSNR bit 1

NR1      NSNR bit 2

NR2      NSNR bit 3

SES      NSNR bit 4

NS0      NSNR bit 5

NS1      NSNR bit 6

NS2      NSNR bit 7

STS.0    STS bit 0

STS.1    STS bit 1

STS.2    STS bit 2

Macro Cross Assembler - Series I                               34

STS.3    STS bit 3

STS.4    STS bit 4

STS.5    STS bit 5

STS.6    STS bit 6

STS.7    STS bit 7

NSNR.0   NSNR bit 0

NSNR.1   NSNR bit 1

NSNR.2   NSNR bit 2

NSNR.3   NSNR bit 3

NSNR.4   NSNR bit 4

NSNR.5   NSNR bit 5

NSNR.6   NSNR bit 6

NSNR.7   NSNR bit 7

Note: Consult  a   data   book   on   the  8044   microprocessor

architecture for  more  details  on  the special  function

registers.

Assembler directives:

For this microprocessor, the  standard assembler directives  have

been predefined as follows:

DATA     instead of EQU

ORG

END

DB

DW

SET

DS       instead of RSB

DSW      instead of RSW

6.2  Intel 8048

The reserved words in this assembler are:

A, R0, R1, R2, R3, R4, R5, R6, R7, @R0, @R1, BUS, P1, P2, P4, P5,

P6, P7, C, F0, F1, I, TCNTI, CLK, @A, PSW, T, MB0, MB1, RB0, RB1,

TCNT, CNT

Assembler directives:

For this microprocessor, the  standard assembler directives  have

been predefined as follows:

DATA     instead of EQU

ORG

END

DB

DW

SET

Macro Cross Assembler - Series I                               35

DS       instead of RSB

RSW      is not supported, because it is not necessary

6.3  Intel 8051/52

The reserved words in this assembler are:

A, AB, C, DPTR, R0, R1, R2, R3, R4, R5, R6, R7

Special Function Registers:

The 8051  has Special  Function  Registers. These  constants  are

words representing the  8051 microprocessor registers  accessible

through the use of their addresses in the on-chip RAM.

These are predefined in this assembler:

ACC, B, PSW, SP, DPL,  DPH, P0, P1, P2,  P3, IP, IE, TMOD,  TCON,

TH0, TL0, TH1, TL1, SCON, SBUF, PCON

If the extended special function  register set option is  invoked

(x option), the 8052  microprocessor additional special  function

registers are also predefined in the assembler.

These are:

T2CON, TH2, TL2, RCAP2H, RCAP2L

Special Function Register Bits:

These Special  Function Registers  have bits  which are  directly

accessible. These  constants  are  words  representing  the  8051

microprocessor register bits accessible through the use of  their

addresses in the on-chip RAM.

These are predefined in the assembler:

SFR bit  Description

P        PSW bit 0

OV       PSW bit 2

RS0      PSW bit 3

RS1      PSW bit 4

F0       PSW bit 5

AC       PSW bit 6

CY       PSW bit 7

EX0      IE bit 0

ET0      IE bit 1

EX1      IE bit 2

ET1      IE bit 3

ES       IE bit 4

ET2      IE bit 5

Macro Cross Assembler - Series I                               36

EA       IE bit 7

PX0      IP bit 0

PT0      IP bit 1

PX1      IP bit 2

PT1      IP bit 3

PS       IP bit 4

PT2      IP bit 5

IT0      TCON bit 0

IE0      TCON bit 1

IT1      TCON bit 2

IE1      TCON bit 3

TR0      TCON bit 4

TF0      TCON bit 5

TR1      TCON bit 6

TF1      TCON bit 7

RI       SCON bit 0

TI       SCON bit 1

RB8      SCON bit 2

TB8      SCON bit 3

REN      SCON bit 4

SM2      SCON bit 5

SM1      SCON bit 6

SM0      SCON bit 7

RXD      Port 3 bit 0

TXD      Port 3 bit 1

INT0     Port 3 bit 2

INT1     Port 3 bit 3

T0       Port 3 bit 4

T1       Port 3 bit 5

WR       Port 3 bit 6

RD       Port 3 bit 7

ACC.0    Accumulator bit 0

ACC.1    Accumulator bit 1

ACC.2    Accumulator bit 2

ACC.3    Accumulator bit 3

ACC.4    Accumulator bit 4

ACC.5    Accumulator bit 5

ACC.6    Accumulator bit 6

ACC.7    Accumulator bit 7

B.0      Register B bit 0

B.1      Register B bit 1

B.2      Register B bit 2

B.3      Register B bit 3

B.4      Register B bit 4

B.5      Register B bit 5

B.6      Register B bit 6

B.7      Register B bit 7

PSW.0    PSW bit 0

PSW.1    PSW bit 1

PSW.2    PSW bit 2

PSW.3    PSW bit 3

PSW.4    PSW bit 4

PSW.5    PSW bit 5

Macro Cross Assembler - Series I                               37

PSW.6    PSW bit 6

PSW.7    PSW bit 7

P0.0     Port 0 bit 0

P0.1     Port 0 bit 1

P0.2     Port 0 bit 2

P0.3     Port 0 bit 3

P0.4     Port 0 bit 4

P0.5     Port 0 bit 5

P0.6     Port 0 bit 6

P0.7     Port 0 bit 7

P1.0     Port 1 bit 0

P1.1     Port 1 bit 1

P1.2     Port 1 bit 2

P1.3     Port 1 bit 3

P1.4     Port 1 bit 4

P1.5     Port 1 bit 5

P1.6     Port 1 bit 6

P1.7     Port 1 bit 7

P2.0     Port 2 bit 0

P2.1     Port 2 bit 1

P2.2     Port 2 bit 2

P2.3     Port 2 bit 3

P2.4     Port 2 bit 4

P2.5     Port 2 bit 5

P2.6     Port 2 bit 6

P2.7     Port 2 bit 7

P3.0     Port 3 bit 0

P3.1     Port 3 bit 1

P3.2     Port 3 bit 2

P3.3     Port 3 bit 3

P3.4     Port 3 bit 4

P3.5     Port 3 bit 5

P3.6     Port 3 bit 6

P3.7     Port 3 bit 7

IP.0     IP bit 0

IP.1     IP bit 1

IP.2     IP bit 2

IP.3     IP bit 3

IP.4     IP bit 4

IP.5     IP bit 5

IP.6     IP bit 6

IP.7     IP bit 7

IE.0     IE bit 0

IE.1     IE bit 1

IE.2     IE bit 2

IE.3     IE bit 3

IE.4     IE bit 4

IE.5     IE bit 5

IE.6     IE bit 6

IE.7     IE bit 7

TCON.0   TCON bit 0

TCON.1   TCON bit 1

TCON.2   TCON bit 2

Macro Cross Assembler - Series I                               38

TCON.3   TCON bit 3

TCON.4   TCON bit 4

TCON.5   TCON bit 5

TCON.6   TCON bit 6

TCON.7   TCON bit 7

SCON.0   SCON bit 0

SCON.1   SCON bit 1

SCON.2   SCON bit 2

SCON.3   SCON bit 3

SCON.4   SCON bit 4

SCON.5   SCON bit 5

SCON.6   SCON bit 6

SCON.7   SCON bit 7

If the extended special function  register set option is  invoked

(x option), the 8052  microprocessor additional special  function

register bits are also predefined in the assembler.

These are:

SFR bit  Description

CPRL2    T2CON bit 0

CT2      T2CON bit 1

TR2      T2CON bit 2

EXEN2    T2CON bit 3

TCLK     T2CON bit 4

RCLK     T2CON bit 5

EXF2     T2CON bit 6

TF2      T2CON bit 7

T2CON.0  T2CON bit 0

T2CON.1  T2CON bit 1

T2CON.2  T2CON bit 2

T2CON.3  T2CON bit 3

T2CON.4  T2CON bit 4

T2CON.5  T2CON bit 5

T2CON.6  T2CON bit 6

T2CON.7  T2CON bit 7

Note: Consult a  data  book  on  the  8051, 8052  microprocessor

architectures for  more  details on  the  special function

registers.

Assembler directives:

For this microprocessor, the  standard assembler directives  have

been predefined as follows:

DATA     instead of EQU

ORG

END

DB

DW

Macro Cross Assembler - Series I                               39

SET

DS       instead of RSB

DSW      instead of RSW

6.4  Intel 8080

The reserved words in this assembler are:

A, B, C, D, E, H, L, M, BC, DE, HL, SP

Assembler directives:

For this microprocessor, the  standard assembler directives  have

been predefined as follows:

EQU

ORG

END

DB

DW

SET

DS       instead of RSB

RSW      is not supported, because it is not necessary

6.5  Intel 8085

The reserved words in this assembler are:

A, B, C, D, E, H, L, M, BC, DE, HL, SP

Assembler directives:

For this microprocessor, the  standard assembler directives  have

been predefined as follows:

EQU

ORG

END

DB

DW

SET

DS     instead of RSB

RSW    is not supported, because it is not necessary

6.6  Intel 8096

There are no reserved words in this assembler.

Macro Cross Assembler - Series I                               40

Assembler directives:

For this microprocessor, the  standard assembler directives  have

been predefined as follows:

EQU

ORG

END

DCB      instead of DB

DCW      instead of DW

SET

DSB      instead of RSB

DSW      instead of RSW

6.7  Zilog Z80

The reserved words in this assembler are:

A, B, C, D, E, H, L, BC, DE, HL, SP, IX, IY, M, NC, NZ, Z, P, PE,

PO, AF, I, R

Assembler directives:

For this microprocessor, the  standard assembler directives  have

been predefined as follows:

EQU

ORG

END

DEFB     instead of DB

DEFW     instead of DW

DEFL     instead of SET

DEFS     instead of RSB

RSW      is not supported, because it is not necessary

7. BUG REPORTING PROCEDURE

Although each software is extensively tested prior to release, no

software is  perfect  and may  contain  a  few bugs.  It  is  the

intention of  Micept  Instruments  Inc. to  correct  any  genuine

problem that  will be  considered as  a problem  by the  company,

although this  is  not  a  warranty  that  any  problem  will  be

corrected.

If you have any comments or suggestions, please send them to  the

following address or e-mail them to us.

If you think you have found  a bug in this software, please  take

the time to report it. Any report will be helpful, but you should

follow these directions:

1. Provide the name and version of the software.

2. Provide a brief  description of  the problem  and provide  a

copy or listing of  the problem source  file. Please try  to

isolate the problem area in the source and only provide that

part of the source file.

3. Send it to:  Micept Instruments Inc.

377 Julien St.

Cap De La Madeleine, PQ

Canada  G8T 6W6

e-mail: jveillet@aei.ca

8. ORDERING INFORMATION

To register please use the order form.

The registration fee will licence one  copy of the software  that

you will receive, for use on any one computer at any one time.

Macro Cross Assembler - Series I                               43

Series I

Cat.  Description                                  License

No.                                                Fee

----------------------------------------------------------

MA44  Macro Cross Assembler for the Intel 8044     $20.00

MA48  Macro Cross Assembler for the Intel 8048     $20.00

MA51  Macro Cross Assembler for the Intel 8051/52  $20.00

MA80  Macro Cross Assembler for the Intel 8080     $20.00

MA85  Macro Cross Assembler for the Intel 8085     $20.00

MA96  Macro Cross Assembler for the Intel 8096     $20.00

MAZ80 Macro Cross Assembler for the Zilog Z80      $20.00

SERIES-I Complete series containing the Macro      $99.00

Cross Assemblers for the Intel 8044,

8048, 8051/52, 8080, 8085, 8096 and

the Zilog Z80 (manual on disk)

Series II

Cat.  Description                                  License

No.                                                Fee

----------------------------------------------------------

MA03  Macro Cross Assembler for the Motorola 6803  $20.00

MA05  Macro Cross Assembler for the Motorola 6805  $20.00

MA09  Macro Cross Assembler for the Motorola 6809  $20.00

MA65  Macro Cross Assembler for the Rockwell 6502  $20.00

MA65C Macro Cross Assembler for the Rockwell 65C02 $20.00

MA68  Macro Cross Assembler for the Motorola 6800  $20.00

SERIES-IIComplete series containing the Macro      $79.00

Cross Assemblers for the Motorola 6800,

6803, 6805, 6809 and the Rockwell 6502

and 65C02 (manual on disk)

Complete Package

Cat.    Description                                License

No.                                                Fee

----------------------------------------------------------

MICRO-8 Package containing all the Macro Cross     $139.00

Assemblers of the series I and II

(Both manuals on disk)

Macro Cross Assembler - Series I                               44

ORDER FORM

Date: ___________________                   #96-MC224-000

Qty  Cat. No. Description           Unit Price      Price

---  -------- --------------------- ----------  ---------

___  ________ _____________________ $____.___   $____.___

___  ________ _____________________ $____.___   $____.___

Subtotal  $_____.___

Add $6.00 Shipping + handling  $_____.___

(Outside North America Add $10.00)

Subtotal  $_____.___

Canada residents Add GST  $_____.___

Subtotal  $_____.___

Quebec residents Add Provincial Sales Tax  $_____.___

Total U.S. dollars  $_____.___

Canadian Dollar Exchange Rate______

Payments in Canadian Dollars $_____.__

Please Specify Disk Format: ___ 5 1/4''                                                 or ___ 3 1/2''

Send your check or money order to:

Micept Instruments Inc.

377 Julien St.

Cap De La Madeleine, PQ

Canada  G8T 6W6

Name:    _______________________________________

Company: _______________________________________

Address: _______________________________________

_______________________________________

_______________________________________

* Prices are in U.S. dollars. We accept only U.S. and

Canadian funds.  All other will be refused. Prices

are subject to change without notice.

INDEX

INCLUDE                      16

8044                        31    Installation                  3

8048                        34    Intel 8044                   31

8051/52                     35    Intel 8048                   34

8080                        39    Intel 8051/52                35

8085                        39    Intel 8080                   39

8096                        39    Intel 8085                   39

Intel 8096                   39

ACLIST                      13    Introduction                  1

Assembler directives        11

Assembler language           7    Label                         7

Assembling                   4    LIST                         16

LLEN                         17

Bug reporting procedure     41    Location counter              9

Character constants          9    MACRO                        17

Comment                      7    Macro assembly               20

Common errors               23    Manual                        1

Conditional assembly        19    Microprocessors           2; 31

Constants                    8

Conventions                  2    NOACLIST                     17

NOEXPAND                     17

DB                          13    NOLIST                       17

Disclaimer                 iii    Notice                      iii

Disk contents                3    Numeric constants             8

DW                          13

Opcode                        7

ELSE                        13    Operand                       7

END                         13    Operators                     9

ENDIF                       14    Options                       4

ENDM                        14    Ordering information         42

EQU                         14    ORG                          18

Error codes                  6    Overview                      1

Error messages              23

EXITM                       14    PAGE                         18

EXPAND                      14    PLEN                         18

Expressions               8; 9

REDEF                        18

FAIL                        15    RSB                          18

Fatal errors                30    RSW                          19

File formats                 6    Running the assembler         4

FLIST                       15

FNOLIST                     15    SET                          19

Setting up the environment    3

Getting started              3    Source line format            7

Specifications                1

IF                          15    Symbols                       8

IFC                         15    System requirements           3

IFN                         16

IFNC                        16    Table of contents            iv

Macro Cross Assembler - Series I                               46

TITLE                       19

Trademarks                 iii

Warnings                    29

Z80                         40

Zilog Z80                   40

Notes

Notes