��������     ���� ��������

���  ���   ����   ���  ���

�������  ���� ��� ��������

������� �������� ��������

���  ��� �������� ���  ���

��������      ��� ��������

A Full Screen Simulator for 8048 Compatible Microprocessors

Version 1.3

William Luitje

23 July 1996

�����������������������������������������������������������������Ŀ

���� Background ���������������������������������������������������

�������������������������������������������������������������������

I  wrote this simulator as part of my work on a consulting job  a

few years ago.   There is still quite a lot of work one could  do

to improve it, such as providing a way to manually change RAM and

ports,   allowing  disassembly of any part  of  ROM,   simulating

changes in inputs,  command files...  A LOT!  Still, I think it's

useful,   as  is,   for learning the 8048   instruction  set  and

debugging small programs.   I hope it's useful to you; if you use

it,   please  see  the registration section at the  end  of  this

document (relax, no money is involved).

With  the  work I mentioned above and some features  specifically

for  an educational environment,  this program would make a great

tool   for  teaching  microprocessor  programming.     Commercial

programs like this cost from US$200 to $300.  If I make an educa-

tional version of S48,  I  envision a site licensing  arrangement

which would cost a very modest amount per student.  If you are an

instructor or administrator and are interested in pursuing  this,

contact  me at the address given at the end of this document even

if you use a different micro in your course.

�����������������������������������������������������������������Ŀ

���� Introduction �������������������������������������������������

�������������������������������������������������������������������

S48   simulates the instruction set and some of the hardware  for

the 8048,  8049,  8050, 8021, 8022  microprocessors originated by

Intel and now widely second-sourced.  The simulator handles these

hardware parameters:

� 4096 bytes of ROM

� 128 bytes of internal RAM

� 256 bytes of external RAM

� 7 I/O ports

� one 8 bit timer

The  hardware  registers  and  flags  and  RAM  are  continuously

displayed  on  screen  and  are updated as  each  instruction  is

executed.  There is a disassembly window showing the instructions

as they are executed.   You can trace through instructions one at

a time or execute them continuously until a breakpoint is reached

or you stop the program manually.

The  hardware requirements to run S48  are quite simple;   you'll

need  a  PC  with a monochrome,  Hercules,  CGA,   EGA,   or  VGA

compatible  video card (the program does direct screen writes  so

you'll  see snow using CGA).   A  color display is  not  strictly

necessary, but is certainly helpful in reading the screen easily.

Obviously,  the faster your CPU,  the faster the simulation  will

run.  A 4.77 MHz PC is adequate for small programs.  The software

requirements  are even simpler:  you'll need some version of DOS,

S48 and an 8048 program to simulate.

�����������������������������������������������������������������Ŀ

���� Sample Program �����������������������������������������������

�������������������������������������������������������������������

I've included a very simple sample program for you to play  with,

called demo.bin.  demo.asm is the source file and implements a 16

bit  counter in software.   There is an interrupt service routine

which  captures  the low byte of that counter and  stores  it  in

consecutive  locations  in  external RAM when  the  interrupt  is

invoked.  The timer interrupt service routine increments an 8 bit

counter  when the timer overflows.  It's short enough that I  can

reproduce  its  assembly listing here:

.org   0

0000 00                   nop

0001 04 09                jmp    start

0003                      .org   3

0003 00                   nop

0004 00                   nop

0005 04 1B                jmp    intsvc

0007                      .org   7

0007 04 21                jmp    tmrsvc

0009             ; Initialization

0009 27          start    clr    a

000A A8                   mov    r0,a

000B A9                   mov    r1,a

000C AB                   mov    r3,a

000D 62                   mov    t,a

000E B8 18                mov    r0,#24

0010 A0                   mov    @r0,a

0011 25                   en     tcnti

0012 55                   strt   t

0013 05                   en     i

0014             ; Main Loop

0014 18          loop     inc    r0

0015 F8                   mov    a,r0

0016 96 14                jnz    loop

0018 19                   inc    r1

0019 04 14                jmp    loop

001B             ; External Interrupt Service Routine

001B F8          intsvc   mov    a,r0

001C D5                   sel    rb1

001D 90                   movx   @r0,a

001E 18                   inc    r0

001F C5                   sel    rb0

0020 93                   retr

0021             ; Timer Overflow Interrupt Service Routine

0021 1B          tmrsvc   inc    r3

0022 93                   retr

0023                      .end

You'll also need an understanding of the 8048 instruction set and

architecture.  You won't find much help in this document.

�����������������������������������������������������������������Ŀ

���� Tutorial �����������������������������������������������������

�������������������������������������������������������������������

The 8048 program to be executed can be in either Intel hex format

or binary, like you would use to program an EPROM.  The simulator

tries  to  read  the file in hex format first and if  that  fails

assumes that the file is binary.   To invoke the simulator,  just

give the name of the program file as an argument, e.g.:

s48 demo.bin

Take  a  minute to familiarize yourself with the  layout  of  the

screen:

����������������������������������������������������������������������������Ŀ

�Cmd:                                                                        �

� 35 bytes read from demo.bin                                                �

���PC� �AĿ�PSW�CY�AC�F0�F1�RB�STK�TIE�TS�TF�TM�                             �

��000� �  ���08�� �� ��0��0��0���0���0��0�00�  �                             �

� �MB� �Bus��P1�P2�P3�P4�P5�P6�P7� �T0�T1�II�IE�                             �

� ��0� ��  ��  �  �  �  �  �  �  � �� �� �� �� �                             �

�    ��00��10��20��30��40��50��60��70��ͻ          �000�00����NOP�������     �

� 00 � ��                               � 08        001 04 06 JMP  006       �

� 01 � ��                               � 09        003 00    NOP            �

� 02 � ��                               � 0A        004 00    NOP            �

� 03 � ��                               � 0B        006 04 1B JMP  01B       �

� 04 � ��                               � 0C        007 04 21 JMP  021       �

� 05 � ��                               � 0D        �����Breakpoints���Ŀ    �

� 06 � ��                               � 0E        ��� ��� ��� ��� ����    �

� 07 � ��                               �������������������������������ͻ    �

�    ��00��10��20��30��40��50��60��70��80��90��A0��B0��C0��D0��E0��F0��͹    �

� 00 �                                                                  �  08�

� 01 �                                                                  �  09�

� 02 �                                                                  �  0A�

� 03 �                                                                  �  0B�

� 04 �                                                                  �  0C�

� 05 �                                                                  �  0D�

� 06 �                                                                  �  0E�

� 07 �                                                                  �  0F�

�    ����0F��1F��2F��3F��4F��5F��6F��7F��8F��9F��AF��BF��CF��DF��EF��FFͼ    �

������������������������������������������������������������������������������

The  top  line is where you type your commands to the  simulator.

The  second  line is where any messages from S48  are  displayed.

The hardware registers and flags are displayed with a light  blue

background  at  the  top  left  of  the  screen.    There  is  an

instruction disassembly window at the top right with the  current

instruction (the one to be executed next) shown in reverse video.

Below   it   are  the  5  instructions  following   the   current

instruction.    Above  it will be displayed the 6  most  recently

executed instructions.  Below the disassembly window is where the

breakpoints are displayed.

The large L shaped box shows all of RAM.   The top part shows 128

bytes of internal RAM and the lower shows 256  bytes of  external

RAM.   There isn't enough room on the screen to have blank  space

between  the columns so they are distinguished by the  background

color.   The first column contains locations 00  through 07;  the

second holds 08 through 0F, and so on.  The light blue background

in internal RAM shows the active register bank.

When  the  simulator is started the processor is in  reset.   The

values  of registers which are initialized by RESET,  e.g.    the

Program Counter,  are shown;  registers which are undefined after

RESET, such as the Accumulator and RAM are shown as blank.

Let's do some simulation.  Give a Trace command by pressing the t

key  and  return  (or  more  simply,   just  return).    The  NOP

instruction has now moved above the current instruction line, the

PC  has  changed to 001  and the current instruction is JMP  006.

Give another trace command and the current instruction changes to

CLR   A.    Notice  that  the  instructions  after  the   current

instruction  have now changed to reflect the jump.   Their  color

has  changed,   too,  to make it easier to see that the jump  was

taken.    (This  is especially useful for  following  conditional

jumps.)     Trace  again  and  notice  how  the  Accumulator   is

initialized to zero.

Now  let's  watch the simulation run for a while.   Give  the  Go

command  by  pressing the g key and return.   As  the  simulation

runs, notice how the timer register changes and memory location 0

counts up quickly and memory location 1 counts slowly.  Press any

key to stop the simulation.

To invoke the interrupt, press the i key and return.   Notice how

the  current instruction changes to the first instruction of  the

interrupt  service routine at location 003.   Trace  through  the

rest  of  the  interrupt  service  routine  and  notice  how  the

background  of the register banks changes as the active  register

bank is switched.

To  demonstrate setting a breakpoint,  type "b 18"   and  return.

This  will  set a breakpoint at the instruction which  increments

the high byte of the 16 bit counter. Now give a go command and in

a few seconds the simulation will stop at location 18.  To remove

the  breakpoint,   use the kill command with the address  of  the

breakpoint you want to remove.   In this case, "k 18"  and return

will do the trick.

To Quit the simulator, press q and return.

�����������������������������������������������������������������Ŀ

���� Command Summary ����������������������������������������������

�������������������������������������������������������������������

You've  seen all of the commands except for Help,  Reset and Port

in the tutorial; here we'll summarize them and give a little more

information about them.   Only the first letter of the command is

parsed  and  it can be either upper or lower case.   Some of  the

commands can take a numeric argument which is always expressed in

hexadecimal and need not be given in some cases.

g #   Go: Start simulating at address #.  Simulation stops when a

breakpoint  is reached or when a key is pressed.   If #  is

not given,  simulation starts at the current instruction.

t #   Trace:  Trace  #  instructions.   If # is not  given,   one

instruction is traced.

<ret> Trace one instruction; equivalent to "t 1".

b #   Breakpoint:  Set  a breakpoint at the given address.    The

command has no effect if a number is not given.

k #   Kill  breakpoint:   Remove  the  breakpoint  at  the  given

address.    The  command has no effect if a number  is  not

given or if there is no breakpoint at that address.

p # # Port value:  Set the port  specified by the first # to  the

value  specified by the second #.   Two parameters must  be

given.

r     Reset:  Simulate a  reset.   The Program Counter is set  to

zero and other registers are set as appropriate.  Registers

which are not affected by RESET are not changed.

i     Interrupt:  Simulate  an external interrupt.   The  Program

Counter  is  set to the location of the  interrupt  service

routine, 0003H, and further interrupts are disabled.

q     Quit:  Quit the simulation.  Come back again soon!

h     or  any  unrecognized  command  will  display  a small help

screen listing the available commands and their syntax.

�����������������������������������������������������������������Ŀ

���� Timer Simulation ���������������������������������������������

�������������������������������������������������������������������

The boxes which display the state of the timer are located in the

top  row of registers at the extreme right of the register  area.

They are explained as follows:

TIE - the Timer Interrupt Enable bit.  This is set by an EN TCNTI

instruction and cleared by DIS TCNTI.

TS - the Timer Started bit. This is set by the STRT T and cleared

by the STOP TCNT instructions.

TF - contains two bits related to timer overflow.   The right one

is  the  Timer  Flag  which is tested  and  cleared  by  the  JTF

instruction.   The left one is the Timer Overflow Flipflop  which

is  set  when the timer overflows and the Timer Interrupt  Enable

bit is set.  It is cleared when the timer interrupt is invoked so

you  won't see this bit set unless the external interrupt  is  in

progress when the timer overflows.

TM - the  timer register itself.   It  is  updated after each in-

struction is simulated.  The state of the prescaler is not shown.

The  counter is not simulated at this time since there is no  way

to simulate changes in external inputs.

�����������������������������������������������������������������Ŀ

���� Screen Colors ������������������������������������������������

�������������������������������������������������������������������

If you don't have a color monitor or simply don't like all of the

colors  but have a color capable display adapter,  you can  force

S48 to display in monochrome by using the DOS MODE command to set

the   default  display  mode  to  either  BW80   or  MONO.    S48

automatically detects MDA and Hercules display adapters.

�����������������������������������������������������������������Ŀ

���� Distribution �������������������������������������������������

�������������������������������������������������������������������

The S48 package consists of the following files:

� s48.exe

� s48.doc

� demo.asm

� demo.bin

� also_by.wvl

This package and the individual files in it are copyrighted works

of William Luitje.  The author grants you a non-exclusive license

to   copy  and  distribute  these  works  as  long  as  they  are

distributed as a group and are not modified in any way  including

the deletion of any files or addition of other files (such as BBS

ads) in the distribution archive.

On a related note,  I am distributing this package for free.   If

you paid money for a floppy or CD-ROM with this package on it, be

aware that I am not seeing a penny of it.   What you paid for  is

the  service of distribution.   I'm sure that you understand  the

nature of transactions like that but I have heard from people who

"bought  one  of  my  programs"   and  expect  me  to  conduct  a

correspondence  course on basic PC techniques,  theory and repair

because  of  their  "purchase".   While I am always open  to  bug

reports  and  well  thought out questions  and  suggestions  (and

postcards!),   please read the documentation carefully and  check

with local experts before you fire up your word processor.

�����������������������������������������������������������������Ŀ

���� Registration �������������������������������������������������

�������������������������������������������������������������������

While this software may be freely copied and used,  I  do request

that  you  register  it if you use it.   Just send  me  either  a

postcard   or   some  e-mail  telling  which  program   you   are

registering.   I'd prefer a local,  picture postcard;  after all,

whether  it's great or whether it's a pit,  the place  where  you

live is probably interesting in some way, right?

This is only partially an ego massage for the author.   I   enjoy

writing  useful  utilities  and  have  several  other  pieces  of

freeware  out  floating  around  the  net  (see  ALSO_BY.WVL  for

details).    I'd  like  feedback about which  of  them  is  worth

expanding and supporting.   Postware really works!  I support the

programs  which  are  most  used.   I'm not  planning  on  adding

features to this program, but I might if I get enough postcards!

�����������������������������������������������������������������Ŀ

���� Conditions of Use ��������������������������������������������

�������������������������������������������������������������������

The program has not been tested against a real live 8048  because

I  don't  have  access to an 8048  hardware  development  system.

That's why I wrote this simulator in the first place!   The Intel

data  books  I  used were ambiguous in places so I  made  guesses

about  how  some of the subtler points of the  simulation  should

work.    E.g.,  does INC set AC?   Until one of my correspondents

wrote some test software and ran it on real silicon, I could only

guess that it did not.   That turned out to be correct but  there

may be instances where I guessed wrong.  Bug reports are welcome,

especially  when  you  can provide documentation  about  how  the

simulation really should work.

So, keep in mind that this software is provided as is.  I make no

guarantees about it except that it will take up space on any disk

you place it on.  Further, any software you write with the aid of

this simulator is your responsibility.   Verify that it  operates

properly  with  actual  hardware before you use it  in  the  real

world.

William Luitje

2677 Wayside Drive

Ann Arbor, MI 48103

USA

My  e-mail address is somewhat unstable at the current  time  and

one  or both of the following addresses may disappear.   Try  the

first  address  and if that doesn't work,  try the  second.    If

neither works, try looking for "William Luitje"  (I think I'm the

only  one  in  the world!)   on one of  the  WWW  e-mail  address

databases, such as www.four11.com or www.whowhere.com.  I plan to

stay connected so be persistent and you'll get through to me!

luitje@m-net.arbornet.org

luitje@cyberspace.org

�����������������������������������������������������������������Ŀ

���� History ������������������������������������������������������

�������������������������������������������������������������������

Version 1.0  1 Aug 94

Initial public release

Version 1.1 11 Feb 95

Added command to set ports

Added color change of prospective disassembly window after jump,

branch or interrupt.

Version 1.2  24 Nov 95

Added automatic detection and support for MDA/Hercules video cards

Added <ret> as a synonym for T 1

Rewrote user input routine to set background color, handle long

lines and ESC correctly

Added ability to read Intel format hex files (Rainer Glaschick

supplied the code for this)

Fixed several emulation bugs (thanks to Rainer Glaschick,

Leonhard Schneider, Gabriel Patulea and Mike Loving for

their reports)

Version 1.3 23 Jul 96

Added timer emulation

Added a small on-line help screen

Increased speed of emulation by 70%

Fixed an emulation bug (thanks to Motti Grinberg for the

report)

Copyright 1994-1996 by William Luitje