DISKANL.COM - DISKETTE ANALYZER

PROGRAM DOCUMENTATION

by Kevin Patz

BBS: (603) 228-3820

GEnie:  KPATZ

CompuServe: 73007,3311

DISKANL.COM is a program designed to find and identify errors

on floppy disks.  It is compatible with all standard DOS-for#

matted diskettes:  5.25" (single or double-sided, 8-9 sectors per

track), 5.25" high-density 1.2M (AT), 3.5" 720K diskettes, and

3.5" 1.4M diskettes.

DISTRIBUTION:  Unmodified copies of DISKANL may be dis#

tributed freely with this documentation as long as profit is not

involved.

PROGRAM OPERATION

DISKANL is designed to work on floppy disks only.  Hard disks

are not supported at this time, and specifying a hard disk drive

will result in an "invalid drive" message.

To use DISKANL, type DISKANL at the DOS prompt (specify a

drive and/or subdirectory if DISKANL is not in the current

directory on the default drive).  You'll be asked to insert the

diskette to be tested into drive A.  If you want to test a

diskette in drive B (or other floppy drive), put the drive letter

after the DISKANL command, such as DISKANL B:.  Insert the

diskette into the drive indicated and press any key to begin

testing.

DISKANL will begin by identifying the diskette type, and

will report the number of tracks (cylinders), heads, and sectors

per track.  The program will then read the diskette track by

track, indicating the current track on the screen.  If an error

is found, a message will appear indicating the nature of the

error and the location (track, side, sector) of the error.

Disk errors can be caused by one of the following conditions:

The diskette is damaged, either physically (a defect on the

surface or damage such as a fingerprint) or magnetically (the

disk was placed too close to a magnetic object).

The diskette is copy-protected.  Copy protection schemes

either create an oddly formatted track or puts intentional disk

errors on the disk.  Both conditions can cause DISKANL to report

errors.  DISKANL can help to detect whether a disk is copy-

protected or damaged.

Errors that DISKANL can detect:

Track xx head xx is blank:  No sectors were found on this track.

Either the diskette wasn't completely formatted or it is copy-

protected.

CRC error in ID field:  A sector ID field (identifying

information for the sector) contains a CRC (cyclic redundancy

check) error.  A CRC is a "check digit" placed at the end of the

ID and data areas used to determine if the data was read

correctly.  A CRC error in the ID field is caused by some

protection schemes or by a damaged diskette.

Sector not found:  One or more of the standard DOS-formatted

sectors could not be found on the track.  A damaged diskette can

cause this error, and nearly every protection scheme will cause

this error.

CRC error in data field:  The data read from the sector is

stable, but the CRC indicates that the data was not read

correctly.  This error is caused by a copy protection scheme.

Weak bits - unreadable data:  The data read from the sector is

unstable (different every time it's read).  This is most likely

caused by a damaged spot on the diskette, but a few protection

schemes can cause it (particularly those using intentionally

damaged spots).

Address mark not found:  The data address mark (a group of bytes

that identifies the start of the data field) was not found.  A

number of protection schemes can cause this error, or the

diskette is damaged.

Track xx head xx is unrecognizable - probably copy-protected:

This error occurs when a time-out occurs when reading the track.

Some protection schemes with a large number of overlapping

sectors can cause this error.  Removing the diskette from the

drive during analysis can also cause this error.

Other Errors:

Invalid drive letter specified:  The drive you specified on the

command line is nonexistent or not a floppy drive.

Not enough memory:  DISKANL requires at least 70K available

memory to run.

DOS 2.0 required:  DISKANL will not run on versions of DOS

previous to 2.0.

########