OMTIFIX.SYS

by

Steve Gibson

If your system has an OMTI 8620 hard disk controller on a 256 or

512 KB system, you will find that the OMTI controller acquires 1 KB out

of the top of the system board memory at startup, thus reducing your

system to 255 or 511 KB of base memory.  Because there is a hole from

255-256 or 511-512 KB, some software can't backfill to 640 KB.  To

overcome this problem, copy the file OMTIFIX.SYS onto your hard disk and

place a line similar to the following into your CONFIG.SYS file:

Device = d:\path\OMTIFIX.SYS

where "d:\path\" represents the drive and path containing the

OMTIFIX.SYS driver.  Ensure that this line PRECEDES the other device

drivers.  This device driver copies the OMTI's 1 KB region

to low DOS memory and restores your system to 256 or 512 KB of base

memory.  Thanks go to Steve Gibson, Gibson Research Corporation,

22991 La Cadena, Laguna Hills, CA 92653 (714) 830-2200 who writes the

highly regarded Infoworld column "Tech Talk" and is the author of the

great hard disk optimizing and repair utility "SpinRite", for providing

this driver to us.