Sparta PCBoard

Hard Drive Conference

PERSTOR Conference

(201) 729-7056

Perstor PS180 Compatible Drive List

MANUFACTURER      MODEL     CYLS    HEADS    ORIGINAL   CAPACITY

_____________________________________________CAPACITY___W/PS180__

MINISCRIBE     * 8425      615       4      21.4       39.0

* 8438      615       4      21.4       39.0

* 8425F     615       4      21.4       39.0

* 8438F     615       4      21.4       39.0

* 3650      809       6      42.2       77.0

* 6032     1024       3      26.7       48.7

* 6053     1024       5      44.6       81.0

* 6085     1024       8      71.3      130.0

* 6128     1024       8      71.3      130.0

* 6079     1024       5      44.6       81.0

SEAGATE        * ST225     615       4      21.4       39.0

* ST238     615       4      21.4       39.0

* ST4026    615       4      21.4       39.0

* ST251     820       6      42.8       78.0

* ST277     820       6      42.8       78.0

* ST4051    977       5      42.5       77.0

* ST4096   1024       9      80.2      146.0

* ST125     615       4      21.4       39.0

NEWBURY DATA   * NDR1065   918       7      55.9      101.0

* NDR1085  1024       8      71.3      130.0

1105      918      11      87.9      160.0

* 1140     1024      15     133.6      243.7

* 2190     1224      15     159.8      291.4

MAXTOR         * 1065      918       7      55.9      100.0

* 1085     1024       8      71.3      130.0

1105      918      11      87.9      160.0

* 1140     1024      15     133.6      243.7

* 2190     1224      15     159.8      291.4

CDC WREN 2     * 94155-19  697       3      18.2       33.1

* 36        697       5      30.3       55.3

* 38        733       5      31.9       58.1

* 48        925       5      40.2       73.4

* 51        989       5      43.0       78.4

* 57        925       6      48.3       88.1

* 67        925       7      56.3      102.7

* 77        925       8      64.4      117.4

* 86        925       9      72.5      132.1

NEC            * D5126     615       4      21.4       39.0

* D5127     615       4      21.4       39.0

* D5146H    615       8      42.8       78.1

* D5147H    615       8      42.8       78.1

MANUFACTURER      MODEL     CYL     HEADS    ORIGINAL  CAPACITY

_____________________________________________CAPACITY___W/PS180__

MICROSCIENCE     HH-325    615       4      21.4       39.0

HH-725    615       4      21.4       39.0

* HH-1050  1024       5      44.6       81.0

RODIME         * 3055      872       6      45.0       83.0

* 352       306       4      10.0       19.4

LAPINE         * TITAN 20  615       4      21.4       39.0

* TITAN 30  615       4      21.4       39.0

PTI            * PT225     615       4      21.4       39.0

* PT338     615       6      32.1       58.6

* PT238R    615       4      21.4       39.0

* PT357R    615       6      32.1       58.6

PRIAM          * ID130    1224      15     159.8      291.0

*  Denotes  drives that have been tested and approved by  Perstor

Systems,  Inc. for use with the Model PS180 Controller.  Other

drives  may run with Model PS180 and/or Model PS200,  but have

not yet been tested by Perstor Systems, Inc.

------------------------------------------------------------------------------

PERSTOR 200 SERIES 8 BIT CONTROLLER

TECHNICAL SUPPORT MANUAL

SOFTWARE

1.  DiskManager by ONTRACK uses command line DM/M.   Use  ONTRACK

software for partitioning and high-level formatting only.  DO

NOT USE IT FOR LOW-LEVEL FORMATTING.

2.  Speedstor uses command line PARTED/XT or PARTED/NO TYPE.  Use

Speedstor software for partitioning and high-level formatting

only. DO NOT USE IT FOR LOW-LEVEL FORMATTING.

3.  MS-DOS has a partition bug.  You should use PC-DOS for parti-

tioning and then continue with MS-DOS 3.2

4.  MS-DOS  3.31  allows you to make the entire  drive  bootable.

However  it  enlarges the sector size,  so the software  that

checks the file allocation table will fail.

5.  With PC-MOS you must specify in the CONFIG.SYS file the exact

area of memory available.  (i.e. Free memory = C9000, D9000).

You must make sure that the statement does not interfere with

any other addressed cards.

6.  ITT DOS 3.1 and ITT DOS 3.2 allow the entire drive to be  one

bootable volume.

7.  Windows/386 does not work at this time with the PERSTOR Model

PS180.  Microsoft is working on the problem.  WIN86, which is

an  unprotected  mode  program  in  Windows/386,  does  work.

However, it will not run some programs correctly.

8.  Novell  286 does NOT work with the PERSTOR Controllers at the

present time.   However,  software drivers are being  written

for  Novell 286.  In addition,  Novell 86 does work with  the

PERSTOR Controllers.

9.  If  you  have  an AT or AT compatible computer  and  use  any

software  program that checks the CMOS to see what drives are

installed,  this  software will fail.   You must have  an  XT

software switch for this to work.

10. UNIX  and XENIX do NOT work with the current PERSTOR Control-

lers.  However, when the 16 bit controller is available, UNIX

and XENIX will be compatible.

11. The  Sperry  IT (AT compatible) with the  PERSTOR  Controller

installed  will not format floppy diskettes with  the  Sperry

software.   We  suggest PC TOOLS or other software to  format

floppy diskettes.

12. Any  software that is addressed to use ports 1F0 through  1F7

directly  and  software that does not go through a BIOS  call

will fail.

13. The  Low-Level Format (PS2FMT) should take  approximately  10

minutes per 20 megabytes.  If this process takes longer, test

data  cables by rebooting (CTRL-ALT-DEL) back to  the  system

diskette,  then  enter  PS2FMT,  the drive  number,  and  the

interleave value.   If there is NOT a warning message stating

"Drive  Has Already Been Formatted",  then change the 20  pin

data  cable.   If there is a warning message,  then  continue

formatting the drive.

14. The  PS2WP  program  on the Perstor diskette will  clear  all

partitions from the drive.   It is a good idea to use this on

all drives before low-level formatting (PS2FMT).

HARDWARE

1.  WD  1002 Fox Floppy controllers do not work with the  PERSTOR

Model  PS180.   Use a different stand alone floppy controller

or  replace  it  with  the  WD  1003  dual  floppy/hard  disk

controller.

2.  The  Everex  tape backup board must be set to DMA 1  if  used

with the Teac tape backup machine.

3.  Leave terminating resistors on Seagate drives even when using

two drives.

4.  The  Seagate  ST251 will not boot if the first  partition  is

approximately  25  megabytes.   We suggest  using  partitions

under 20 megabytes or using a full 33 megabyte partition.

5.  If the drive does not boot,  check to see if the first parti-

tion is activated.

6.  If  the  Perstor  Copyright message does not  appear  on  the

screen after the RAM count of the AT compatible,  check which

BIOS is being used on the motherboard.   If you are using the

Award BIOS,  you must have either two drives attached to  the

controller or radially jumper the one drive that is attached.

7.  With an Award BIOS on the motherboard, the PERSTOR Controller

will take a longer time to boot because the drive is radially

jumped.   The  controller  will wait for the second drive  to

come ready.  Since there is no second drive,  the  controller

will wait until the timeout occurs, and then it will boot.

8.  The  IBM BIOS used on a clone motherboard will always come up

with  the F1 message.   You must change to the BIOS  designed

for  your specific motherboard.   We suggest using a  Phoenix

BIOS.

9.  Always check to make sure that jumper block JMP6 is  jumpered

correctly for the drives being installed.

10. Check  to  make sure that drive select jumpers on the  drives

are correct according to the type of 34 pin J1 control cable.

For example:

2 drives attached with straight flat cable J1:

Jumper Drive Unit 0 as DS0.

Jumper Drive Unit 1 as DS1.

2 drives attached with twist in cable at Drive Unit 0:

Jumper both drives at DS1

2 drives attached with twist in cable at Drive Unit 1:

Jumper both drives at DS0.

11. If  an  AT compatible computer freezes up during the  initial

startup  after installing the PERSTOR Controller,  check  the

CMOS  to see if there are any hard disks installed.   If  so,

you  must specify that no hard disks are attached before  re-

inserting the PERSTOR Controller into the computer.

----------------------------------------------------------------------------

Perstor Systems, Inc.                              Ronald W. Fife

ADVANCED RUN LENGTH LIMITED CONTROLLER TECHNOLOGY

Perstor Systems,  Inc.  manufactures, markets, and sells two

Advanced RLL controllers, known as the Model PS180 operating at 9

megabits per second, and Model PS200 operating at 10 megabits per

second.   In  general,  the Model PS180 is intended for use  with

oxide  media   having  high  coercivity,   or  with  3  1/2  inch

Winchester media which is plated.  Model PS200 is recommended for

plated  drives or drives with thin film having high  performance,

RLL approved internal circuitry.

As a brief review of the background of controller  interface

technology, originally in 1980, Seagate Technology introduced the

first  ST506   5 1/4 form factor Winchester disk  drive  for  the

small  computer market.   This specification,  which later became

modified and known as the IEEE 412 specification,  dictated  some

general  characteristics  about  the interface to the  drive  and

relied on certain disk drive technology available at the time.

Generally at that time,  the magnetic media had a coercivity

of 300-350 oersted, motor speed tolerance was from 1% to 4%, head

flying  heights were above 12 microns,  and a significant  degree

of error existed in the stepper motor positioning.   The  quality

of  the  recovery  circuit and automatic gain  control  circuitry

within  the  drive  was not as advanced as it is today  in  1987.

Therefore,  the interface specification was limited to 5 megabits

per   second  and  tightly  controlled  as   Modified   Frequency

Modulation  (MFM).  Incidentally,  MFM is in effect a form of Run

Length Limited (RLL) encoding that would typically be called  RLL

1,3.

Later,   two   simultaneous  proposals  for  advancing   the

interface speed were presented:  one by Seagate Technology  known

as the ST506/412HP specification in 1984, and the other by Maxtor

Corporation  known  as the ESDI specification.   The  ST506/412HP

specification  suggested the use of MFM encoding at  10  megabits

per  second  and  added a recovery microstep  capability  to  the

positioner   mechanism,   in  the  belief  that   this   recovery

positioning  was necessary for recovery of the off-track data  at

the higher data rates.

Since the inception of the original ST506 drives,  a  number

of  changes  occurred in the industry.   The  original  ST506/412

controller  designs  did  not  utilize  electronic  checking  and

correction (ECC);  therefore the soft and hard error rates of the

disk drive needed to be kept at an absolute minimum for the drive

to  be  tolerable and usable.   The specification for soft  error

rates  therefore  emerged  at  1 in  10  to  the  10th,  and  the

specification  for  hard  error rates emerged at 1 in 10  to  the

12th.

The  ESDI  specification  is very similar to  the  ST506/412

interface  specification.   However, the data recovery circuit is

located  within the drive in closer proximity to  the  read/write

circuitry  and therefore is more immune to transient  noise.   In

this case,  however, the question of suitable product is really a

system problem.   The finest drive can be reduced to an un-usable

and unsuitable product at any statistical error rate when coupled

with  a bad controller design or a failing controller,  and  vice

versa.

Significant  improvements  have  occurred  in  the  following

areas:

Motor speed tolerance has dropped to 1% or less.

Flying  heights are 10-12 microns and becoming lower.

Coercivity has moved from 350 oersted to 600 and 700 oersted.

Thin film media has become widely available.

Positioner accuracy  has improved.

Heat dissipation has decreased.

Internal  circuitry  for the recovery of the recorded signal  and

digitization of the recorded signal has improved dramatically.

Yet, the fundamental ST506/412 specification has not changed

since  1980.   Therefore the abundant  mass-produced  controllers

simply  are  unable  to benefit from the  improvements  in  drive

technology that have occurred.

Beginning  in  1985,  a large number of  high  quality  disk

drives were available from manufacturers such as Seagate, Maxtor,

Miniscribe,   Newbury  Data,   Fujitsu,  NEC,  and  others  which

incorporated all of the improvements necessary to raise recording

and interface rates.

The  original ST506/412 5 megabit specification  controllers

had  typical  512 byte sectors with 17 sectors  per  track.   The

concept  of  1  1/2  capacity  or 7.5  megabit   per  second  RLL

controllers from Adaptec,  OMTI,  and Western Digital have raised

this sector count to 26 sectors per track.

The  two  models  of the PERSTOR 200  Series  are  what  the

industry has referred to as Advanced RLL,  recording  information

at  9 and 10 megabits per second,  31 and 34 sectors  per  track.

They utilize the same ST506/412 interface and drives as have been

popular  and  known  in  the  industry.  These  controllers  were

introduced in the fall of 1986 and have been installed throughout

the   world  since  that  time.    They  were  introduced   after

approximately  two  years of design and validation testing in  an

effort  to  raise the interface specification for  the  ST506/412

interface,   and  to  make  use  of  the  improvements  in  drive

technology that have occurred over the past few years.

The RLL  recording technique was originally developed by IBM

on  the 3370 drive,  that drive being a soft-sectored  Winchester

style  drive  used  with  the IBM 4300  series.   It  utilized  a

specific  code  structure  for the assignment  of  flux  reversal

patterns  to groups of binary  information.   A group  of  binary

information  is examined and and encoded into an optimal encoding

pattern,  rather  than examining one single bit and  recording  a

simple  clock or data recording method,  which would lead to very

high flux reversal rates if the recording interface was increased

in data rate.

The 3370 relied on a larger ECC word and dictated a  certain

assignment of the code and data patterns.  The method used in the

ST506/412 drives and PERSTOR Controllers is specific in an effort

to assign the high  frequency pattern to the all-zeros condition,

which  normally  would occur in the sector  preamble.   The  code

assignment  will yield flux reversal data rates which are  within

the tolerance of the original ST506/412 data rates (at or below 5

mega flux changes per second).

An example of this structure is the all-ones condition.   It

can  be  noted that at 10 megabits per  second  data  rates,  the

actual  code rate to the drive will only be 5 megabits or mHz per

second,  yielding a flux reversal rate the same as experienced in

MFM at 5 megabits per second.

A problem inherent in the 1 1/2 capacity controllers is that

the  high  frequency pattern does not produce a reversal rate  at

any time which is less than 200 nanoseconds rising edge to rising

edge.   The  7  pattern  of the code  sequence  will  produce  an

exaggerated  time  period  of up to 600 nanoseconds  between  the

rising  edge  of  the flux  reversals  and  encoded  data.   This

compares  to  the worst case MFM rising edge to rising edge  time

period of 400 nanoseconds.

Operating  at  a  higher recording rate,  as the  PS180  and

PS200 Controllers do,  the worst case rising edge to rising edge,

(the  high  frequency pattern),  drops to 168 nanoseconds on  the

PS180 at 9 megabits per second,  or 150 nanoseconds on the  PS200

at  10 megabits per second.   The long wave 7 pattern becomes 448

nanoseconds,  or  400 nanoseconds rising edge to rising edge  for

the respective models.

On  the one hand,  a logical compromise is made at the  high

frequency end,  but the compromise is within the flux change  per

inch  rate of current drives.   On the other hand,  the long wave

side is brought closer to that which would have been expected  by

the drive designers when patterning it after 5 megabit per second

MFM.   Further, the time period between pulses remains at the 100

nanoseconds rate specified by the original ST506 specification.

There  are  many  factors  which affect  these  advanced  RLL

controllers.  The narrower the pulse, the more we are affected by

the following factors:

1.  Media resolution.

These  controllers  are recommended on media that is oxide  media

having high coercivity in the 600-700 oersted range.

2.  Head wobble.

Head  wobble  must be minimized so that the jitter  of  the  data

signal is less than or equal to + 20 nanoseconds.

3.  Motor speed.

Motor  speed  must be less than 1% tolerance and  preferably  .5%

tolerance.

4.  Time domain filters.

The  time  domain filters present in the drive and the  automatic

gain  control circuitry,  which recover the analog  head  signal,

should  be  designed  with the time periods  expected  for  these

controllers in order to achieve optimum results.

5.  Noise.

The  noise  from  the cable system,  the noise  inherent  in  the

controller,  and  the noise inherent in the drive and  its  power

supply must all be reduced.

6.  Ontrack positioning.

The  ontrack  positioning must  be equal to or greater  than  the

current ontrack positioning of the drives that are listed.

7.  Temperature range.

The  temperature  range  of  the product should be  held  to  its

typical  operational  environment rather than  specified  to  the

extreme  limits  that  might be present in 5 megabit  per  second

operation.

At  9 megabits per second,  the encoded data rate,  or  code

data rate,  becomes 6 megabits or 6 mega flux changes per second.

At  10 megabits per second,  the code rate becomes,  in its worst

case mode, 6.6 megabits per second.

When we translate these data rates to flux changes per inch,

in typical 5 1/4 inch media,  we can get an understanding of what

the  media requirements must be.   At 9 megabits per second,  the

flux change per inch rate at the outer track will be approximate-

ly  6,200  and approximately 10,362 at the inner  track.   At  10

megabits  per second,  the outer track flux change rate rises  to

6,900 and the inner track rate rate rises to 11,514.   These flux

change rates will vary based upon media size, the amount of media

that  is  actually utilized by the drive manufacturer  and  their

position, and the rotational rate of the drive.

There  is  a  difference  in the  typical  32  bit  computer

generated  code  probability  for miscorrection and  the  56  bit

computer generated code utilized in the PERSTOR 200 Series.  In a

32  bit computer generated code having an 11 bit drop  out  span,

the probability of a miscorrection is 1 in 10 to the  4th.   Once

an error is detected,  the probability of correcting it wrong and

not  knowing about it is 1 in 10 to the 4th.   With a 56 bit code

having a 22 bit drop out span,  the probability of  miscorrection

rises to 1 in 10 to the 7th.

Because  the  flux reversal rate in the worst case mode  has

risen  above  5 megabits per second,  by  approximately  20%,  we

should expect a worst case raw error rate of the drive to decline

and to be approximately 1 in 10 to the 8th or 1 in 10 to the 9th,

depending  on  the  model number and the characteristics  of  the

drive.   Typically,  a  56  bit  ECC capability  will  raise  the

effective system error rate from 1 in 10 to the 8th to 1 in 10 to

the 12th or greater.

Coupling  with  this   must  be  the nature  of  the  re-try

algorithms  by applying algorithms that insure that a stable  and

repeatable  ECC  has been detected through rotational re-try  and

coupling with it techniques to remove the hysteresis error of the

positioner    during   re-calibration   operations   and   re-try

operations.   All  become additive in the process of raising  the

effective  system  error  rate to values that  are  equal  to  or

greater  than  those experienced with 5 megabit  controllers  and

equal  to  or  greater  than  those  specified  by  the  original

ST506/412 specification.

In  addition,  it  is important that the controller  provide

sufficient  signal recovery intelligence to reject splinter  type

pulses  and noise from the interface before the recovery  circuit

or before the phase lock loop,  and that the phase lock loop  not

be  of  such a design as to errantly track  signal  jitter.   The

cumulative   effect   of  tracking  signal  jitter   raises   the

probability  of pushing one cell out of one window  into  another

and thereby introducing error.

There  are  a  number of other  factors  and  considerations

in these controller designs, which include the overall subject of

noise  reduction,  improved re-try and recovery algorithms,  pre-

compensation on the write side, post-compensation on the recovery

side,

Design  validation included approximately 1 year  of  testing

and is an ongoing process for temperature extreme testing, system

reliability,  and  data reliability,  conducted with most  drives

from  most manufacturers,  new and used drives,  over this 1 year

period  of  time.   We  did not expect,  nor did we  design  this

controller  series to operate with all drives ever  manufactured.

The  older ST506/412,  those drives  with low coercivity  values

and  the other factors which we have discussed are simply not  of

the caliber that must be used with this type of controller.

------------------------------------------------------------------------------

Copyright 1987 Perstor Systems, Inc.

7631 East Greenway Road                       Scottsdale, AZ 85260

.op

GET MORE OUT OF YOUR SYSTEM

The  PERSTOR 200 Series line of  Advanced RLL Controllers  allows

you  to  increase the storage capacity of your hard  disk  drive,

giving  you more room to grow.   These controllers also  increase

the  speed of your computer system,  giving you more time  to  be

productive.

Drive Compatibility

The  PERSTOR  200  Series Controllers work  with  most  ST506/412

Winchester hard disk drives,  either MFM,  RLL, oxide, or plated.

Most  disk drive brands and models have been tested and  approved

for  use  with  the  PERSTOR  Controllers.   Two  hard  disks  of

different  capacities  may  be attached to  the  controllers  for

greater storage space at less cost.

System Compatibility

The PERSTOR 200 Series line is compatible with most PC,  XT,  AT,

or  386 based systems.  Because these controllers use a  standard

IBM compatible BIOS, they work with MS-DOS and PC-DOS.

Speed Performance

Because  the  PERSTOR  200 Series  Controllers  use  higher  data

transfer  rates,  they deliver faster system access time,  giving

you  a  speed  increase  of as much as  110%  over  systems  with

standard MFM or RLL controllers.

THE STORAGE SOLUTION

With  a  PERSTOR Controller you can solve today's and  tomorrow's

hard  disk  storage  problems.   Whether you  are  on  your  last

megabyte on your hard disk or simply want to ensure that you have

plenty of storage capacity for future projects,  the PERSTOR  200

Series is the answer to your needs.

MODELS OFFERED

Model PS180

This  controller  is  intended for MFM or RLL hard  disks  having

oxide or plated media.   This model, which operates at 9 megabits

per  second,  provides  a 90% increase in formatted  capacity  by

allocating 31 sectors per track.

Model PS200

This  controller is intended for RLL approved hard  disks  having

plated  media.   This model,   which operates at 10 megabits  per

second,   completely  doubles formatted capacity by allocating 34

sectors per track.  This is primarily an OEM product.

THE SPECS

PHYSICAL                          RELIABILITY

Length             10.00 in.      MTBF                  25,000POH

Length w/bracket   10.75 in.      PM                         None

Width               4.25 in.      MTTR                 30 minutes

Weight              7.50 oz.      Component Design Life   5 years

AMBIENT TEMPERATURE               DATA TRANSFER RATE *

Operating      50 F to 113 F      4.77 mhz XT       225.6 KB/sec.

(10 C to 45C)      6 mhz AT          220.9 KB/sec.

Non-operating -40 F to 140 F      8 mhz AT          225.0 KB/sec.

(-40 F to 60 C)      10 mhz AT         300.0 KB/sec.

RELATIVE HUMIDITY                 POWER

Operating          8% to 80%      Maximum Dissipation     8 watts

Non-operating      8% to 80%      Requirements        +5V DC + 5%

ERROR RATES

Soft Read Errors     1 per 10   bits transfered

Hard Read Errors     1 per 10   bits transfered

Seek Errors          1 per 10   seeks

* Transfer rates may vary based on system configuration

------------------------------------------------------------------------------

Sparta PCBoard

Hard Drive Conference

PERSTOR Conference

This Bulletin has been prepared to inform you of the various

versions of the Perstor PS200 series controller BIOS that is

available. I will list the current versions that are available

and what their purpose is.

BIOS Version                   Checksum of BIOS

------------                   ----------------

XT9-1.03                            9100

AT9-1.03                            9700

XT10-1.03                           9000

AT10-1.03                           9600

XAT9-1.03                           8900

AT9T-1.01                           1B00

AT9-1.05                            2900

Description of BIOS

XT9-1.03 - Current version of BIOS for PS180 Controller used in

IBM PC/XT and compatible clones.

AT9-1.03 - Current version of BIOS for PS180 Controller used in

IBM PC/AT and compatible clones.

XT10-1.03- Current version of BIOS for PS200 Controller used in

IBM PC/XT and compatible clones.

AT10-1.03- Current version of BIOS for PS200 Controller used in

IBM PC/AT and compatible clones.

XAT9-1.03- Current version of BIOS for PS180 Controller used in

IBM PC/XT or IBM PC/AT where the controller is

installed in an expansion bus box.

AT9T-1.01- BETA version of BIOS to allow formatting to 1224

cylinders for large drives. This BIOS uses the same

methodology as Western Digital to overcome the DOS

limitation of 1024 cylinders. The BIOS does

translations between the controller and the operating

system to make the OS believe that the drive has half

the number of cylinders, but twice the number of heads

on the drive. This BIOS is still in beta test.

AT9-1.05 - Current version of BIOS for PS180 Controller for use

in certain Compaq computers, and AT compatible clones

using the DTK, Award, or AMI bios, to solve an

annoyance or two where the motherboard bios does not

properly handle the reserved stack space at POST time.

If you need a replacement bios for the Perstor controller, and

you bought your controller from Sparta PCBoard, then I will

custom burn a new BIOS for you for the sum of $10. If you bought

your controller elsewhere, the above information is for your use

in determining whether a new bios is in order. You may purchase a

new BIOS from Perstor Systems, Inc. for $25. Please call their

bulletin board at (602) 998-4939 and leave a message to the

Sysop, Ted Buck, that you would like to have your BIOS upgraded.

The information contained in this bulletin is provided for your

information and is offered to any caller to Sparta PCBoard. It is

not to be posted on any other electronic bulletin board, or

distributed via any other means electronic or otherwise.

Richard Driggers

Sysop

Sparta PCBoard

(201) 729-7056

(c) Copyright 1988, Sparta PCBoard.

########