WD1002S-WX2 Hard Drive Controller

The following information on the Western Digital Controller is for

the WD1002S-WX2 taken from the Western Digital Installation Guide.

If you need help doing a low-level format on the drive, please

leave a message in the Hard Drive Conference, Sparta PCBoard

(201) 729-7056.

The BIOS ROM is used to define individual disk drive parameters.

Currently there are two varieties of BIOS ROMs available. The

standard BIOS ROM contains a selection of four sets of drive

parameters that can be used with most existing disk drives.

In addition to this same selection of parameters, another BIOS

ROM, called Auto Config BIOS ROM, allows manual assignment and

modification of these drive parameters; has the ability to load

a drive defect list; and lets the user logically format one disk

drive as two virtual drives.

Both versions of the BIOS ROM contain four drive configuration

tables or BIOS tables as listed below:

BIOS Drive Tables contained within the following BIOS ROMS

Standard - WD62-000042-013

Auto Config - WD62-000043-010

Table 0

Specifications                          Drive Types

Drive Capacity  20MB                    Microscience HH725/HH325

Cylinders       612                     Tandon TM262/362

Heads           4

RWC             613(none)

WPC             613(none)

Step Rate       Option 5

Table 1

Drive Capacity  10MB                    Miniscribe 3012/3212

Cylinders       612

Heads           4

RWC             128

WPC             128

Step Rate       Option 5

Table 2

Drive Capacity  20MB                    CDC 9415-525AT/9415-528

Cylinders       612                     Miniscribe 3425

Heads           4                       Mivil M125/M225/M325

RWC             613(none)               Seagate ST225

WPC             128

Step Rate       Option 5

Table 3

Drive Capacity  10MB                    Fujitsu m2233

Cylinders       306                     LaPine 3065/3512/3522

Heads           4                       Microscience HH612/HH312

RWC             306(none)               Miniscribe 2012

WPC             0 (all)                 Rodime 252/352

Step Rate       Option 5                Seagate ST412

SW1 Jumper Positions 1 - 4

BIOS Table      Drive 1                 Drive 0

SW 1-1  SW 1-2          SW 1-3  SW 1-4

0               Close   Close           Close   Close

1               Open    Close           Open    Close

2               Close   Open            Close   Open

3*              Open    Open            Open    Open

*Shipping Configuration

SW1 Jumper Positions 5 - 8

SW 1-8  Open    Reserved for BIOS ROM

SW 1-7  Open    Reserved for BIOS ROM

SW 1-6  Open    Reserved for BIOS ROM

SW 1-5  Open    Reserved for BIOS ROM

The following jumpers cover the W1 - W7 jumper positions

Jumper  Pin Connects    Status

W1      2-1             Normal*

2-3             Factory Test Only

W2      2-1             Normal*

2-3             Factory Test Only

W3      Closed          BIOS ROM Enabled*

Open            BIOS ROM Disabled

W4      2-3             Device Address 320H*

2-1             Device Address 324H

W5      2-1             BIOS ROM Size 32K or 64K**

2-3             BIOS ROM Size 16K

W6      2-3             Reduced Write Current (8 heads)*

2-1             Head Select 3 (16 heads)

W7      2-1             INTRQ to INTRQ 5 on host connector**

2-3             INTRQ to INTRQ 2 on host connector

* - As Shipped - Jumper is installed

** - As Shipped - Jumper is installed, Pins 1 and 2 are joined via an

etch on the PCB. To change, the etch must be carefully

cut and a jumper installed.

Step Rates

The WD1015 Control Processor uses the CCB Option Byte to establish

the actuator step rate.  This step rate is defaulted to either 70 or

75 microseconds on the installed version of the Control Processor. It

can be modified by the Auto-Config BIOS ROM.  The control processor

is the chip located at U11 on the circuit board.

An older version of the control processor, P/N WD1015-14, supports a

maximum of 8 heads and is completely suitable for the lower capacity

ST506 type disk drives.  The default step rate is 70 microseconds.

Today's control processor, the WD1015-24, supports up to 16 heads. It

includes controller support to the DMA and Syquest removable drives.

Other differences include: it no longer performs the unnecessary

controller self-test after power-on or soft resets; the processor allows

faster step rates; and the default step rate is 75 microseconds.

The Auto-Config BIOS ROM can be used to change the step rate of either

the WD1015-14 or -24 control processors.  Table 5 lists the available

step rates for both the WD1015-14 and the WD1015-24.  Overall system

performance will be enhanced by selecting the fastest possible step rate

supported by the attached drive.

Table 5

WD1015-14 Step Rates

DEC     BIN     Step Rate       Comments

0       000     3 ms

1       001     3 ms

2       010     3 ms

3       011     3 ms

4       100     200 us

5*      101     70 us           Standard Buffered Seek Rate

6       110     3 ms

7       111     3 ms

WD1015-24 Step Rates

0       000     3 ms

1       001     45 us

2       010     60 us

3       011     18 us

4       100     210 us

5*      101     75 us           Standard Buffered Seek Rate

6       110     30 us

7       111     18 us

* Default value in WD BIOS ROMS