October/3/1987

WESTERN DIGITAL CORP. AT FORMAT ROUTINE

WDFMT   2.10

WDFMT can be used on any Western DIgital AT Disk Controller to perform

any of the following tasks.

FORMAT	   do  a low level format of the disk. This will

remove all data from the drive!!!

VERIFY	   verify the entire disk.

BAD TRACK	   mark  bad  tracks that  were found

and/or enter the bad track map.

SURFACE TEST   perform a surface test of the disk. This will

remoxe all data from the drive!!!

In order to format a drive, one must complete the following steps;

1)  Enter drive parameters when prompted by the program

or at the DOS command line as shown below.

WDFMT  drive#   #cyls  #head  #sectors  interleave  precomp-cyl skew

2)  Select the FORMAT option from the main menu and press <CR>.

3)  When the format is complete select the VERIFY option from

the main menu.

4)  When the verify is complete,  select the BAD TRACK option

from  the  main  menu.  The program  will  give a list of

bad tracks found	and ask if the tracks  should be marked

as bad. Next, the user is asked to enter the manufacturer

bad track map.

5)  If a surface test  is  desired  this can	be  done now by

selecting  SURFACE  TEST	from  the  main menu.  When the

test  is	complete,  the	user  should  return to the BAD

TRACK option to mark bad tracks found.

WDFMT can keep track of 300 errors at a time. If more than 300 errors

are found and are to be marked bad,  the user will have to repeat the

VERIFY/SURFACE TEST followed by the BAD TRACK for each group of 300.

In the drive parameter list the user is asked about skew. Skew is the

shifting of sectors  in  a	track relative to an ajacent track in the

same cylinder. Below is an example of a skew of 3.

head 0 :   1  2  3  4	5  6 . . . 13 14 15 16 17

head 1 :  15 16 17  1	2  3 . . . 10 11 12 13 14

head 2 :  12 13 14 15 16 17 . . .  7  8  9 10 11

head 4 :   9 10 11 12 13 14 . . .  4  5  6  7	8

In the drive parameter list the user is asked about alternate sector.

If alternate sector is enabled the formatter will create an alternate

sector that will be used if one sector is found to	be bad during the

verify  or	surface test.  When using this option one less sector per

track will available  to  DOS i.e. if you enter 35 sectors per  track

the formatter will format sectors 1 through 34 and sector 0.