Part of the ENHANCER series of programs EGACHG.COM  Ver. 1.1, Feb 17, 1986

Copyright (C)	I. B. Miller	 1985, 1986

If you find this of value and use it please send a

contribution ($25.00) suggested.

To:  Enhancers 		     It is suggested that companies contribute

231 Otonabee Ave.	      more for use on more than one machine.

Willowdale, Ontario, Canada

M2M-2S9			       Please contact me for a site license

Phone (416) 226-9753

EGACHG is a program which provides a means to preset 12 palette color

combinations for EGACLR.COM a companion program which Controls the seventeen

palette registers on the enhanced color card ( Enhanced Graphics Adapter),

12 different user selected color combinations of any 16 of the 64 colors

available can be made active by use of the Alt key at the same time as any of

the number keys 1 to = at the top of the qwerty keyboard. EGACHG will change

the colors in the Palette registers so that you can see what is available but

unless you use EGACLR these will be lost at the next video reset called by

any program.

The actual colors available to you depend on your actual hardware set

up and so may be different. As an example if you have only a regular monitor

you will have only standard colors though you will be able to swap them around.

In my experience THE limiting factor for text display is the monitor. Though

for graphic display memory size is  also a limiting factor.

General Information

-------------------

With the Enhanced Monitor color coding is as follows.

Bit				  Hex

---				  ---

1 is    Medium	Blue	 - 01

2 is    Medium	Green	 - 02

3 is    Medium	Red	 - 04

4 is Low	intensity Blue	 - 08

5 is Low	intensity Green  - 10

6 is Low	intensity Red	 - 20

1+4 is High	intensity Blue	 - 09	 Low and medium add to give

2+5 is High	intensity Green  - 12	  high	- this is the basic

3+6 is High	intensity Red	 - 24	  principle the card works on

Following is the standard color palette combination.

----------------------------------------------------

B  B  G  C  R  M	B  W  G High intensity colors O

l  l  r  y  e  a	r  h  r --------------------- v

a  u  e  a  d  g	o  i  e  B  G  C  R  M	B  W  e

c  e  e  n     e	w  t  y  l  r  y  e  a	r  h  r

k     n	     n	n  e	 u  e  a  d  g	o  i  s

t		 e  e  n     e	w  t  c

a		    n	     t	n  e  a

a	      n

palette

register # = 0  1  2  3  4  5	6  7  8  9  A  B  C  D	E  F 10

^-this is the

Seventeenth or in hex it

is 10. It really cannot be used

in Enhanced Mode because of a seeming

bug in the EGA. I cannot believe IBM

wanted it that way. Try it and see.

How the program works

---------------------

The program works by loading in the entire EGACLR.COM file

and then selecting the color controlling bytes which are all near

the start as initial values for setting the colors. It then follows

the user through the registersets updating them in memory and

changing them and the colors on display as the user dictates.

Once the user is finished changing colors to his liking and

has pressed the End key EGACHG first checks to see how high in the

sequence of possible EGACLR1.COM to EGACLR99.COM exists on the disk.

once found it increments this and writes out a temporary file. After

it has succeeded in the write it renames the original EGACLR.COM to

EGACLR(whatever #).COM and then renames the temporary file to

EGACLR.COM. In theory you could change names and have almost unlimited

numbers of color combinations which can be made active by using these

variants of EGACLR.COM, but in practice I find it adequate to have

a few and use the command parameters with EGACLR.COM to change <alt 1>

Instructions

------------

EGACHG has been written to work on the EGACLR.COM file on the

same drive as EGACHG.COM. Make sure both are present on the same disk.

Important:  Unless you use the EGACLR o command line to turn off auto reset,

or have not yet activated EGACLR, then EGACLR will proceed to

not allow this program to show the colors to you. You must

Deactivate (or not start) EGACLR before starting EGACHG.

The program will proceed to load into itself the EGACLR.COM file.

It will then paint a window on the top half of the screen showing the

palette colors currently in use with EGACLR use of <Alt 1> which

I call registerset # 1. The program will next paint the bottom window

which is the working area and will display the current register of the 17

included in the current registerset as well as the current value in that

register displayed in both decimal and Hex. It will also show the RGB comp-

onents of the color corresponding to that value.

The screen painted appears approximately as follows with of course the

addition of color.

EGACHG.COM  A program to change colors to any 16 of 64 available on the EGA

for use with a companion program EGACLR.COM which must be on same drive.

Register 0	  1	  2	  3	   4	    5	    6	    7

_______________________________________________________________________

|	     |	     |	      |        |	|	 |	  |	   |

|	     |	     |	      |        |	|	 |	  |	   |

| Blocks of colors appear here and change as palette registers change  |

|________|_______|________|________|________|________|________|________|

|	     |	     |	      |        |	|	 |	  |	   |

|	     under the control of the operator	|	 |	  |	   |

|	     |	     |	      |        |	|	 |	  |	   |

|________|_______|________|________|________|________|________|________|

Register 8	  9	   A	    B	     C	      D        E	F

Below is current color being worked on.

____________________________________________________________________________

|									     |

|									     |

|									     |

|__________________________________________________________________________|

PgUp & PgDn change registerset, Up & Dn change registers, + & - change colors,

End to quit & save , Q to quit & not save! ,W to write BAT file ,R,G,B to vary

Current Palette Registerset is :  5	10,11,12 are 0,-,=

Current Palette Register is :  01 Color is:   2   Color in Hex is :  02

Current RGB values are :	   R= off

G=		 med

B= off

From then on you are in the part of the program that does all the

work and the following keys are the only ones which will do anything.

PgUp	    Moves Up one registerset from 1 to 12 then back to 1

PgDn	    Moves Down one registerset from 12 to 1 then back to 12

Up-Arrow  Moves up one Pallette register from 0 to 16 then back to 0

0 to 10 Hex

Dn-Arrow  Moves Down one Pallette register from 16 to 0 then back to 16

10 to 0 Hex

+	    Increases value in Pallette register from 0 to 63 then back to 0

0 to 3F hex

-	    Decreases value in Pallette register from 63 to 0 then back to 63

3F to 0 hex

R	    Changes Red component of color through off, low, med, high & back

G	    Changes Green component of color through off, low, med, high & back

B	    Changes Blue component of color through off, low, med, high & back

Q	    Allows you to quit without creating an output file

S	    Standardizes colors In current Registerset

W	    writes out a .BAT file to reset colors in Alt 1 . A sample appears

here.

EGACLR 02A,126,202,337,404,505,614,707,818,919,A1A,B1B,C3C,D2D,E36,F3F,s00

End	    Writes out all the values to an EGACLR.COM files while saving the

previous ones as EGACLR1.COM up to EGACLR99.COM

Use the End key when you have completed your selection.

the program will then update EGACLR.COM with your selected

colors available from <alt 1> to <alt =>.

you may then activate the colors by entering   at the DOS prompt

EGACLR R

Disclaimer

----------

For obvious reasons the user is totally responsible for use of this

program and determining that it is suitable for use by him, and he uses this

program with no warranty from myself.

It should work properly with the EGA from IBM or any card which later

maintains compatibility with it

This program and EGACLR.COM are used by myself with LOTUS Development

programs and work well solving a colors shift bug (or less than perfect EGA

driver routine) but obviously LOTUS does not support it.

General

--------

Again please note that if you have the regular monitor or the Enhanced

Monitor operating in regular mode none of the non-standard colors are available

to you!

Of Course

----------

The program EGACHG.COM will allow you to work with many colors and

see them on screen interactively without any of the work involved in

remembering all the numbers and color coding schemes. I believe it will

make color control easy.

If you have any comments or suggestions please contact

me either at Home in the evenings at	       (416) 226-9753

or at the following BBS's   PCanada(pc-1266)   (416) 751-6337

SGV-BBS        (416) 673-0557

I am also on Compuserve as  72167,2563

Ivor B. Miller