RAM4EGA Version 1.20

(C) 1988 - DLF Solutions!

This Device Driver will relocate EGA ROM at Address C000:0000

to RAM, and fix up the vector table to point to the "Shadow RAM". This

will be of benefit to 16 and 32 bit machines (8086/80286/80386) that are

slowed by 8 bit access to the EGA Video Adapter Card.  Since this

Driver searches the low memory vector table for references to the

EGA ROM BIOS Segment it MUST be the first Device entered in your

CONFIG.SYS file (Device=RAM4EGA.SYS). If EGA ROM BIOS is not found at

Address C000:0000 it will do nothing, otherwise it will dramatically

speed up Video BIOS Routines.  As always, use this at your own risk,

however, I would be interested in hearing about any problems.

Douglas L. Fraleigh

5-D Brookline Court

Princeton, NJ 08540

REVISION HISTORY

04/01/88	Version 1.00	First version released.

04/02/88	Version 1.10	Updated to use memory equal in size to

EGA ROM BIOS found rather than 32K default.

(Will only execute on 80286/80386 machines)

07/25/88	Version 1.20	Modified instruction set to allow execution

on 8086 machines.