WESTERN DIGITAL

WD8003-DRV

Sample Software Driver For

WD8003-Family LAN Adapter Boards

Version 3.0

6 April 1988

This driver shows examples of programming the Western Digital

WD8003 family of LAN adapter boards:

Ethernet    WD8003E    EtherCard PLUS

WD8003EBT  EtherCard PLUS With Boot ROM Socket

WD8003ET/A EtherCard PLUS For Micro Channel (*)

StarLAN     WD8003S    StarCard PLUS

WD8003SH   StarLink PLUS

WD8003ST/A StarCard PLUS For Micro Channel

This sample driver is part of an actual operating software

system.  You may use it as is, modify it to suit your particular

environment, or simply use it as an example.

This driver also applies to new boards being developed in the

WD8003 family, such as 10 Mbps/Twisted Pair products, because

these all have the same software interface.

DISK CONTENTS

There is 1 .C file and 1 .ASM file in the MAC layer, with 2 .EQU

files and 5 .H files for definitions of variables, constants and

stucture:

MACMOD.C

WD8STAR2.ASM

FRAMEBLK.EQU

WD8STAR2.EQU

MAC.H

LANCONS.H

MACDEFS.H

MACCONS.H

VERSION.H

Files @6FC0.ADF (for WD8003ET/A) and @6FC1.ADF (for WD8003ST/A)

are sample .ADF files for use with Micro Channel adapter boards.

This document is contained in the file named README.DOC.

The MACMOD.C file contains routines written for the Microsoft

C compiler, Version 4.0.

DESCRIPTION OF FILES

MACMOD.C

This file contains the interface routines to the LLC

sublayer and to the LAN adapter board.

There are six routines in this C file:

1.  init_lanc       One time LAN controller set up routine

2.  stop_mac        Reset 8390, take it off line

3a. init_mac        MAC sublayer initialization routine

3b. start_mac       MAC sublayer start routine

4.  M_DATArequest   Handle a msdu from LLC layer

5.  do_mac_queue    Process rx_queue until it is empty

WD8STAR2.ASM

This asm file and the MACMOD.C file will handle the interface

between upper layer modules and the Western Digital WD8003

family adapter boards.

Since the Microsoft compiler is used, the public varibles

and routines must be named with a leading underscore.

There are six routines in this asm file:

1.  _starlan_wait   Initiate transmit wait loop count for

StarLAN to avoid transmitting back-to-

back packets from AT-class machines to

slower machines

2.  _init_8390      Initiate 8390 Network Interface Controller

Initiate the LAN controller registers

3.  _hdw_isr        Services all LAN hardware interrupts

4.  _cp_frm         Copy a frame in system memory to tx buff

of shared memory

(argument: ptr to the frame buffheader)

5.  _xmt_frm        Initiate a tx of frame in shared memory

6.  _rcv_frm        Copy a rx frame from rx buff ring of

shared memory to system memory and put

it into rx_queue

(argument: ptr to the frame in ring)

FRAMEBLK.EQU

Defines the structure that is used to pass arguments to

public routines.

WD8STAR2.EQU

Provides constants and data definitions for WD8STAR2.ASM file

for WD8003 family adapter boards.

Contents:   external definition

I/O port offset definition

I/O register mask definition

miscellaneous constants definition

structure definition

MAC.H

General header file for Media Access Control module for the

WD8003 family of adapter boards.

LANCONS.H

Machine-specific MAC module constants for the 8390-based

WD8003 adapter board family:

These constants are for a generic IBM PC (*).

MACDEFS.H

Media Access Control Sublayer module data structure

definitions for 8390-based WD8003 adapter board family.

MACCONS.H

MAC module constants for the 8390-based WD8003

adapter board family.

VERSION.H

Contains version level definitions.

MICRO CHANNEL ADAPTER INFORMATION

Earlier versions of the Sample Driver, up through Version 2.1,

did not provide for Micro Channel access.  The following changes

were made for Micro Channel access; the same changes must be

made in any driver written for Western Digital WD8003-family LAN

adapter boards to ensure proper operation with Micro Channel

adapters:

1) Even Boundary Word Mode Access To Shared Memory

Western Digital Micro Channel boards support only word mode

access to their shared buffer memory, and the word access

must be on even boundaries.  Routines using byte-mode access

to shared memory must be changed.  Furthermore, the memory

enable bit pattern for the Micro Channel boards is

different (refer to the init_lanc routine in the MACMOD.C

module).

2) NIC 8390 Programming

Since the Micro Channel boards support only word-mode access,

the 8390 has to be programmed for word-wide DMA transfer.

This can be done by programming a "1" to the D0 bit of the

8390 Data Configuration Register (DCR).

3) Size Of Shared Memory

Instead of 8 Kbytes, the size of the shared memory on

Micro Channel adapter boards is 16 Kbytes.  Therefore, the

size of the receive ring buffer must be adjusted.

4) Level-Sensitive Interrupt

To support the level-sensitive interrupt mechanism used

on all Micro Channel machines, the interrupt service routine

must not attempt to end the interrupt sequence (EOI command)

until it has reset the interrupt line of the 8390.  The

8390 interrupt line can be reset by writing 0xFF to its

Interrupt Status Register (ISR).

Note also that WD8003-family adapter boards for conventional

PC busses support interrupt levels 2-7;  Micro Channel

adapter boards support interrupt levels 3, 4, 10 and 15.

5) Machine Identification

The BIOS call

INT 15H,  (AH) = C0H

can be used at a higher level to decide if the machine on

which the driver is running has a Micro Channel

implementation or not.  The programmer must make this call

and set the _m_channel flag in WD8STAR2.ASM:

Micro Channel      _m_channel = 1

PC/XT/AT           _m_channel = 0

The Sample Driver tests this flag and adjusts its operation

accordingly.

To help programmers make the above changes (1-5) in other

drivers, each change has been marked in this Sample Driver as

follows:

/* %%1%%+ */     Beginning of change 1

- - -

/* %%1%%- */     End of change 1

Changes 2, 3, 4 and 5 are marked in a similar way.  These

markings begin in column 1 of each line to make them easy to

find with a text editor.

To read the configuation of the Micro Channel adapter board

from the POS registers, please refer to the following two

documents:

"POS Implementation Procedure" on page 2-33

of the IBM Personal System/2 Model 50 And 60

Technical Reference (*)

Western Digital WD8003ET/A High Performance

Ethernet Adapter Board For Micro Channel

(Engineering Specification)

Western Digital supplies an ADF file with each Micro Channel

adapter board.  Sample ADF files are located on the Sample

Driver disk:

@6FC0.ADF     For WD8003ET/A (Ethernet)

@6FC1.ADF     For WD8003ST/A (StarLAN)

WD8003 DIAGNOSTIC PROGRAM

The Western Digital WD8003 Diagnostic Program is available to

verify correct operation of WD8003-family adapter boards in a

network and to serve as a tool for testing software.  It can be

used to generate test data as well as to echo data sent to it.

VERSION HISTORY

Following is information about changes that have been made

in this Sample Driver:

Version 3.0  (6 April 1988)

a) The compiler was changed from LATTICE C (Version 3.0)

to Microsoft C (Version 4.0).  The major change was

the use of register variables for higher-speed

execution.

b) The _starlan_wait routine was added.

c) The provisions described above for Micro Channel

adapters were added.

NOTE

This Sample Driver Version 3.0 only addresses 8 Kbtye

shared RAM buffers on PC/XT/AT adapter boards.  The

WD8003EBT Ethernet adapter contains 32 Kbytes of

buffer RAM, and can operate in either 8K or 32K mode.

The next version of this Sample Driver (Version 3.1,

approximately 1 May 1988) will adapt to both

sizes of RAM buffers automatically.

Version 2.1  (6 October 1987)

Widely distributed version that supported adapter boards

for conventional PC bus machines.

FACTORY TECHNICAL SUPPORT

For technical support on Western Digital LAN products, call:

(800) 638-5323 or

(714) 474-2033    ext. 4900

Fax:  (714) 660-4909

TRADEMARKS

EtherCard PLUS, StarCard PLUS and StarLink PLUS are trademarks

of Western Digital Corporation.

(*) IBM and PC AT are registered trademarks, and PC XT, Micro

Channel and Personal System/2 are trademarks of International

Business Machines Corporation.

#############################################################################################################################