�˻�ͻ�˻

����ͼ��� - print  V2.2

�

(C) 1985, 1986, 1987

�˻�ͻ�˻

����ͼ��� enterprises, inc.

�

306 Park Avenue

Wilmette, Illinois  60091

�˻�ͻ�˻

����ͼ��� - print is a memory-resident utility that lets

�        your Epson printer print virtually all of the

characters available on your PC's screen.

Supported Epson printers include the FX, MX, RX,

EX, JX, LX and LQ series models.

Some characters that show on the screen of an IBM

personal computer are actually printer control

characters.  During screen prints, MPM-Print

properly prints ALL characters that can be shown

on the screen.

�˻�ͻ�˻

����ͼ��� - print also supports all of the pitch combinations

�        available for the Epson printers.  If your

software sets your printer to use compressed

print, for example, then MPM-print will

automatically print all of its characters using

the compressed pitch.

�˻�ͻ�˻

����ͼ��� - print AUTOMATICALLY "connects" any of the PC's

�        line drawing characters (like those used for the

MPM logo) both horizontally and vertically, for

any pitch or line spacing you set for your printer.

�˻�ͻ�˻

����ͼ��� - print is distributed as USER SUPPORTED SOFTWARE.

�        If you find it useful, please register with

MPM Enterprises, Inc.  Fees are:

Registration ......................... $10.00

Registration (including a diskette with

the latest version) ............ $15.00

Commercial use without registration is prohibited.

Multiple-copy prices and site licenses are

available.

�˻�ͻ�˻

����ͼ��� - print is provided on an "as-is" basis, and no

�        warranty is expressed or implied.  In no event

shall MPM Enterprises, Inc., be liable for any

commercial or non-commercial damage arising from

the use of MPM-print.

�˻�ͻ�˻

����ͼ��� - print

�

TABLE OF CONTENTS

INTRODUCTION .........................................  1

USING MPM-PRINT ......................................  2

PRINT PITCHES ........................................  4

SCREEN PRINTS ........................................  5

CONNECTED CHARACTERS .................................  6

HOW MPM-PRINT WORKS ..................................  7

SETTING MPM-PRINT ....................................  8

TESTING .............................................. 12

DISTRIBUTION ......................................... 13

TECHNICAL NOTES

SPEED ........................................... 14

LOSING SPECIAL CHARACTERS ....................... 14

SUPPORTED PRINTERS .............................. 15

LQ SERIES PRINTERS .............................. 16

EPSON CONTROL CODES ............................. 16

ADDING CONTROL CODES

(MODIFYING MPM-PRINT FOR COMPATIBLE PRINTERS)  16

SOFTWARE CHECKS FOR MPM-PRINT ................... 18

OTHER PRINT MODES ............................... 19

SETTING PITCHES AT THE PRINTER .................. 20

REINSTALLING MPM-PRINT .......................... 20

PRINT SPOOLERS .................................. 20

COMPATIBILITY ................................... 21

ASCII CODES .................................... APPENDIX

�˻�ͻ�˻

����ͼ��� - print                                      Page 1

�

�˻�ͻ�˻

����ͼ��� - print is a utility which allows an Epson printer

�              to print virtually ALL of the characters

available on your PC's screen.  MPM-print

works in a way that is completely transparent to you and to

any program or operation on the computer (including screen

prints using the PrtSc key).

When it is run, MPM-Print becomes memory-resident, that is,

it stays active in a small portion of your computer's memory.

MPM-Print can be turned off or on, or removed from memory at

your command.

After it is run, MPM-Print allows an Epson printer to print

240 of the 255 graphics characters that IBM PC monitors can

display (which is more IBM characters than even the IBM

Graphics Printer can print).  The only characters that MPM-

Print does NOT print are those that are used to control the

Epson printer.

When MPM-Print has been run, screen prints (using your

computer's PrtSc key) will print ALL the IBM characters.

����������������������������������������Ŀ

� MPM-Print also "connects" the line-    �

� drawing characters (like those used to �

� make this box) -- automatically -- for �

� any line spacing or pitch you set your �

� printer to use.                        �

������������������������������������������

MPM-Print is an ideal addition to your "AUTOEXEC.BAT" file;

if you include the line:

MPMPRINT

in your AUTOEXEC.BAT file, then MPM-Print will become

resident in memory whenever you restart your computer, and

will then always allow IBM's special characters to print on

your printer.

MPM-Print works with Epson FX, MX (with Graftrax Plus), RX,

EX, JX, LX and LQ series printers, AND with the IBM Graphics

Printer (which is a version of the Epson MX printer).  MPM-

Print uses less than 12,000 bytes of system memory after it

is run, and works with PC-DOS versions 2.0 and above.

�˻�ͻ�˻

����ͼ��� - print                                      Page 2

�

USING MPM-PRINT

���������������

You can run MPM-Print simply by typing its name at the DOS

prompt, i.e.

DOS prompt

����������

�

A>MPMPRINT <enter>

�         �

����������Ŀ   �� carriage return or

program name      enter key

MPM-Print will then "attach" itself to DOS, and will display

an initial message.

After you run MPM-Print, just run your software as you

normally do.  Whenever you print something, MPM-Print will

automatically print any of the IBM's "special" characters

that you use.  Whenever your software tells the printer to

change pitch (to go into compressed print, for example) or to

change line spacing, MPM-Print will ALSO automatically

switch.

If MPM-Print has already been run, and you run it again, the

program will give you the following message:

���������������������������������������������������������������Ŀ

�                                                               �

� �˻�ͻ�˻                         �˻�ͻ�˻                   �

� ����ͼ��� - print  V2.2           ����ͼ��� enterprises, inc. �

�    �                                 �                        �

�     (C) 1985, 1986, 1987          306 Park Avenue             �

�                                   Wilmette, Illinois  60091   �

�                                                               �

�            Status is MEMORY RESIDENT and ON(OFF)              �

�                                                               �

�����������������������������������������������������������������

where ON indicates that MPM-Print is active (converting

characters before sending them to the printer), and OFF

indicates that MPM-Print is inactive, and will simply pass

all characters straight to the printer.

�˻�ͻ�˻

����ͼ��� - print                                      Page 3

�

You can also include one of several commands for MPM-Print to

use.  Those commands include:

ON         --  Turns MPM-Print ON (if it is off);

OFF        --  Turns MPM-Print OFF (if it is on);

REMOVE     --  Removes MPM-Print from memory (if it is

already memory-resident);

SETUP      --  Lets you set MPM-Print to use different

printers; and

HELP or ?  --  Displays a help screen which lists the

allowable commands.

The commands are all mutually exclusive.  If you enter more

than one command, MPM-Print will only act on the first one.

MPM-Print will ignore leading spaces or tab characters, and

will ignore the - and / characters if you type them before

one of the commands.  You also do not need to type the full

command name; simply enter enough characters for MPM-Print

to distinguish between the commands.  For example:

MPMPRINT SETUP

MPMPRINT SET

MPMPRINT S

MPMPRINT /S

MPMPRINT -S

will all tell MPM-Print to allow you to set it to use a

different printer.

If you type:

MPMPRINT OFF

at the DOS prompt, it will turn MPM-Print OFF (the program

will stay in memory, but will NOT convert any of the

characters sent to the printer).  To turn MPM-Print ON again,

type:

MPMPRINT ON

To remove MPM-Print from memory, type:

MPMPRINT REMOVE   or

MPMPRINT R

The program will remove itself from memory, and will thus

"free" the memory that it uses for other applications.

Please be careful when removing MPMPRINT to make sure that it

is the LAST memory-resident program that was run.  If you try

to remove MPM-Print, and other programs were made memory

resident after it, then your computer may behave unpredictably.

�˻�ͻ�˻

����ͼ��� - print                                      Page 4

�

PRINT PITCHES

�������������

MPM-Print supports all of the print pitches available on the

Epson printers:

Pitch             Characters per inch      Printers

�����             �������������������      ��������

Pica (normal)            10                All

Elite                    12                All except

MX series

15 CPI                   15                LQ-800,

LQ-1000 &

LQ-2500 only

Compressed               17.16             All

Compressed Elite         20                LX, EX, &

LQ series only

Expanded Pica             5                All

Expanded Elite            6                All except

MX series

Expanded 15 CPI           7.5              LQ-800,

LQ-1000 &

LQ-2500 only

Expanded Compressed       8.58             All

When you set MPM-Print for your printer (see SETTING MPM-

PRINT, page 8), the program will automatically recognize

the pitches supported by your printer.  It will also

automatically recognize the priorities between print pitches

(with FX series printers, for example, elite has priority

over emphasized pica which has priority over compressed

printing).

�˻�ͻ�˻

����ͼ��� - print                                      Page 5

�

Whenever your software sends a command to change pitches to

the printer, MPM-Print AUTOMATICALLY switches to the proper

pitch.  For example:

1         2         3         4

���������������������������������������¶

Pica

������������������¶

#MElite#P

#M������������������¶#P

#Compressed#

#������������������¶#

#Expanded Pica

#������������������¶

##MExpanded Elite#P

##M������������������¶#P

##Expanded Compressed#

##������������������¶#

Other print modes (such as underline, italics, superscript

and subscript) work only for the "normal" characters printed

by the Epson.  The special characters that MPM-Print uses

will still print in the proper pitch, but will not be

underlined, or printed in italics, etc. (See TECHNICAL NOTES,

page 19).

SCREEN PRINTS

�������������

MPM-Print will print ALL of the characters that can show on

your computer's screen when you print the screen.  The only

characters that will not print are the ASCII 0 and the ASCII

255, neither of which are defined for the IBM's screen.

Some characters that can be shown on the screen are also

Epson control characters.  Those characters should be printed

when printing the screen, but sent to the printer unchanged

at other times.  MPM-Print must therefore know when a screen

print is in progress, and sets an internal "flag" whenever

you hit the PrtSc key.

MPM-Print uses your computer's original screen print logic to

actually get information from the screen.

�˻�ͻ�˻

����ͼ��� - print                                      Page 6

�

If you have problems getting screen prints to work properly

with MPM-Print, it means that your computer prints screens in

a way that is significantly different from that used by IBM

PC's and true compatible computers.

If this is the case, you can disable MPM-Print's screen-

printing logic (see SETTING MPM-PRINT, page 8).  If you

disable the screen-printing logic, all special characters

EXCEPT those used as Epson control characters will print

during a screen print -- Epson control characters will be

sent directly to the printer.

The standard screen-printing logic sends only the characters

from the screen to the printer.  No information about the

attribute (bold, or inverse, or colored) of the character is

sent.  Because it works with your computer's existing screen-

printing logic, MPM-Print also does NOT show the attributes

of the characters from the screen.

CONNECTED CHARACTERS

��������������������

MPM-Print automatically connects the IBM line-drawing

characters when they are adjacent, either horizontally (like

����) or vertically (like �

� ).

MPM-Print detects any changes that your software makes to the

line spacing and pitch of your printer, and automatically

adjusts to properly connect the characters -- even at line

spacings as large as one line per inch!

To connect the characters vertically, MPM-Print scans each

line of text as it is printed.  If one of the characters

should connect to the character below (as with the integral

sign in the first paragraph on this section), then MPM-Print

"remembers" that character and its position in the line.

When the original line is done printing, MPM-Print prints a

second line that extends the tails of the appropriate

characters.  MPM-Print automatically adjusts to whatever line

spacing you've set your printer to use.

You can even change print pitch within a line, and MPM-Print

will adjust.  For example:

�����������#W1��������#W0���������͸

� normal � #W1EXPANDED#W0 � normal �

�����������#W1��������#W0���������;

�˻�ͻ�˻

����ͼ��� - print                                      Page 7

�

One warning -- if you switch to compressed mode within a

line, make sure that there are an even number of characters

in the compressed portion of text.  Some Epson printers add

an extra dot's space when switching from compressed print to

Pica print IF an odd number of compressed characters have

been printed.  This apparently happens ONLY when switching

from compressed print to Pica print, and occurs ONLY when an

odd number of compressed characters have been printed.  This

anomaly can, however, make MPM-Print "miss" connecting

vertical lines properly.

Characters that MPM-Print "connects" include:

ASCII CODE 176 177 178 179 180 181 182 183 184 185

CHARACTER  �   �   �   �   �   �   �   �   �   �

ASCII CODE 186 187 188 189 190 191 192 193 194 195

CHARACTER  �   �   �   �   �   �   �   �   �   �

ASCII CODE 196 197 198 199 200 201 202 203 204 205

CHARACTER  �   �   �   �   �   �   �   �   �   �

ASCII CODE 206 207 208 209 210 211 212 213 214 215

CHARACTER  �   �   �   �   �   �   �   �   �   �

ASCII CODE 216 217 218 219 220 221 222     244 245

CHARACTER  �   �   �   �   �   �   �       �   �

HOW MPM-Print WORKS

�������������������

After it has been run, MPM-Print is completely transparent

both to you and to programs running on your computer.

MPM-Print "reviews" each character that is sent to the

printer, and takes one of four actions:

1.  If the character is "shared" by your printer and

IBM monitors, it will be printed normally.  Normally,

only the "standard" ASCII characters, ranging from

the space (ASCII 32) to the ~ (ASCII 126) are shared.

Some printers are, however, able to emulate the IBM

character set  --  for those printers, you can instruct

MPM-Print to "pass" all but the line drawing characters

to the printer, and therefore still make use of MPM-

Print's ability to properly connect the line drawing

characters.  Please see SETTING MPM-PRINT, page 8.

�˻�ͻ�˻

����ͼ��� - print                                      Page 8

�

2.  If you are printing normal text, and if the

character is an Epson control character, it will be

passed without change to the printer.  If that

control character changes the pitch used for printing,

then MPM-Print will remember the new pitch.

If you are printing a screen (using the PrtSc key),

MPM-Print will print even control characters exactly as

they appear on the screen.

3.  If the character is an Escape (ASCII 27), and you

are NOT printing the screen, MPM-Print will send the

Escape character AND the appropriate number of

following characters to the printer without change

(Epson printers use "Escape sequences" to control a

wide variety of printing functions). If the escape

sequence changes the pitch or line spacing used for

printing, then MPM-Print will remember the new

pitch/line spacing.

4.  Finally, if the character is one of the 145

characters that is neither "shared" nor a printer

control character, then MPM-Print will "translate" the

character and print the IBM monitor's version of that

character using the Epson printer's graphics

capability.

If the character is a line-drawing character (see

CONNECTED CHARACTERS, page 6), then MPM-Print will

remember its location, and extend the tail of the

character so that it connects with the character below.

SETTING MPM-PRINT

�����������������

MPM-Print is initially set up to run with an Epson FX-80

printer.  If you've got another model of printer, then you

should use the SETUP command with MPM-Print to make sure that

the program will work correctly with your printer.

�˻�ͻ�˻

����ͼ��� - print                                      Page 9

�

To set up MPM-Print, at the DOS prompt type:

MPMPRINT SETUP      or

MPMPRINT S

You'll see the message:

���������������������������������������������������������������Ŀ

�                                                               �

� �˻�ͻ�˻                       �˻�ͻ�˻                     �

� ����ͼ��� - print  V2.2         ����ͼ��� enterprises, inc.   �

�    �                               �                          �

�     (C) 1985, 1986, 1987        306 Park Avenue               �

�                                 Wilmette, Illinois  60091     �

�                                                               �

�                                                               �

� Press the Escape key to stop now, or press any other key to   �

� continue setting up MPM-Print.                                �

�                                                               �

�����������������������������������������������������������������

If you press the Escape key (either now or at any other

prompt during the setup routine), then MPM-Print will simply

return you to the DOS prompt.  If you press any other key,

then you'll see:

����������������������������������������������������������������������Ŀ

�                                                                      �

�                           MPM-Print Setup                            �

�                                                                      �

� FX-80                             LQ-800                             �

� FX-100                            LQ-1000                            �

� FX-80 with Letter Quality ROM     LQ-2500                            �

� FX-100 with Letter Quality ROM    LX-80                              �

� FX-80+                            LX-86                              �

� FX-100+                           MX-80 with GRAFTRAX (old GRAFTRAX) �

� FX-80+ with Letter Quality ROM    MX-80 with GRAFTRAX+ (Type III)    �

� FX-100+ with Letter Quality ROM   MX-100 Type II (original MX-100)   �

� FX-85                             MX-100 with GRAFTRAX+ (Type III)   �

� FX-185                            RX-80                              �

� FX-285                            RX-80 F/T                          �

� FX-86e                            RX-100                             �

� FX-286e                           RX-80+                             �

� JX-80                             RX-80 F/T+                         �

� JX-80 with Letter Quality ROM     RX-100+                            �

� EX-800                            RX-80+ with Letter Quality ROM     �

� EX-1000                           RX-80 F/T+ with Letter Quality ROM �

� LQ-1500 version 1.8               RX-100+ with Letter Quality ROM    �

� LQ-1500 version 2.0                                                  �

�                                                                      �

������������������������������������������������������������������������

One of the printers will be "highlighted"; use the cursor

keys (on the right hand side of your keyboard) to move the

"highlight" to your printer.

�˻�ͻ�˻

����ͼ��� - print                                      Page 10

�

The list of printers is so extensive because even similar

printers will often have subtly different command sets.

If you're using an "Epson-compatible" printer, choose the

model that your printer emulates.  You may need to experiment

a little to find the Epson model whose command set is closest

to that used by your printer.

When you've chosen the model of printer that you use, simply

hit the Return (or Enter) key.

NOTE: For the Epson MX-series printers, most of the models

available in this country used the GRAFTRAX+ module.  Printers

produced by Epson with the GRAFTRAX+ chip should all be labelled

TYPE III somewhere on the back of the printer.  Therefore, if your

MX printer is a TYPE III, then you can be reasonably confident

that it has the GRAFTRAX+ installed.

However, even if your MX printer is NOT a TYPE III, there is

also a good chance that it has the GRAFTRAX+ (rather than an

older GRAFTRAX chip) installed.  Epson provided user upgrades

so that older GRAFTRAX chips were usually upgraded to the

GRAFTRAX+.

Therefore, if you are not sure whether you have the GRAFTRAX+

or an older version, first try configuring MPM-Print for

the MX-80 (or MX-100) with GRAFTRAX+.  Run our TEST program

(see page 12); if the printer works without problems, then

your printer has the GRAFTRAX+ chip.

�˻�ͻ�˻

����ͼ��� - print                                      Page 11

�

The setup routine will ask next about screen prints:

���������������������������������������������������������������Ŀ

�                                                               �

� Some characters that show on the screen are printer control   �

� characters -- MPM-Print can translate those characters when   �

� you print the screen using the PrtSc key.  This feature may   �

� not, however, work properly with some "IBM-compatible"        �

� computers.  If you have problems with screen prints, then you �

� can stop MPM-Print from trying the translation by answering   �

� No to the next question:                                      �

�                                                               �

� Should MPM-Print "translate" all characters on the screen     �

� during a screen print?  >> _                                  �

�                                                               �

�  Press Y for Yes, anything else for No, Escape key to abort   �

�                                                               �

�����������������������������������������������������������������

If you answer Yes, then MPM-Print will translate all

characters that appear on the screen when you do a screen

print.  We suggest that you answer No ONLY if you try screen

prints with MPM-Print and have problems (and if you do have

problems, please notify us).

If you answer Yes, the MPM-Print will pause briefly while it

determines how the print screen function works on your

computer.

If MPM-Print is already memory-resident, the setup routine

will then ask you:

���������������������������������������������������������������Ŀ

�                                                               �

� MPM-Print has already been run.  Should the copy in memory be �

� updated with the new printer information?  >> _               �

�                                                               �

�����������������������������������������������������������������

If you answer Yes, then the copy of MPM-Print that is in

memory will be updated for the new printer that you've

chosen.

The setup routine will next ask:

���������������������������������������������������������������Ŀ

�                                                               �

� Should an updated copy of MPMPRINT.COM be saved to disk?  >> _�

�                                                               �

�����������������������������������������������������������������

�˻�ͻ�˻

����ͼ��� - print                                      Page 12

�

If you answer Yes, then a new copy of MPMPRINT.COM will be

written to the current directory.  If MPMPRINT.COM already

exists in the current directory, it will be replaced.

Finally, if MPM-Print is NOT already memory resident, then

the setup routine will ask:

���������������������������������������������������������������Ŀ

�                                                               �

� MPM-Print was not found in memory.  Should a copy be made     �

� memory-resident now?  >> _                                    �

�                                                               �

�����������������������������������������������������������������

If you answer yes, then MPM-Print will install itself in

memory, using all of the new information that you entered.

TESTING

�������

Included with MPM-Print is a testing program, TEST.PAS.

Written in TURBO Pascal, the testing program "exercises" most

of the print commands available for the Epson printers.

The program prints the full character set for each of the

print pitch combinations, and also tests combinations of

print modes and pitches.  With some Epson printers, certain

print modes have "priority" over other modes.  For example,

for FX series printers, Elite pitch has priority over

compressed print which has priority over Pica print.

For each control code and escape sequence, the program sets

the printer's mode, then (if appropriate) it prints the 240

non-control characters, and finally restores the printer to

its original mode.

The purpose of the test is to verify that MPM-Print properly

transmits the control codes/escape sequences to the printer,

and that it properly prints the full 240 MPM-print character

set after setting the printer's mode.

TEST.COM is a compiled version of TEST.PAS.  It was compiled

using TURBO Pascal version 3.0, and requires DOS version 2.0

or later.

The compiled version of TEST takes about 10-15 minutes to run

(it is limited by the speed of the Epson printers), and

prints ten to fifteen pages.

�˻�ͻ�˻

����ͼ��� - print                                      Page 13

�

DISTRIBUTION

������������

MPM-Print is distributed as "user-supported software", with

the following conditions:

MPM-Print and its associated documentation are the

copyrighted property of MPM Enterprises, Inc.

We grant NON-COMMERCIAL users the right to use, copy

and share MPM-Print.

Any distribution of MPM-Print must contain unaltered

versions of the program itself and all associated

documentation:

MPMPRINT.COM

MPMPRINT.DOC

PRINTDOC.EXE

READ    .ME

TEST    .PAS

TEST    .COM

MPM-Print may NOT be sold or distributed for profit

without the express, written consent of MPM Enterprises,

Inc.

We encourage you to register your copy with MPM

Enterprises, Inc.  Fees are:

Registration ............... $10.00

Registration (including a

diskette with the latest

version) ................. $15.00

Registered users will be notified of updates,

enhancements, and new products from MPM Enterprises, Inc.

Registered users will also receive instructions to

disable most of the initial ("sign-on") screen shown

by MPM-Print.

Commercial use WITHOUT a $10 per copy registration fee is

prohibited.  Multiple-copy prices and site licenses are

available.  Dealer pricing is also available.

MPM-Print is provided on an "as-is" basis, and no warranty is

expressed or implied.  In no event shall MPM Enterprises,

Inc., be liable for any commercial or non-commercial damage

arising from the use of MPM-Print.

�˻�ͻ�˻

����ͼ��� - print                                      Page 14

�

TECHNICAL NOTES

���������������

SPEED

�����

MPM-Print uses the graphics printing capabilities of the

Epson printers to produce the 145 IBM characters that the

Epson printers normally don't print.

Printing using graphics modes requires more information and

more effort on the part of the printer, so the 145 non-Epson

characters take longer to print than do the "normal"

characters.  "Normal" text is printed at the normal speed.

To help speed up printing of the special characters, MPM-

Print uses a special internal buffer and is optimized for

speed.

Connecting special line-drawing characters vertically

requires MPM-Print to print additional "lines" between the

original line of characters and the next line -- this, too,

adds to the time required to print special characters.  No

extra printing time is needed if there are no characters that

need to be "connected" to the line below.

There is one problem with using the graphics buffer to speed

printing: the Epson printers do NOT "wrap" graphics

characters at the right margin -- any characters remaining in

the graphics buffer can be lost.  Please see LOSING SPECIAL

CHARACTERS below.

LOSING SPECIAL CHARACTERS

�������������������������

MPM-Print uses a "graphics buffer" to speed up printing

special characters.  Characters stored in the buffer will be

held either until the buffer is full, or until a non-special

character is printed.

If something happens BEFORE the buffer is printed (such as a

"crash" or a reboot of your computer), then the characters

will be lost.

A second problem can occur with special characters -- Epson

printers DO NOT WRAP characters printed using graphics modes.

If special characters extend past the right margin, the

printer simply truncates the line at the right margin.  In

contrast, Epson printers automatically "wrap" lines of normal

text that are too long (text that would extend past the right

margin is printed on the next line.)

This problem will occur ONLY when the information being

printing is improperly formatted, and is therefore easy

to avoid.

�˻�ͻ�˻

����ͼ��� - print                                      Page 15

�

SUPPORTED PRINTERS

������������������

Printer Number    Printer Model

��������������    ������������������������������������

0          FX-80

1          FX-100

2          FX-80 with Letter Quality ROM

3          FX-100 with Letter Quality ROM

4          FX-80+

5          FX-100+

6          FX-80+ with Letter Quality ROM

7          FX-100+ with Letter Quality ROM

8          FX-85

9          FX-185

10          FX-285

11          FX-86e

12          FX-286e

13          JX-80

14          JX-80 with Letter Quality ROM

15          EX-800

16          EX-1000

17          LQ-1500 version 1.8

18          LQ-1500 version 2.0

19          LQ-800

20          LQ-1000

21          LQ-2500

22          LX-80

23          LX-86

24          MX-80 with GRAFTRAX (old GRAFTRAX)

25          MX-80 with GRAFTRAX+ (Type III)

26          MX-100 Type II (original MX-100)

27          MX-100 with GRAFTRAX+ (Type III)

28          RX-80

29          RX-80 F/T

30          RX-100

31          RX-80+

32          RX-80 F/T+

33          RX-100+

34          RX-80+ with Letter Quality ROM

35          RX-80 F/T+ with Letter Quality ROM

36          RX-100+ with Letter Quality ROM

The list of printers is so extensive because there are

typically subtle differences in the command set supported by

printers even in the same family.  For example, the FX-80+

printer supports certain commands that are not available for

the older FX-80 printer.

�˻�ͻ�˻

����ͼ��� - print                                      Page 16

�

LQ SERIES PRINTERS

������������������

MPM-Print completely supports all of the print pitches and

commands used by the LQ series printers.  HOWEVER, MPM-Print

only uses 8-pin graphics when printing its special

characters.  The standard version of MPM-Print does not

approach the print quality available from the 24-pin standard

LQ character set.

We have a version of MPM-Print (called MPM-LQ) devoted

exclusively to the LQ series printers.  Please contact MPM

Enterprises, Inc. for information about MPM-LQ.

EPSON CONTROL CODES

�������������������

Epson printers use certain one-byte codes (ranging from ASCII

7 to 27) to control some of the printers' features.  Each

feature can be activated by sending EITHER the code OR the

code+128 to the printer (for example, both ASCII 7 and ASCII

135 would "beep" the printer's speaker).  We have chosen to

print IBM characters for all ASCII codes above 127, since

printer control codes are already provided by ASCII

characters 7-27.

We have also chosen to print the IBM character  (ASCII 127)

instead of the "delete" code for the Epson printers.  The

"delete" code deletes the last character in the Epson

printer's line buffer; since it is rarely used, we chose to

print the IBM character instead.

All other control codes and escape sequences are passed

directly to the printer.

ADDING CONTROL CODES (MODIFYING MPM-PRINT FOR COMPATIBLE PRINTERS)

������������������������������������������������������������������

Some printers are Epson-compatible, but extend the control

codes that can be sent to the printer.  You can "patch" MPM-

Print to recognize (and pass) those new control codes.

A new control code must have three characteristics to allow it

to be added to MPM-Print:

1. The control code must begin with the Escape

character (ASCII 27);

2. The escape character must be followed by a code

character that is between ASCII 0 and ASCII 127;

3. Following the code character must be a constant

number of additional characters.  That constant

number can be anything from 1 to 127.

�˻�ͻ�˻

����ͼ��� - print                                      Page 17

�

For example, the ADS 2000 printer emulates the FX-80 printer,

but extends the control codes.  One of its additional control

codes is:

Esc 'a' chr$(n)

which turns ON high density printing if n=1, and turns it OFF

if n=0.

This "new" control code fits all of the characteristics

required for you to add it to MPM-Print: It begins with an

Escape character; the Escape is followed by the character 'a'

(ASCII 97); and the 'a' is followed by one additional

character.

The "patch" area for MPMPRINT.COM begins at an offset of 270

bytes (270 decimal, 10E hex) from the beginning of the file,

and is 256 bytes long.

To patch MPMPRINT.COM using DEBUG (the DOS utility), first

work ONLY on a copy of MPM-Print.  Next, set up that copy for

the Epson model that your printer most closely emulates (see

SETTING MPM-PRINT, page 8).  Finally, determine the offset

for the code character you want to add:

Offset = 100H + 10EH + 2 * character code in hex

(using DEBUG)

where H indicates a hexadecimal number.

For our example, the character code 'a' is 61H.  Its offset

is 100H + 10EH + 2 * 61H = 20EH + C2H = 2D0H.

Make sure the DOS program DEBUG.COM is available, either in

the current directory or along the PATHs you've set for your

computer, and type:

DEBUG MPMPRINT.COM

At the DEBUG prompt, type:

e <offset>   where <offset> is the

offset you've calculated,

in hexadecimal

In our example, you'd type:

e 2D0

�˻�ͻ�˻

����ͼ��� - print                                      Page 18

�

DEBUG responds with:

xxxx:02D0  00.

where xxxx is the code segment value.  The 00 is the value

currently stored at that offset.

Enter the number of characters that follow the code character

(remember, you've got to enter the number in hexadecimal).

In our example, the 'a' is followed by one character, so we'd

enter:

xxxx:02D0  00.01

then hit the space bar (which tells DEBUG to let you enter

the next byte), giving:

xxxx:02D0  00.01  00.

and enter 0 (if the next byte is not already 0), followed by

a carriage return.  Please note that the second zero byte is

essential to having your modified code work properly.

When you've done this, type w at the DEBUG prompt to Write

the file to disk (skip this step if you want to quit WITHOUT

changing the disk's copy of MPMPRINT.COM).

To quit, type the letter q at the DEBUG prompt.

SOFTWARE CHECKS FOR MPM-Print

�����������������������������

Version 2.2 of MPM-Print gives your programs a way to check

to make sure that MPM-Print is resident in memory, and to

check whether MPM-Print is active or not.  To check, your

software must be able to use the BIOS Printer I/O interrupt

(17H), and use the 8088/8086/80286 registers.

Contents

Register   Hex   Decimal

AX     0CAFE   51966

BX       0       0    to check status

1       1    to turn MPM-Print ON

0FFFF    -1    to turn MPM-Print OFF

To test if MPM-Print is resident, put 0CAFEh in the AX

register, and either 0, 1, or -1 in the BX register (see the

table above), and execute an interrupt 17h.

�˻�ͻ�˻

����ͼ��� - print                                      Page 19

�

Upon return from the interrupt 17h:

Contents

Register   Hex   Decimal

AX     0FEED   65261

BX       0       0    MPM-Print is OFF

1       1    MPM-Print is ON

DX                    printer number, from the

table under SUPPORTED

PRINTERS, page 15

If AX equals anything other than 0FEEDh, then MPM-Print is

NOT installed.  If AX does equal 0FEEDh, then the BX register

will indicate whether MPM-Print is active or not, according

to the table above.

The TEST program gives a TURBO Pascal example of testing to

make sure that MPM-Print is installed and active, and of

retrieving the number of the printer for which MPM-Print was

installed.

OTHER PRINT MODES

�����������������

MPM-Print supports all of the pitches available for Epson

printers.  Most of the other print modes available will not

affect the way MPM-Print prints the "special" characters:

Print Mode      Action

����������      ������

Graphics        All graphics modes work properly.

Underline       "Normal" characters are underlined;

MPM-Print's characters are not.

Italics         Normal characters are printed in

italics; special characters are

not.

Emphasized      With the Pica pitch, special

characters are emphasized; with

Expanded Pica, only normal

characters are emphasized.

Double Strike   Only normal characters are "double-

struck".

Super- and      Only normal characters print in

Subscript       the script mode; special characters

print at their normal heights.

Proportional    Only normal characters print

proportionately; special characters

print with the current pitch.

�˻�ͻ�˻

����ͼ��� - print                                      Page 20

�

SETTING PITCHES AT THE PRINTER

������������������������������

If your software sends printer control characters to the

printer, then MPM-Print will always know the proper pitch to

use.

If, however, you change pitches for the printer WITHOUT MPM-

Print's knowledge, (for example, by using printer switches)

then MPM-Print will use the last known pitch when printing

special characters.

Suppose you were printing in compressed pitch, and turned the

printer OFF.  When you turn the printer ON again, it will

print in the Pica pitch.  MPM-Print, however, will continue

to print the special characters in compressed pitch.

To solve the problem of confused pitches, send the printer

reset code (Escape 64) to the printer.  MPM-Print will

automatically reset its pitch.

REINSTALLING MPM-PRINT (or trying to)

�������������������������������������

MPM-Print works by taking control of IBM's ROM BIOS interrupt

17h (Printer I/O). When you try to re-run MPM-Print, it

checks the ROM BIOS interrupt to see if it has already been

run.  If it has, MPM-Print notifies you of that fact.

A problem can occur, however, if you run other memory-

resident software (such as a print spooler; please read the

PRINT SPOOLERS section below) that also takes control of

interrupt 17h AFTER MPM-print has been run.  If this is the

case, and IF you try to re-run MPM-Print, it may not find a

copy of itself when it checks.  If MPM-Print does NOT, then

it WILL proceed to install itself again.

PRINT SPOOLERS

��������������

Print spoolers are memory-resident programs that "reserve" a

portion of memory to buffer information sent to the printer.

To achieve good performance, most print spoolers completely

replace the ROM BIOS printer interrupt logic.  Therefore,

most print spoolers will completely ignore MPM-Print if they

are run AFTER MPM-Print.  This problem occurs when the

spooler (or any other memory-resident program) replaces

rather than chains to the previous printer interrupt.  If you

are using one of those spoolers, then please install MPM-

Print AFTER installing your print spooler.

�˻�ͻ�˻

����ͼ��� - print                                      Page 21

�

COMPATIBILITY

�������������

We have not been able to test MPM-Print with every

available piece of software.  We HAVE, however, tested

it with:

IBM PC-DOS versions 2.1,3.0,3.1,3.2

Lotus 1-2-3/Printgraph

Graphics screen dumps (as long as the

DOS utility GRAPHICS.COM is also

installed)

TimeLine project management system

from Breakthrough Software

MathCAD from MathSoft, Inc.

Sidekick (from Borland International)

Text files produced with a variety of

word processing programs.

If you have compatibility problems with other memory-

resident programs, try changing the order in which the

programs are loaded into memory.  If you discover

compatibility problems that you are unable to solve,

please let us know about them.

Some word processing packages try to do the same job as

MPM-Print -- they convert the IBM graphics characters

into graphics commands before sending the information to

the printer.  Because the characters have already been

converted BEFORE MPM-Print ever sees them, MPM-Print is

unable to do its job.

If MPM-Print works with some of your software, but you find

that the "line drawing" characters are not properly connected

when you print a document from your word processor, it is

most probably because your word processor is converting the

line drawing characters.  Try "reinstalling" your word

processor for another printer (try the IBM Graphics printer

first) to find a printer driver that instructs your word

processor to send ALL characters WITHOUT CHANGE to the printer.

�˻�ͻ�˻

����ͼ��� - print                                      Page 22

�

���������������������������������������������������������������

�˻�ͻ�˻

MPM-Print, ����ͼ��� - print, and its associated documentation

�

are copyright (C), 1985, 1986, 1987, MPM Enterprises, Inc.

Please direct all comments and registrations to:

�˻�ͻ�˻

����ͼ��� enterprises, inc.

�

306 Park Avenue

Wilmette, Illinois  60091

���������������������������������������������������������������

Special thanks are due to Ms. Carole Anne Ogdin of Semiotics

Designers for her suggestions about command line parsing and

the logic for the REMOVE command for MPM-Print.

�˻�ͻ�˻

����ͼ��� - print                                 APPENDIX 1

�

APPENDIX

��������

ASCII CODES

�����������

Decimal

� Hexadecimal

�

�   �   Character or function

�   �

�   �   �

�   �   �

0  00           32  20           64  40   @       96  60   `

1  01   #       33  21   !       65  41   A       97  61   a

2  02   #       34  22   "       66  42   B       98  62   b

3  03   #       35  23   #       67  43   C       99  63   c

4  04   #       36  24   $       68  44   D      100  64   d

5  05   #       37  25   %       69  45   E      101  65   e

6  06   #       38  26   &       70  46   F      102  66   f

7  07 BEL       39  27   '       71  47   G      103  67   g

8  08  BS       40  28   (       72  48   H      104  68   h

9  09  HT       41  29   )       73  49   I      105  69   i

10  0A  LF       42  2A   *       74  4A   J      106  6A   j

11  0B  VT       43  2B   +       75  4B   K      107  6B   k

12  0C  FF       44  2C   ,       76  4C   L      108  6C   l

13  0D  CR       45  2D   -       77  4D   M      109  6D   m

14  0E  SO       46  2E   .       78  4E   N      110  6E   n

15  0F  SI       47  2F   /       79  4F   O      111  6F   o

16  10   #       48  30   0       80  50   P      112  70   p

17  11 DC1       49  31   1       81  51   Q      113  71   q

18  12 DC2       50  32   2       82  52   R      114  72   r

19  13 DC3       51  33   3       83  53   S      115  73   s

20  14 DC4       52  34   4       84  54   T      116  74   t

21  15   #       53  35   5       85  55   U      117  75   u

22  16   #       54  36   6       86  56   V      118  76   v

23  17   #       55  37   7       87  57   W      119  77   w

24  18 CAN       56  38   8       88  58   X      120  78   x

25  19   #       57  39   9       89  59   Y      121  79   y

26  1A           58  3A   :       90  5A   Z      122  7A   z

27  1B ESC       59  3B   ;       91  5B   [      123  7B   {

28  1C   #       60  3C   <       92  5C   \      124  7C   |

29  1D   #       61  3D   =       93  5D   ]      125  7D   }

30  1E   #       62  3E   >       94  5E   ^      126  7E   ~

31  1F   #       63  3F   ?       95  5F   _      127  7F   

NOTE: Character 26 is not shown here, although it is printed

by MPM-print.  ASCII 26 is the end-of-file (EOF) marker used

by a number of programs; many programs will not read past the

character.

�˻�ͻ�˻

����ͼ��� - print                                 APPENDIX 2

�

APPENDIX (continued)

��������������������

ASCII CODES

�����������

Decimal

� Hexadecimal

�

�   �   Character or function

�   �

�   �   �

�   �   �

128  80   �      160  A0   �      192  C0   �      224  E0   �

129  81   �      161  A1   �      193  C1   �      225  E1   �

130  82   �      162  A2   �      194  C2   �      226  E2   �

131  83   �      163  A3   �      195  C3   �      227  E3   �

132  84   �      164  A4   �      196  C4   �      228  E4   �

133  85   �      165  A5   �      197  C5   �      229  E5   �

134  86   �      166  A6   �      198  C6   �      230  E6   �

135  87   �      167  A7   �      199  C7   �      231  E7   �

136  88   �      168  A8   �      200  C8   �      232  E8   �

137  89   �      169  A9   �      201  C9   �      233  E9   �

138  8A   �      170  AA   �      202  CA   �      234  EA   �

139  8B   �      171  AB   �      203  CB   �      235  EB   �

140  8C   �      172  AC   �      204  CC   �      236  EC   �

141  8D   �      173  AD   �      205  CD   �      237  ED   �

142  8E   �      174  AE   �      206  CE   �      238  EE   �

143  8F   �      175  AF   �      207  CF   �      239  EF   �

144  90   �      176  B0   �      208  D0   �      240  F0   �

145  91   �      177  B1   �      209  D1   �      241  F1   �

146  92   �      178  B2   �      210  D2   �      242  F2   �

147  93   �      179  B3   �      211  D3   �      243  F3   �

148  94   �      180  B4   �      212  D4   �      244  F4   �

149  95   �      181  B5   �      213  D5   �      245  F5   �

150  96   �      182  B6   �      214  D6   �      246  F6   �

151  97   �      183  B7   �      215  D7   �      247  F7   �

152  98   �      184  B8   �      216  D8   �      248  F8   �

153  99   �      185  B9   �      217  D9   �      249  F9   �

154  9A   �      186  BA   �      218  DA   �      250  FA   �

155  9B   �      187  BB   �      219  DB   �      251  FB   �

156  9C   �      188  BC   �      220  DC   �      252  FC   �

157  9D   �      189  BD   �      221  DD   �      253  FD   �

158  9E   �      190  BE   �      222  DE   �      254  FE   �

159  9F   �      191  BF   �      223  DF   �      255  FF