HGCIBM.COM Version 2.02

Copyright (C) Athena Digital, Gary Batson 1986, 1987

2351 College Station Road, Suite 567, Athens, Ga. 30605

404/354-4522

This Manual Dated June 1, 1987

1

TABLE OF CONTENTS

HGCIBM.COM

Acknowledgments ..............................................  3

Trademarks ...................................................  3

Using HGCIBM.COM .............................................  3

Precautions ..................................................  3

Installation .................................................  4

Using the Command Line .......................................  5

HGCTEST program ..............................................  5

Sales and Distribution Policies ..............................  6

User Comments ................................................  6

History ......................................................  6

Future .......................................................  7

Changes and Corrections ......................................  7

Technicalities ...............................................  8

Athena Digital Video BIOS

Purpose ......................................................  9

Features .....................................................  9

Control Program .............................................. 11

Sales and Distribution Policies .............................. 11

Software Author Note ......................................... 12

Extended Functions ........................................... 13

Library Versions ............................................. 19

QBDEMOC program .............................................. 19

This program is distributed as Shareware. All rights are reserved

except for distribution of the program, its documentation, and

any DEMO materials IN ITS ENTIRETY and un-modified. You may test

and use this program for 30 days, after which you must register

it, destroy it, or pass in along to a friend. If you would like

to keep HGCIBM, see the pricing within. This DOES NOT APPLY to

those who contributed to HGCIBM Version 1.11. Those individuals

are welcome to this upgrade without further remuneration. Please

accept it with my THANKS!!

Athena Digital's program and emulation algorithms were first

Copyrighted June 10, 1986. It is now the basis for the existence

of Athena Digital and represents months of effort and a large

personal investment. Any information presented in this document

or available in the associated programs is and shall remain the

Copyrighted Property of Athena Digital. Any attempt to use this

information to create a competitive product, whether the product

is distributed "free" or otherwise is in violation of copyright

laws.

Athena Digital

2351 College Station Road, Suite 567, Athens, Ga. 30605

2

DISCLAIMER

Athena Digital makes every effort to ensure that this

documentation is accurate. However, because we are constantly

striving to improve our products, we are unable to guarantee the

accuracy of the contents of this document after the date of

publication and we disclaim liability for any changes, errors or

omissions. We have also made every effort to warn you of possible

damage to your equipment from careless use of other applications

with ours. If you do somehow damage your display or anything

else, Athena Digital ASSUMES ABSOLUTELY NO RESPONSIBILITY FOR IT.

If it will make you more confident, no such claims have ever

been made against us .. but we have to protect ourselves.

ACKNOWLEDGMENTS

Athena Digital would like to thank IBM for creating the Personal

Computer and Hercules Computer Technology for making PC graphics

as pleasurable as PC TTL text.

TRADEMARKS MENTIONED

Hercules and RamFont are trademarks of Hercules Computer

Technology.

IBM is a trademark of International Business Machines.

Microsoft, QuickBASIC, CodeView and MS-DOS are trademarks of

Microsoft Corporation.

Athena Digital Video BIOS is a trademark of Athena Digital.

USING HGCIBM.COM

The purpose of this program is to provide owners of Hercules

Graphics Cards a means to run software written for the IBM Color

Graphics Adapter, and to introduce a commercial version of the

BIOS called Athena Digital Video BIOS which offers added

functionality.

PRECAUTIONS - MONOCHROME DISPLAY DAMAGE !!!

This program will not damage your display, but certain

application software certainly can. Programs which attempt to by-

pass the BIOS (Basic Input/Output System) to set the CRT

operating parameters will cause severe problems. Fortunately

since most software operates in standard modes, most software

developers permit BIOS to set up the controller for them even if

they later write to screen ram directly.

(cont'd)

3

HGCIBM will intercept the BIOS calls for all standard modes and

program the controller correctly. Writing directly to screen ram

is permitted and the emulator will still work properly. When

trying an un-tested (with HGCIBM) program for the first time,

watch your display carefully for signs of gross distortion or

shrinking when the program sets graphics mode. IF THIS OCCURS,

RE-BOOT OR TURN OFF YOUR COMPUTER IMMEDIATELY. The display should

return to normal quickly when you re-boot.

HGCIBM INSTALLATION

To install the emulator, you must tell it what mode to start in,

and a little information about your hardware. There are three

"modes" of operation for HGCIBM as follows:

The /E[mulate] mode enables the emulator to respond to BIOS calls

to set standard CGA modes and provides maximum compatibility with

programs expecting to use the Color Graphics Adapter, but

displays the information on your Hercules adapter. When the

emulator is in this mode, the text buffer is at segment B800 and

the equipment flag is set to indicate that a CGA card is present.

Alas, you can't fool all the programs all the time. Some of them

read the equipment flag and then try to manipulate the crt

controller directly in order to provide faster screen output.

Naturally they try to do this at the CGA port addresses (which

don't exist) and will hang the system (This problem is overcome

with Athena Digital Video BIOS). A re-boot will cure the problem

and you should then remember to either un-install the emulator

before running these programs or simply place it in /M(ono) mode

or /C[GA] mode (if you have a physical CGA) as described below.

The /M[ono] mode effectively disables HGCIBM without un-installing

it. The monochrome card is selected and Video BIOS control

returns to the computer's ROM (Read Only Memory).

The /C[ga] mode also disables HGCIBM and returns control to ROM

except the physical Color Graphics Adapter is selected (if it

exists).

There are also 3 installation switches available to warn HGCIBM

about special hardware. During initialization, the program tries

to determine if a Color Graphics Adapter is present. If it is,

the program will not install unless you assure it that the Color

Graphics Adapter in your system is compatible with Hercules

Graphics cards. If yours is or you have a Leading Edge computer,

then you should use the following switches:

The /H[ercules Compatible] switch instructs HGCIBM to install and

allow the emulation mode even though a Color Graphics Adapter is

present. You may use this switch on installation if you have a

Hercules compatible CGA card in your system. If a CGA card is

present, HGCIBM will abort installation unless the /H switch is

used.

(cont'd)

4

Also note that some software will output to the CGA registers

(3D4, 3D5, 3D8, etc.) to adjust video parameters. This can wreak

havoc on the real CGA adapter while in Emulation mode. If the CGA

is not physically present, this output causes no problems and is

ignored by Hercules adapters.

The /L[eading Edge] switch instructs HGCIBM to install if the

Leading Edge Model D is connected to a TTL display.

The /U[n-install] switch instructs the emulator to reset all

vectors and remove itself from memory, freeing the space for

other programs. You may un-install the emulator provided that

other programs have not altered vectors 8h and 10h, but memory

may become segmented if you have loaded another resident program

after HGCIBM. The system will be left in the same condition it

was in before the installation. (i.e. monochrome display).

USING THE COMMAND LINE

Simply type the command HGCIBM with a mode switch to install or

change to that mode. If you have special hardware, ADD the

hardware switch to the mode switch on the same command line as

shown in the following examples:

HGCIBM /M      - installs in (or changes to) MONO mode (emulation

disabled)

HGCIBM /C      - installs in (or changes to) CGA mode (emulation

disabled)

HGCIBM /E      - installs in (or changes to) Emulation mode.

HGCIBM /U      - un-installs the emulator

HGCIBM /M/H    - informs HGCIBM that a Hercules compatible CGA is

physically present.

HGCIBM /M/L    - informs HGCIBM that you have a Leading Edge

Model D computer.

These commands may be issued from a batch file if desired.

Hardware switches (/H or /L) are needed only on installation.

To change modes after installation, just enter the command as

above. HGCIBM always checks to see if it is already installed and

will process the command correctly.

HGCTEST PROGRAM

This program is provided for you to test the operation of HGCIBM

with your computer configuration. Before running HGCTEST, Load

the emulator with the /E[mulate] switch. HGCTEST.BAS runs under

BASICA. Just enter BASICA HGCTEST to run it (assuming BASICA is

on the default disk drive with HGCTEST.BAS). The program is a

simple graphics demo and is self explanatory. If you have a basic

compiler, the demo program can be compiled to test the compiler.

5

SALES AND DISTRIBUTION POLICIES - HGCIBM.COM

There is no limitation on distribution of HGCIBM.COM as long as

no remuneration is accepted in connection with it's distribution,

and as long as it is distributed un-modified, in it's entirety

(including any DEMO materials), and BY ITSELF. IT MUST NOT BE

DISTRIBUTED WITH your own application program or hardware, but it

is OK to mention that your software/hardware will/wont run with

it and our name and telephone number (so he can obtain a copy to

run your program). You are permitted a 30 day trial period to

decide whether or not you wish to keep HGCIBM or purchase Athena

BIOS after which you must either register it with Athena Digital,

destroy it, or pass it along to someone else. The registration

fee for HGCIBM.COM is $10.00 in check or money order (see

REGISTER file in this package).

OEMS and Dealers are encouraged to support Athena BIOS with their

products. HGCIBM is an extremely limited version of Athena

Digital's software. Those interested should contact Athena

Digital for details.

USER COMMENTS

Let us know what you think. Any criticism is better than none and

will help to steer us along a path which is in the majority's

best interest. Your suggestions need not be limited to graphics.

Any comment regarding Athena Digital programs or documentation is

welcome.

Thank you for your support.

HISTORY

In June of 1986, Athena Digital released a simple "Freeware"

emulator for Hercules cards which supported only the 200 by 640

Color Graphics Card mode (HGCIBM.COM Version 1.11). To our

knowledge, this was the first such program publicly released. At

first glance there would appear to be no physical way to

effectively display CGA graphics on a ttl card. It not only could

be done, but done in such a way that resulting display quality is

actually better than the real CGA though it is in shades of one

color. Support came in from users as far as Israel and eventually

from CGA software developers. Though it represented no serious

income, This support inspired Athena Digital to create HGCIBM.COM

Version 2.00 which offers emulation support of all of the CGA

modes and to start the development of Athena's own Video BIOS to

support not only emulation, but to provide an upgradable and

expandable Video BIOS which enhances any display system.

6

FUTURE

Version 2.02 provides only a SUBSET of the Athena Digital support

functions. It is a functional emulator only, and does not offer

support for Microsoft Mouse, Extended Video BIOS calls, RamFont,

or EGA. Due to restrictions imposed by the "resident" utility

implementation, the commercial release of Athena's BIOS

(ATHENA.SYS Version 3.00) is in the form of an MS-DOS Device

Driver. Installed in this way, the program becomes as much a part

of the operating system as is the ROM Video BIOS, except that it

can be easily upgraded and enhanced. Future enhancements will be

limited to Athena BIOS, thus obsoleting HGCIBM.COM except as a

simple emulator. If bugs are found in HGCIBM, they will be

corrected. For more information about the commercial products,

read the section called Athena Digital Video BIOS in this

document.

HGCIBM CHANGE AND CORRECTION NOTES

o    Version 1.11 would sometimes crash with programs that

intercepted the timer interrupt while in Graphics, and

then changed to text.

o    If interrupts were redirected, Version 1 could not find

itself for a mode change and would install a second

copy. Now it can find and control itself unless it is

by-passed altogether.

o    Version 2.00 (beta test version) would not install on a

Leading Edge due to differences in the way the Leading

Edge video buffers are implemented. This was corrected

with the /L[eading Edge] switch.

o    Version 2.01 failed to detect the EGA and would treat

it as a CGA. Now HGCIBM will abort installation as it

is supposed to.

ENHANCEMENTS

o    Version 2.02 contains a SUBSET of Athena Digital's own

Video BIOS and will support a wider range of

compatibles as well as the AT. This also provided a

means for improved screen behavior in color text mode,

and faster screen operations if BIOS is used.

o    All standard video modes are now supported. The 320 x

200 mode supports two intensities(colors). Color 1 and

color 2 will appear identical, where color 3 will be

high intensity. Forty column text is supported but will

be displayed on the left half of the screen only at the

same resolution as eighty column text. All four pages

of 80 column text (or 8 pages of 40 column text) are

available on CERTAIN Hercules cards. Some cards will

display only one page (Athena Video BIOS displays all

pages on all Hercules Adapters and clones).

7

TECHNICALITIES

HGCIBM installs itself between two BIOS (Basic Input/Output

System) interrupts. The two interrupts used in this way are hex

10 (BIOS video) and hex 8 (timer). The later is used only while

in emulation mode. HGCIBM will fail to function properly if it is

not given control during the above interrupts. Other software

using interrupt 8 will be called on a priority basis, preempting

HGCIBM if necessary. HGCIBM version 2.02 no longer passes

interrupt 10h (while in emulation mode) since it was learned that

compatible ROM BIOS was not always entirely compatible. If you

have other software using interrupt 10h, try loading it after

HGCIBM. You can still control HGCIBM after the interrupt is

changed but you will not be allowed to un-install it. HGCIBM need

be loaded only once each time the computer is started since it

attaches itself to BIOS and stays in memory until re-boot or it

is commanded to detach. The memory usage has increased to about

5k now due to bypassing of ROM video BIOS in emulation mode, but

does provide much improved color text emulation and should be

compatible with a wider variety of machines.

8

Athena Video BIOS Version 3.00

Copyright (C) Athena Digital 1986,1987

Preliminary Specifications - Subject to change without notice.

PURPOSE

For some unknown reason, hardware and software vendors have not

seen fit to establish adequate specifications and graphics

support for the widely used Hercules Graphics standard. The most

peculiar aspect of this is the fact that the Hercules standard

was the third available for the IBM personal computer and the

first which offered truly high graphics resolution as well as the

first to support graphics on the ttl monochrome display. Today,

this standard is almost universally accepted as the display

adapter of choice for ttl monochrome PC systems and its display

quality is yet challenged only by the EGA standard which

unfortunately requires an expensive color monitor to exploit it's

capabilities.

Athena Digital will attempt to remedy the problem by making it as

simple as possible for vendors to exploit the capabilities of the

Hercules card, CGA, EGA and others. Athena BIOS now fully

supports the Hercules monochrome card and/or Color Graphics Card,

the EGA (and eventually others) with fast support for standard

modes, CGA emulation, and several new modes.

One of the primary enhancements provided by ATHENA BIOS is

Emulation of the Color Graphics Adapter on the Hercules Graphics

Card. This allows Hercules owners to run the majority of graphics

programs which do not already support the Hercules adapter. The

list includes programs like BASICA, QuickBASIC, CodeView and the

majority of all others.

This eliminates the need for a Color monitor and graphics card in

many cases with obvious price advantages.

Athena also provides BIOS support for the higher resolution

Hercules 350 vertical by 720 horizontal modes, and a new 350

vertical by 360 mode with three intensities and 4 logical pixel

colors. ALL of the Standard BIOS call support for these new modes

including character generation, scrolling, and adapter switching

which greatly simplifies direct support of Hercules high

resolution modes by software vendors.

ATHENA FEATURES

o    Available in Programmer's Libraries for linking with high or

low level languages.

o    Supports 6 new Hercules only modes, which are as

readily available to applications developers as are the

standard modes.

9

o    Provides extended equipment, system capability information

for applications.

o    Supports Color Graphics Adapter Programs on the Hercules

Graphics Card.

o    Fully compatible with Microsoft Mouse or Microsoft Mouse

software look-alikes.

o    Includes a "Buss Terminator" which eliminates ALMOST all

of the application software incompatibility with Emulation

modes. This allows Microsoft's CodeView and QuickBasic

editors to function normally in Emulation mode.

o    BIOS Character generation and scrolling in Emulation Modes

is accomplished without ghosting and no appreciable loss in

speed (it is usually faster than the CGA through ROM BIOS).

o    All 4 pages of 80 column CGA text (or 8 pages of 40 column

text) are available on ALL Hercules cards or compatibles.

o    Fully supports Hercules - Enhanced Graphics (color monitor)

combinations.

o    Fully Compatible with the new Hercules Plus card in 4K

RamFont or Rom text mode.

o    Supports new "Context Switching" between adapters and

certain modes.

o    Supports faster video operation in ALL modes than Standard

ROM BIOS.

o    Simple replacement for "buggy compatible" BIOS.

o    Installs as a device driver for MS-DOS version 2.00 or later

o    Adds PC-AT BIOS print string functions to the PC

Once installed, Athena BIOS replaces the standard ROM video BIOS.

Software installation is a matter of copying ATHENA.SYS to your

boot disk, and inserting a one line command in your MS-DOS

config.sys file. This MUST be the FIRST "DEVICE =" command in the

file following the MOUSE and has the following basic form:

DEVICE = MOUSE.SYS

DEVICE = ATHENA.SYS

Athena BIOS is compatible with a variety of MS-DOS

computers, including the PC, PC-XT, PC-AT, and Leading Edge, and

most any other hardware compatible. Our aim is to work with as

many different computers and display adapters as possible.

10

Version 3.00 Video Bios does not replace EGA BIOS, but is

compatible with it and controls it. If a non-ega adapter is

active, Athena BIOS controls the active adapter. While the EGA is

the active display, EGA BIOS controls the EGA adapter. Athena

BIOS supports the Hercules Graphics Card in FULL modes WITH the

EGA (if it is connected to a color display). The BIOS will

continue to add features supporting as many of the popular

adapters as possible and enhancing them wherever practical in

order to maintain compatibility for your programs.

CONTROLLING ATHENA BIOS AFTER INSTALLATION

With ATHENA.SYS, you are provided a utility which allows you

keyboard control over Athena's variety of modes. This utility

operates as any other external DOS command. Simply place it in

your program directory and use it with switches. If you forget

the switches, typing ATHENA will give you a help screen.

ATHENA /M[ono]      - Places BIOS in MONO or EGA MONO mode

(emulation disabled)

ATHENA /C[ga]       - Places BIOS in CGA or EGA Color mode

(emulation disabled)

ATHENA /E[mulate]   - Places BIOS in CGA EMULATION mode.

ATHENA /S[tatus]    - Gives a report of Athena's status.

ATHENA /Q[uiet]     - ADDING this switch to others to suppress

messages.

ATHENA /D[isable]   - Disables the BIOS and returns control to

ROM

ATHENA /R[eset]     - Resets and enables the BIOS.

These commands may be issued from a batch file if desired.

SALES AND DISTRIBUTION POLICIES - ATHENA.SYS

For the purchase price of $34.95, you will receive the program on

a 5.25" IBM format diskette with documentation and Buss

Terminator, and be notified of upgrades at low cost as they

become available.

Dealers, OEMS, and those desiring Site Licenses are welcome and

encouraged to contact Athena Digital about liberal discounts.

11

SOFTWARE AUTHORS

Using Athena Video BIOS Libraries, you can easily upgrade your

application to support Hercules Adapters. There are several

options available to you with the library:

o    You can link Athena Digital Video BIOS with your

application. This allows your program to use all Athena

functions whether the Athena device driver is present

or not.

o    You can link with the interface module only. This

allows you to detect and and use Athena BIOS if the

Device Driver is present.

o    You can link a PARTIAL BIOS with your application. This

allows you to use the CGA Emulation modes whether the

Device Driver is present or not, but defeats the usage

of Hercules Extended modes.

See the QBDEMO program included for many more details.

SUPPORTED HARDWARE

o    IBM PC /XT /AT and most hardware compatibles which have an

expansion buss. (Expansion buss not necessary for

Libraries or for systems which will not need CGA Emulation).

o    Microsoft Mouse (device drivers) or software compatibles

which provide Microsoft's defined function calls.

o    The following Video hardware combinations have been tested

to date:

Hercules Graphics Adapter, all versions including Plus. (MGA)

IBM Monochrome Text Adapter. (MTA)

IBM Color Graphics Adapter .. Hercules Color Card. (CGA)

IBM Enhanced Graphics Adapter (EGA)

Monochrome EGA with CGA

Color EGA or CGA with MTA or MGA.

12

Athena Digital Video BIOS FUNCTION CALLS

This information is provided for technical users, and

presumes familiarity with Standard BIOS function calls. Presented

here is a subset of available services. A complete listing is

available in the Athena Video BIOS Programmers Reference Manual,

provided with each Programmer's Library.

THIS DOCUMENT CONTAINS PRELIMINARY INFORMATION ABOUT ATHENA VIDEO

BIOS CONTEXT CALLS. THE PURPOSE OF THIS IS TO SHOW THE INTENT OF

THE CALLS. FOR PERMANENT AND ACCURATE INFORMATION, RELY ON THE

PROGRAMMER'S REFERENCE DATA PROVIDED WITH LIBRARY PACKAGES.

; ****

(AH)=B0h     SetContext - (AL) contains context value

These functions first save the current BIOS variables in the

current Context Buffer, then change the current VideoState

"emulation active" flag according to the table below and take one

of three actions depending on the condition of the target

screen's Context save buffer. There are 3 save buffers in the

initial Athena BIOS release:

o    Hercules / Monochrome / Mono EGA adapter page at 0B000h

o    Hercules adapter page at 0B800h

o    Color Graphics Adapter / Color EGA Adapter at page 0B800h

Each save buffer has room for:

o    The current crt_context

o    The current crt_mode

o    The current crt_columns

o    The current crt_len

o    The current crt_start

o    The current cursor_posn[8]

o    The current cursor_mode

o    The current active_page

o    The current addr_6845

o    The current crt_mode_set

o    The current crt_palette

o    The current rows

13

Call B0h (cont'd)

The three possible actions through call B0h are:

o    If register AL bit 7 = 1 (or'd with 80h), the target

screen Context buffer is initialized and the screen

is cleared.

Otherwise, the screen is not cleared and the Context

buffer is treated as follows:

o    If the target context's save buffer has not been

overwritten since it was saved, it is restored intact.

o    If the target context's save buffer has been

overwritten, it is initialized to default values of the

Target Context before restoration.

Context switches do not "blink" the screen unless it is necessary

to re-program the 6845 controller to make the switch.

This is a powerful function. It enables programmers to switch

rapidly back and forth between graphics and text, or between

adapters without having to concern themselves about re-

initializing or restoring the screen(s). It also permits rapid

switching between 40 or 80 column graphics text modes without

loss of screen data, allowing mixed use of the provided (or user

provided) character fonts in Hercules Graphics modes. When

switching between 40 and 80 column text, the Context buffer is

initialized, so it is necessary to locate the cursor wherever you

want it in the new text map.

Context switches also affect the way Athena interprets calls to

the standard BIOS function 0 as shown below. These calls cannot

be used to overide the hardware setup determined by Athens during

initialization. Calls for invalid context switches are ignored

and the call returns with the carry flag set:

14

Athena BIOS function call B0h (continued):

CONTEXT VALUES AND EFFECTS ON CGA EMULATION FLAG

;

TARGET MODE         EMULATION           TARGET SCREEN

Standard Modes (Contexts)

(AL)=0              deactivated         CGA/EGA 40x25   alpha

(AL)=1              deactivated         CGA/EGA 40x25   alpha

(AL)=2              deactivated         CGA/EGA 80x25   alpha

(AL)=3              deactivated         CGA/EGA 80x25   alpha

(AL)=4              deactivated         CGA/EGA 200x320 graph

(AL)=5              deactivated         CGA/EGA 200x320 graph

(AL)=6              deactivated         CGA/EGA 200x640 graph

(AL)=7              deactivated         MONO/EGA 80x25  alpha

(AL)=13             deactivated         EGA     320x200 graph

(AL)=14             deactivated         EGA     640x200 graph

(AL)=15             deactivated         EGA     640x350 graph

(AL)=16             deactivated         EGA     640x350 graph

Call B0h (cont'd)

Hercules only modes

CGA Emulation Contexts: (correspond to Standard modes 0 - 6)

(AL)=20

.                activated           HERC page at B800*

.

(AL)=26

Hercules Logical Text Context 27:

(supports 8 pages of 80 column text on ALL Hercules Cards)

(AL)=27             deactivated         HERC 80x25 alpha

Hercules Graphics Contexts 28,29,30,31:

(AL)=28             deactivated         HERC 350x720 at B000

(AL)=29             deactivated         HERC 350x720 at B800

(AL)=30             deactivated         HERC 350x360 at B000*

(AL)=31             deactivated         HERC 350x360 at B800*

*modes 24,25, 30 and 31 support low and high intensity

15

Certain Context changes are not possible without erasure of the

contents of a previously buffered Context. To gain an

understanding of which context switches can be performed without

erasing another, study the above table. Context switches between

adapters never cause erasure. Context switches between differing

Hercules pages are likewise retained, except for emulation text

Contexts 20,21,22,23, and Logical text Contexts 27.

Emulation and Logical text modes use BOTH Hercules pages, thereby

erasing any previously saved Hercules Context .

; ****

(AH)=D0h      SetAdapter - (AL) contains new Context Value as

above. The new Context is set. This call supersedes

the original BIOS call 0, which re-maps Modes

to Contexts in order to effect transparent

Emulation.

This call will switch to, reset and clear the

target adapter, unless bit 7 is set, which inhibits

clearing of the target regen buffer. The Target CRT

Controller is re-initialized only if necessary.

; ****

(AH)=0        SetMode - al contains then IBM defined Mode value.

This call will not switch adapters unless the

equipment flag is changed prior to the call. The mode

value is re-mapped to an Athena Context value

depending on whether the BIOS is Emulating or not.

This call is designed to support applications which

are unaware of Athena BIOS. It's behavior is

identical to the standard BIOS.

Function 0 is superseded in Athena BIOS by function

D0, which accepts Context Values rather than Mode

values.

Function 0 supports the IBM defined flag bit 7

to inhibit regen buffer erasure.

16

(AH)=BFh      AthenaState - Return the current driver state.

OUTPUT:

(AL) = Current Context (as in call B0h)

(AH) = Character Columns on screen

(BH) = Current Active Display Page

(BL) = Reserved - destroyed

(DX) = Installed options, switch states

D0 = 1 if mono or Hercules card is installed

D1 = 1 if cga card is installed

D2 = 1 if an ega card is installed

D3 = 1 if Hercules modes/cga emulation are allowed

D4 = 1 if emulation modes are activated

D5 = reserved.

D6 = 1 if it is necessary to wait for

horizontal retrace before CGA text

screen updates. This bit is reset by

default when ATHENA initializes. If the

/W(ait) option switch is used on the

installation command line, this bit

will be set to cause ATHENA to wait for

retrace when updating CGA text. This

switch has no effect on Hercules or

Mono modes.

Application software should never

try to wait in Hercules modes since the

CGA registers may not even exist and

the Hercules standard does not require

it.

D7 - D15 = reserved.

(CX) = ATHENA/HGCIBM version number

(CH) has major version

(CL) has minor version

(BP) = Reserved - destroyed

(DI) = Reserved - destroyed

(continued)

17

Function BFh (continued)

(SI) = Extended Flags.

D0 = 1 if the EGA is connected to an Enhanced

Color Display.

D1 = 1 if the EGA is connected to a Monochrome

Display.

D2 = 1 if the system is in CGA Context 2 or 3,

AND the system is directed to wait for

CRT retrace to update the screen.

D3 = 1 if the EGA is currently the active

adapter.

D5 = 1 if the EGA is currently disabled.

(the system is in a Context which

requires ega ram to be disabled). While

disabled, function 12h will be ignored.

D6,D5 = Ega installed memory.

D7 = 1 if the EGA switches indicate it is the

Primary System Adapter.

D8 = 1 if there is a Microsoft Compatible

Mouse and it is currently emulating a

light pen.

D9 - D14 Reserved.

D15 = 1 if there is a Microsoft Compatible

Mouse and it reports -1 to function 0.

* set cx to -1 before calling function BFh, if the major version is

greater than or equal to 3, the response is from an Athena Video

BIOS device driver. If the major version is greater than or equal

to 23, the response is from an Athena Library BIOS. There will be

no Library response unless you linked with it. A response less

than 3 is from HGCIBM.COM, which will not occur if you are linked

with a library. If cx returns -1, there is no Athena BIOS.

HGCIBM.COM will not be upgraded past major version 2, and

behavior in general is somewhat different than from the BIOS. If

HGCIBM is detected, either ignore it or use only the emulation or

CGA modes. All future enhancements will be limited to Athena

Drivers or Libraries.

The above information is only a smattering of available services

and enhancements. Complete technical documentation is provided

with Programmer's Libraries and we don't recommend you try to

communicate with the BIOS without obtaining the technical

information.

18

ATHENA LIBRARY VERSIONS

These are introduced in the accompanying QBDEMOC program. The

DEMO is subject to continuous updates and revisions as the

BIOS continues. It is entirely possible that new features and

capabilities have been added since this file was released. The

DEMO is a short first-hand introduction to Athena Device Drivers

and Athena Language Libraries for Programmers. If you find you

are seriously interested in Athena Digital Video BIOS, we will

send you the latest demo materials on request. There are 2 files

required to run the demo:

QBDEMOC.EXE    ; QuickBasic library DEMO - BCOM20 program module

FEATURES.PRX   ; DEMO text

o    To run the demo, place these two files in your default

directory and type "QBDEMOC".

o    If you have an older style Color Graphics Adapter which

"snows" when text is printed, then start the demo as

"QBDEMOC/W".

o    It makes no difference whether HGCIBM is installed or not to

run the demo, since Athena Video BIOS is linked with the

demo and will be used in either case.

o    DEMO Version 1.01 will now run on the EGA. If the EGA

is connected to a Monochrome Display, the Athena Logo will

display incorrectly. This appears to be a QuickBasic 2.00

bug but we have not yet heard from Microsoft about it so it

may just be our QuickBASIC programing. It definitely has

NOTHING to do with the BIOS.

In case the demo is missing, here are a scant few of the

highlights of Athena Video BIOS Libraries.

o    Allows graphic (or text) programs written using standard

video BIOS for all except regen access to run unmodified on

a Hercules Graphics Card. It is only necessary to initialize

the BIOS on entry, and detach it on exit. An example of one

such program would be anything compiled under Microsoft's

QuickBasic and using QuickBasic's screen and graphics

functions such as QBDEMOC.EXE.

o    Provides all of the functions that are described under

Athena Video BIOS above. Thus it offers immediate Hercules

support for existing applications, and a natural upward path

to extended Hercules support.

19

o    Cooperates with (or without) a resident Athena Device

Driver. If your Library is a later version, it automatically

coordinates a temporary "takeover" from the resident while

your program runs. Control returns to the resident when your

program exits. This assures that your program will always

use the latest BIOS available, whether its the one you

linked with or not.

o    Libraries will probably be released for QuickBASIC, C, and

Macro Assembler by the time anyone could read this. Others

will be developed depending on customer interest. Write or

call us about your favorite programing language if you

would like to see an Athena library to link with it. The

library core is intrinsically compatible with any compiler

supporting external assembly language procedure calls and a

means of linking them with the compiled program code.

More details are available on request.

20

############################################################################################