(*****************************************************************************

COPYRIGHT 1986, 1987 and 1988 by Gene Harris.

First Release : March 10, 1986 by Gene Harris

Second Rel.   : December 19, 1988 by Gene Harris

Third Release : January 3, 1988 by Gene Harris

This program is designed to allow your PC, XT, AT or jr to have access to

a full support RS-232 set of routines written in Turbo Pascal 3.01  This

program is written completely in Turbo Pascal.  I suspect it will only

handle baud rates of about 9600.  It has only been tested to 2400 baud.

The routine has been tested on an IBM XT equipped with an NEC V20,

an XT clone running at six and eight MHz and an 8 MHz PC AT clone

successfully.  The OPENCOM routine must be changed to run with a jr.

Appropriate changes and intructions for implementing these changes is

included as comments.

This code is copyrighted by M. Eugene Harris, Jr.  It is released to the

public domain, and may not be used for commercial gain.  The program is

user supported, and updates may be obtained from Remark BBS, in Oklahoma

City 405 728-2463.

A new version of the program will be released in January, which will sup-

port 16 bit and 32 bit CRC's.

Routines included in this code:

*****************************************************************************

InitComPort (iPort, InBufsize, OutBufsize) ;

iPort : 1 or 2 only, no other serial ports supported.

InBufSize   : Maximum Receive Buffer size for channel.

OutBufsize  : Maximum Send Buffer size for channel.

Purpose of the routine:  This routine sets up the interrupt vectors

as well as initializing the communications

buffers for RS232 i/o.  The maximum size of

InBufsize and OutBufsize is MaxInt.  THIS MUST

BE THE FIRST ROUTINE CALLED PRIOR TO USING THE

COM PORTS!

*****************************************************************************

SetCom (ibaudrate,idatabits,istopbits:integer;

iparity:char) ;

ibaudrate : 110, 150, 300, 600, 1200, 2400, 4800, 9600 baud supported.

idatabits : 5, 6, 7, 8 data bits supported.

istopbits : 1 or 2 stopbits supported.

iparity : 'E'ven, 'O'dd, 'N'one, 'M'ark, 'S'pace are supported.

Purpose of the routine: User supplied communication port values are

converted to the proper set values used by

the asynch routines.  Sets are used internally

to avoid var conflicts and improve speed.

The routine returns an OpenError = TRUE if

the specified COM port does not exist.  Therefore,

the user code needs to check OpenError before

preceding to the next logical phase.

*****************************************************************************

Break - No Parameters.

Purpose : drop DTR to effectively hang up the modem.

*****************************************************************************

Purge - No Parameters.

Purpose : Purge the circular queues.

Reset the 8250 interrupt control register.

*****************************************************************************

RS232ChrAvailable : Boolean ;

Purpose : Returns true if there are characters in the RS232 input queue

otherwise returns false.

****************************************************************************

Function ReadChar : char ;

Purpose : Returns the next character in the RS232 input buffer.  If no

characters are present, the routine returns a null byte.  The

RS232ChrAvailable routine should ALWAYS be checked before

trying to read a character, otherwise you will not be able

to tell if the null returned is correct.  The routine is

designed to be used in the following format:

begin

auxoutptr := ofs(readchar) ; { Install device driver }

....

program body

....

if RS232ChrAvailable then read( aux, byte or char) ;

....

rest of program

***************************************************************************

Procedure WriteChar ;

Purpose : Writes the next character into the RS232 character buffer.

The Asynch_Interrupt routine will output the characters in

the queue as THRE interrupts are generated by the 8250.

This routine is intended to be a device driver for the logical

devices USR or AUX.

EXAMPLE :

begin

auxinptr := ofs(WriteChar) ; { Install Device Driver. }

. . . .

. . . .

Write (aux, variable) ; { Put in RS232 output queue }

***************************************************************************

Procedure WriteBlock (var Block; size: integer) ;

Purpose : Writes the array of bytes contained in Block to the output

ring buffer.  The THRE interrupt is then inabled, and proc-

essing continues without intefering with the calling program.

EXAMPLE :

var

Block_Buffer : array [1..512] of byte ;

begin

....

WriteBlock (Block_Buffer, 512) ;

***************************************************************************

Procedure WriteString (var String_Buffer: String_Type) ;

Purpose : Writes the array of bytes contained in String to the output

ring buffer.  The THRE interrupt is then inabled, and proc-

essing continues without intefering with the calling program.

EXAMPLE :

var

String_Buffer :  String[255];

begin

....

WriteString (String_Buffer) ;

***************************************************************************

Procedure UseCTS ;

Purpose : Enables the use of the CTS signal from the modem.  This is

primarily of use with high speed transfers at 9600 BAUD or

greater.  When CTS is enabled, nothing is transmitted to the

modem until the CTS line registers high in the modem status

register.

***************************************************************************

Caveats : The Asynch_interrupt routine is written in pascal.  It is fast

enough to handle 1200 baud, and may handle 2400 baud.  You may

not get 4800 or 9600 baud support with this routine.

The buffer input routine always checks that the variable

LSRstat is zero.  The LSRstat (Line Status Register) is utilized

to show buffer overflow on input.  Therefore, your program

must monitor LSRstat to determine if a buffer overflow has

occurred.  Bit 1 will be set if buffer overflow has occured.

EXAMPLE :          if LSRstat <> 0 then begin

writeln('Line Status Error: ',LSRstat) ;

. . . .

Handle LSR error condition. . .

. . . .

{ Reset LSRstat so that transmission can

begin again. Turn off bit 1. }

LSRstat := LSRstat and $FD ;

end ;

*)