eXtended Memory Specification (XMS), ver 2.0

July 19, 1988

Copyright (c) 1988, Microsoft Corporation, Lotus Development

Corporation, Intel Corporation, and AST Research, Inc.

Microsoft Corporation

Box 97017

16011 NE 36th Way

Redmond, WA 98073

LOTUS (r)

INTEL (r)

MICROSOFT (r)

AST (r) Research

This specification was jointly developed by Microsoft Corporation,

Lotus Development Corporation, Intel Corporation,and AST Research,

Inc. Although it has been released into the public domain and is not

confidential or proprietary, the specification is still the copyright

and property of Microsoft Corporation, Lotus Development Corporation,

Intel Corporation, and AST Research, Inc.

Disclaimer of Warranty

MICROSOFT CORPORATION, LOTUS DEVELOPMENT CORPORATION, INTEL

CORPORATION, AND AST RESEARCH, INC., EXCLUDE ANY AND ALL IMPLIED

WARRANTIES, INCLUDING WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

PARTICULAR PURPOSE. NEITHER MICROSOFT NOR LOTUS NOR INTEL NOR AST

RESEARCH MAKE ANY WARRANTY OF REPRESENTATION, EITHER EXPRESS OR

IMPLIED, WITH RESPECT TO THIS SPECIFICATION, ITS QUALITY,

PERFORMANCE, MERCHANTABILITY, OR FITNESS FOR A PARTICULAR PURPOSE.

NEITHER MICROSOFT NOR LOTUS NOR INTEL NOR AST RESEARCH SHALL HAVE ANY

LIABILITY FOR SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES ARISING

OUT OF OR RESULTING FROM THE USE OR MODIFICATION OF THIS

SPECIFICATION.

This specification uses the following trademarks:

Intel is a registered trademark of Intel Corporation, Microsoft is a

registered trademark of Microsoft Corporation, Lotus is a registered

trademark of Lotus Development Corporation, and AST is a registered

trademark of AST Research, Inc.

Extended Memory Specification

=============================

The purpose of this document is to define the Extended Memory Specifica-

tion (XMS) version 2.00 for MS-DOS.  XMS allows DOS programs to utilize

additional memory found in Intel's 80286 and 80386 based machines in

a consistent, machine independent manner.  With some restrictions, XMS adds

almost 64K to the 640K which DOS programs can access directly.	Depending on

available hardware, XMS may provide even more memory to DOS programs.  XMS

also provides DOS programs with a standard method of storing data in extended

memory.

DEFINITIONS:

------------

Extended

Memory      -   Memory in 80286 and 80386 based machines which is located

above the 1MB address boundary.

High Memory

Area (HMA)  -   The first 64K of extended memory.  The High Memory

Area is unique because code can be executed in it while

in real mode.  The HMA officially starts at FFFF:10h

and ends at FFFF:FFFFh making it 64K-16 bytes in length.

Upper Memory

Blocks (UMBs)-  Blocks of memory available on some 80x86 based machines

which are located between DOS's 640K limit and the

1MB address boundary.  The number, size, and location

of these blocks vary widely depending upon the types

of hardware adapter cards installed in the machine.

Extended Memory

Blocks (EMBs)-  Blocks of extended memory located above the HMA which

can only be used for data storage.

A20 Line    -   The 21st address line of 80x86 CPUs.  Enabling the A20

line allows access to the HMA.

XMM         -   An Extended Memory Manager.  A DOS device driver which

implements XMS.  XMMs are machine specific but allow

programs to use extended memory in a machine-independent

manner.

HIMEM.SYS   -   The Extended Memory Manager currently being distributed

by Microsoft.

Helpful Diagram:

|                                                       |   Top of Memory

|                                                       |

|                                                       |

|                        /\                             |

|                       /||\                            |

|                        ||                             |

|                        ||                             |

|.......................................................|

|                                                       |

|                                                       |

|             Possible Extended Memory Block            |

|                                                       |

|                                                       |

|.......................................................|

|                        ||                             |

|                        ||                             |

|                       \||/                            |

|                        \/                             |

|                                                       |

|                                                       |

|       Other EMBs could exist above 1088K (1MB+64K)    |

|                                                       |

|                                                       |

|-------------------------------------------------------|   1088K

|                                                       |

|                                                       |

|                The High Memory Area                   |

|                                                       |

|                                                       |

|=======================================================|   1024K or 1MB

|                                                       |

|                        /\                             |

|                       /||\                            |

|                        ||                             |

|                        ||                             |

|.......................................................|

|                                                       |

|             Possible Upper Memory Block               |

|.......................................................|

|                        ||                             |

|                        ||                             |

|                       \||/                            |

|                        \/                             |

|                                                       |

|        Other UMBs could exist between 640K and 1MB    |

|                                                       |

|-------------------------------------------------------|   640K

|                                                       |

|                                                       |

|                                                       |

|             Conventional or DOS Memory                |

|                                                       |

|                                                       |

|                                                       |

|                                                       |

|                                                       |

+-------------------------------------------------------+   0K

DRIVER INSTALLATION:

--------------------

An XMS driver is installed by including a DEVICE= statement in the

machine's CONFIG.SYS file.  It must be installed prior to any other

devices or TSRs which use it.  An optional parameter after the driver's

name (suggested name "/HMAMIN=") indicates the minimum amount of space in

the HMA a program can use.  Programs which use less than the minimum will

not be placed in the HMA.  See "Prioritizing HMA Usage" below for more

information.  A second optional parameter (suggested name "/NUMHANDLES=")

allows users to specify the maximum number of extended memory blocks which

may be allocated at any time.

NOTE: XMS requires DOS 3.00 or above.

THE PROGRAMMING API:

--------------------

The XMS API Functions are accessed via the XMS driver's Control Function.

The address of the Control Function is determined via INT 2Fh.  First, a

program should determine if an XMS driver is installed.  Next, it should

retrieve the address of the driver's Control Function.  It can then use any

of the available XMS functions.  The functions are divided into several

groups:

1. Driver Information Functions (0h)

2. HMA Management Functions (1h-2h)

3. A20 Management Functions (3h-7h)

4. Extended Memory Management Functions (8h-Fh)

5. Upper Memory Management Functions (10h-11h)

DETERMINING IF AN XMS DRIVER IS INSTALLED:

------------------------------------------

The recommended way of determining if an XMS driver is installed is to

set AH=43h and AL=00h and then execute INT 2Fh.  If an XMS driver is available,

80h will be returned in AL.

Example:

; Is an XMS driver installed?

mov     ax,4300h

int     2Fh

cmp     al,80h

jne     NoXMSDriver

CALLING THE API FUNCTIONS:

--------------------------

Programs can execute INT 2Fh with AH=43h and AL=10h to obtain the address

of the driver's control function.  The address is returned in ES:BX.  This

function is called to access all of the XMS functions.  It should be called

with AH set to the number of the API function requested.  The API function

will put a success code of 0001h or 0000h in AX.  If the function succeeded

(AX=0001h), additional information may be passed back in BX and DX.  If the

function failed (AX=0000h), an error code may be returned in BL.  Valid

error codes have their high bit set.  Developers should keep in mind that

some of the XMS API functions may not be implemented by all drivers and will

return failure in all cases.

Example:

; Get the address of the driver's control function

mov     ax,4310h

int     2Fh

mov     word ptr [XMSControl],bx        ; XMSControl is a DWORD

mov     word ptr [XMSControl+2],es

; Get the XMS driver's version number

mov     ah,00h

call    [XMSControl]    ; Get XMS Version Number

NOTE: Programs should make sure that at least 256 bytes of stack space

is available before calling XMS API functions.

API FUNCTION DESCRIPTIONS:

--------------------------

The following XMS API functions are available:

0h)  Get XMS Version Number

1h)  Request High Memory Area

2h)  Release High Memory Area

3h)  Global Enable A20

4h)  Global Disable A20

5h)  Local Enable A20

6h)  Local Disable A20

7h)  Query A20

8h)  Query Free Extended Memory

9h)  Allocate Extended Memory Block

Ah)  Free Extended Memory Block

Bh)  Move Extended Memory Block

Ch)  Lock Extended Memory Block

Dh)  Unlock Extended Memory Block

Eh)  Get Handle Information

Fh)  Reallocate Extended Memory Block

10h)  Request Upper Memory Block

11h)  Release Upper Memory Block

Each is described below.

Get XMS Version Number (Function 00h):

--------------------------------------

ARGS:   AH = 00h

RETS:   AX = XMS version number

BX = Driver internal revision number

DX = 0001h if the HMA exists, 0000h otherwise

ERRS:   None

This function returns with AX equal to a 16-bit BCD number representing

the revision of the DOS Extended Memory Specification which the driver

implements (e.g. AX=0235h would mean that the driver implemented XMS version

2.35).	BX is set equal to the driver's internal revision number mainly for

debugging purposes.  DX indicates the existence of the HMA (not its

availability) and is intended mainly for installation programs.

NOTE: This document defines version 2.00 of the specification.

Request High Memory Area (Function 01h):

----------------------------------------

ARGS:   AH = 01h

If the caller is a TSR or device driver,

DX = Space needed in the HMA by the caller in bytes

If the caller is an application program,

DX = FFFFh

RETS:   AX = 0001h if the HMA is assigned to the caller, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = 90h if the HMA does not exist

BL = 91h if the HMA is already in use

BL = 92h if DX is less than the /HMAMIN= parameter

This function attempts to reserve the 64K-16 byte high memory area for

the caller.  If the HMA is currently unused, the caller's size parameter is

compared to the /HMAMIN= parameter on the driver's command line.  If the

value passed by the caller is greater than or equal to the amount specified

by the driver's parameter, the request succeeds.  This provides the ability

to ensure that programs which use the HMA efficiently have priority over

those which do not.

NOTE: See the sections "Prioritizing HMA Usage" and "High Memory Area

Restrictions" below for more information.

Release High Memory Area (Function 02h):

----------------------------------------

ARGS:   AH = 02h

RETS:   AX = 0001h if the HMA is successfully released, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = 90h if the HMA does not exist

BL = 93h if the HMA was not allocated

This function releases the high memory area and allows other programs to

use it.  Programs which allocate the HMA must release it before exiting.

When the HMA has been released, any code or data stored in it becomes invalid

and should not be accessed.

Global Enable A20 (Function 03h):

---------------------------------

ARGS:   AH = 03h

RETS:   AX = 0001h if the A20 line is enabled, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = 82h if an A20 error occurs

This function attempts to enable the A20 line.  It should only be used

by programs which have control of the HMA.  The A20 line should be turned

off via Function 04h (Global Disable A20) before a program releases control

of the system.

NOTE: On many machines, toggling the A20 line is a relatively slow

operation.

Global Disable A20 (Function 04h):

----------------------------------

ARGS:   AH = 04h

RETS:   AX = 0001h if the A20 line is disabled, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = 82h if an A20 error occurs

BL = 94h if the A20 line is still enabled

This function attempts to disable the A20 line.  It should only be used

by programs which have control of the HMA.  The A20 line should be disabled

before a program releases control of the system.

NOTE: On many machines, toggling the A20 line is a relatively slow

operation.

Local Enable A20 (Function 05h):

--------------------------------

ARGS:   AH = 05h

RETS:   AX = 0001h if the A20 line is enabled, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = 82h if an A20 error occurs

This function attempts to enable the A20 line.  It should only be used

by programs which need direct access to extended memory.  Programs which use

this function should call Function 06h (Local Disable A20) before releasing

control of the system.

NOTE: On many machines, toggling the A20 line is a relatively slow

operation.

Local Disable A20 (Function 06h):

---------------------------------

ARGS:   AH = 06h

RETS:   AX = 0001h if the function succeeds, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = 82h if an A20 error occurs

BL = 94h if the A20 line is still enabled

This function cancels a previous call to Function 05h (Local Enable

A20).  It should only be used by programs which need direct access to

extended memory.  Previous calls to Function 05h must be canceled before

releasing control of the system.

NOTE: On many machines, toggling the A20 line is a relatively slow

operation.

Query A20 (Function 07h):

-------------------------

ARGS:   AH = 07h

RETS:   AX = 0001h if the A20 line is physically enabled, 0000h otherwise

ERRS:   BL = 00h if the function succeeds

BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

This function checks to see if the A20 line is physically enabled.  It

does this in a hardware independent manner by seeing if "memory wrap" occurs.

Query Free Extended Memory (Function 08h):

------------------------------------------

ARGS:   AH = 08h

RETS:   AX = Size of the largest free extended memory block in K-bytes

DX = Total amount of free extended memory in K-bytes

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = A0h if all extended memory is allocated

This function returns the size of the largest available extended memory

block in the system.

NOTE: The 64K HMA is not included in the returned value even if it is

not in use.

Allocate Extended Memory Block (Function 09h):

----------------------------------------------

ARGS:   AH = 09h

DX = Amount of extended memory being requested in K-bytes

RETS:   AX = 0001h if the block is allocated, 0000h otherwise

DX = 16-bit handle to the allocated block

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = A0h if all available extended memory is allocated

BL = A1h if all available extended memory handles are in use

This function attempts to allocate a block of the given size out of the

pool of free extended memory.  If a block is available, it is reserved

for the caller and a 16-bit handle to that block is returned.  The handle

should be used in all subsequent extended memory calls.  If no memory was

allocated, the returned handle is null.

NOTE: Extended memory handles are scarce resources.  Programs should

try to allocate as few as possible at any one time.  When all

of a driver's handles are in use, any free extended memory is

unavailable.

Free Extended Memory Block (Function 0Ah):

------------------------------------------

ARGS:   AH = 0Ah

DX = Handle to the allocated block which should be freed

RETS:   AX = 0001h if the block is successfully freed, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = A2h if the handle is invalid

BL = ABh if the handle is locked

This function frees a block of extended memory which was previously

allocated using Function 09h (Allocate Extended Memory Block).  Programs

which allocate extended memory should free their memory blocks before

exiting.  When an extended memory buffer is freed, its handle and all data

stored in it become invalid and should not be accessed.

Move Extended Memory Block (Function 0Bh):

------------------------------------------

ARGS:   AH = 0Bh

DS:SI = Pointer to an Extended Memory Move Structure (see below)

RETS:   AX = 0001h if the move is successful, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = 82h if an A20 error occurs

BL = A3h if the SourceHandle is invalid

BL = A4h if the SourceOffset is invalid

BL = A5h if the DestHandle is invalid

BL = A6h if the DestOffset is invalid

BL = A7h if the Length is invalid

BL = A8h if the move has an invalid overlap

BL = A9h if a parity error occurs

Extended Memory Move Structure Definition:

ExtMemMoveStruct    struc

Length              dd  ?   ; 32-bit number of bytes to transfer

SourceHandle        dw  ?   ; Handle of source block

SourceOffset        dd  ?   ; 32-bit offset into source

DestHandle          dw  ?   ; Handle of destination block

DestOffset          dd  ?   ; 32-bit offset into destination block

ExtMemMoveStruct    ends

This function attempts to transfer a block of data from one location to

another.  It is primarily intended for moving blocks of data between

conventional memory and extended memory, however it can be used for moving

blocks within conventional memory and within extended memory.

NOTE: If SourceHandle is set to 0000h, the SourceOffset is interpreted

as a standard segment:offset pair which refers to memory that is

directly accessible by the processor.  The segment:offset pair

is stored in Intel DWORD notation.  The same is true for DestHandle

and DestOffset.

SourceHandle and DestHandle do not have to refer to locked memory

blocks.

Length must be even.	Although not required, WORD-aligned moves

can be significantly faster on most machines.  DWORD aligned move

can be even faster on 80386 machines.

If the source and destination blocks overlap, only forward moves

(i.e. where the source base is less than the destination base) are

guaranteed to work properly.

Programs should not enable the A20 line before calling this

function.  The state of the A20 line is preserved.

This function is guaranteed to provide a reasonable number of

interrupt windows during long transfers.

Lock Extended Memory Block (Function 0Ch):

------------------------------------------

ARGS:   AH = 0Ch

DX = Extended memory block handle to lock

RETS:   AX = 0001h if the block is locked, 0000h otherwise

DX:BX = 32-bit linear address of the locked block

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = A2h if the handle is invalid

BL = ACh if the block's lock count overflows

BL = ADh if the lock fails

This function locks an extended memory block and returns its base

address as a 32-bit linear address.  Locked memory blocks are guaranteed not

to move.  The 32-bit pointer is only valid while the block is locked.

Locked blocks should be unlocked as soon as possible.

NOTE: A block does not have to be locked before using Function 0Bh (Move

Extended Memory Block).

"Lock counts" are maintained for EMBs.

Unlock Extended Memory Block (Function 0Dh):

--------------------------------------------

ARGS:   AH = 0Dh

DX = Extended memory block handle to unlock

RETS:   AX = 0001h if the block is unlocked, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = A2h if the handle is invalid

BL = AAh if the block is not locked

This function unlocks a locked extended memory block.  Any 32-bit

pointers into the block become invalid and should no longer be used.

Get EMB Handle Information (Function 0Eh):

------------------------------------------

ARGS:   AH = 0Eh

DX = Extended memory block handle

RETS:   AX = 0001h if the block's information is found, 0000h otherwise

BH = The block's lock count

BL = Number of free EMB handles in the system

DX = The block's length in K-bytes

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = A2h if the handle is invalid

This function returns additional information about an extended memory

block to the caller.

NOTE: To get the block's base address, use Function 0Ch (Lock Extended

Memory Block).

Reallocate Extended Memory Block (Function 0Fh):

------------------------------------------------

ARGS:   AH = 0Fh

BX = New size for the extended memory block in K-bytes

DX = Unlocked extended memory block handle to reallocate

RETS:   AX = 0001h if the block is reallocated, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = 81h if a VDISK device is detected

BL = A0h if all available extended memory is allocated

BL = A1h if all available extended memory handles are in use

BL = A2h if the handle is invalid

BL = ABh if the block is locked

This function attempts to reallocate an unlocked extended memory block

so that it becomes the newly specified size.  If the new size is smaller

than the old block's size, all data at the upper end of the old block is

lost.

Request Upper Memory Block (Function 10h):

------------------------------------------

ARGS:   AH = 10h

DX = Size of requested memory block in paragraphs

RETS:   AX = 0001h if the request is granted, 0000h otherwise

BX = Segment number of the upper memory block

If the request is granted,

DX = Actual size of the allocated block in paragraphs

otherwise,

DX = Size of the largest available UMB in paragraphs

ERRS:   BL = 80h if the function is not implemented

BL = B0h if a smaller UMB is available

BL = B1h if no UMBs are available

This function attempts to allocate an upper memory block to the caller.

If the function fails, the size of the largest free UMB is returned in DX.

NOTE: By definition UMBs are located below the 1MB address boundary.

The A20 Line does not need to be enabled before accessing an

allocated UMB.

UMBs are paragraph aligned.

To determine the size of the largest available UMB, attempt to

allocate one with a size of FFFFh.

UMBs are unaffected by EMS calls.

Release Upper Memory Block (Function 11h):

------------------------------------------

ARGS:   AH = 11h

DX = Segment number of the upper memory block

RETS:   AX = 0001h if the block was released, 0000h otherwise

ERRS:   BL = 80h if the function is not implemented

BL = B2h if the UMB segment number is invalid

This function frees a previously allocated upper memory block.  When an

UMB has been released, any code or data stored in it becomes invalid and

should not be accessed.

PRIORITIZING HMA USAGE:

-----------------------

For DOS users to receive the maximum benefit from the High Memory Area,

programs which use the HMA must store as much of their resident code in it as

is possible.  It is very important that developers realize that the HMA is

allocated as a single unit.

For example, a TSR program which grabs the HMA and puts 10K of code into

it may prevent a later TSR from putting 62K into the HMA.  Obviously, regular

DOS programs would have more memory available to them below the 640K line if

the 62K TSR was moved into the HMA instead of the 10K one.

The first method for dealing with conflicts such as this is to require

programs which use the HMA to provide a command line option for disabling

this feature.  It is crucial that TSRs which do not make full use of the HMA

provide such a switch on their own command line (suggested name "/NOHMA").

The second method for optimizing HMA usage is through the /HMAMIN=

parameter on the XMS device driver line.  The number after the parameter

is defined to be the minimum amount of HMA space (in K-bytes) used by any

driver or TSR.	For example, if "DEVICE=HIMEM.SYS /HMAMIN=48" is in a

user's CONFIG.SYS file, only programs which request at least 48K would be

allowed to allocate the HMA.  This number can be adjusted either by

installation programs or by the user himself.  If this parameter is not

specified, the default value of 0 is used causing the HMA to be allocated

on a first come, first served basis.

Note that this problem does not impact application programs.  If the HMA

is available when an application program starts, the application is free to

use as much or as little of the HMA as it wants.  For this reason,

applications should pass FFFFh in DX when calling Function 01h.

HIGH MEMORY AREA RESTRICTIONS:

------------------------------

-   Far pointers to data located in the HMA cannot be passed to DOS.  DOS

normalizes any pointer which is passed into it.  This will cause data

addresses in the HMA to be invalidated.

-   Disk I/O directly into the HMA (via DOS, INT 13h, or otherwise) is not

recommended.

-   Programs, especially drivers and TSRs, which use the HMA *MUST* use

as much of it as possible.  If a driver or TSR is unable to use at

least 90% of the available HMA (typically ~58K), they must provide

a command line switch for overriding HMA usage.  This will allow

the user to configure his machine for optimum use of the HMA.

-   Device drivers and TSRs cannot leave the A20 line permanently turned

on.  Several applications rely on 1MB memory wrap and will overwrite the

HMA if the A20 line is left enabled potentially causing a system crash.

-   Interrupt vectors must not point into the HMA.  This is a result of

the previous restriction.  Note that interrupt vectors can point into

any allocated upper memory blocks however.

ERROR CODE INDEX:

-----------------

If AX=0000h when a function returns and the high bit of BL is set,

BL=80h if the function is not implemented

81h if a VDISK device is detected

82h if an A20 error occurs

8Eh if a general driver error occurs

8Fh if an unrecoverable driver error occurs

90h if the HMA does not exist

91h if the HMA is already in use

92h if DX is less than the /HMAMIN= parameter

93h if the HMA is not allocated

94h if the A20 line is still enabled

A0h if all extended memory is allocated

A1h if all available extended memory handles are in use

A2h if the handle is invalid

A3h if the SourceHandle is invalid

A4h if the SourceOffset is invalid

A5h if the DestHandle is invalid

A6h if the DestOffset is invalid

A7h if the Length is invalid

A8h if the move has an invalid overlap

A9h if a parity error occurs

AAh if the block is not locked

ABh if the block is locked

ACh if the block's lock count overflows

ADh if the lock fails

B0h if a smaller UMB is available

B1h if no UMBs are available

B2h if the UMB segment number is invalid

IMPLEMENTATION NOTES FOR DOS XMS DRIVERS:

-----------------------------------------

-   A DOS XMS driver's control function must begin with code similar to the

following:

XMMControl  proc    far

jmp     short XCControlEntry    ; For "hookability"

nop 			    ; NOTE: The jump must be a short

nop 			    ;  jump to indicate the end of

nop 			    ;  any hook chain.	The nop's

;  allow a far jump to be

;  patched in.

XCControlEntry:

-   XMS drivers must preserve all registers except those containing

returned values across any function call.

-   XMS drivers are required to hook INT 15h and watch for calls to

functions 87h (Block Move) and 88h (Extended Memory Available).  The

INT 15h Block Move function must be hooked so that the state of the A20

line is preserved across the call.	The INT 15h Extended Memory

Available function must be hooked to return 0h to protect the HMA.

-   In order to maintain compatibility with existing device drivers, DOS XMS

drivers must not hook INT 15h until the first non-Version Number call

to the control function is made.

-   XMS drivers are required to check for the presence of drivers which

use the IBM VDISK allocation scheme.  Note that it is not sufficient to

check for VDISK users at installation time but at the time when the HMA

is first allocated.  If a VDISK user is detected, the HMA must not be

allocated.	Microsoft will publish a standard method for detecting

drivers which use the VDISK allocation scheme.

-   XMS drivers which have a fixed number of extended memory handles (most

do) should implement a command line parameter for adjusting that number

(suggested name "/NUMHANDLES=")

-   XMS drivers should make sure that the major DOS version number is

greater than or equal to 3 before installing themselves.

-   UMBs cannot occupy memory addresses that can be banked by EMS 4.0.

EMS 4.0 takes precedence over UMBs for physically addressable memory.

-   All driver functions must be re-entrant.  Care should be taken to not

leave interrupts disabled for long periods of time.

-   Allocation of a zero length extended memory buffer is allowed.  Programs

which hook XMS drivers may need to reserve a handle for private use via

this method.  Programs which hook an XMS driver should pass all requests

for zero length EMBs to the next driver in the chain.

-   Drivers should control the A20 line via an "enable count."	Local En-

able only enables the A20 line if the count is zero.  It then increments

the count.	Local Disable only disables A20 if the count is one.  It

then decrements the count.	Global Enable/Disable keeps a flag which

indicates the state of A20.  They use Local Enable/Disable to actually

change the state.

IMPLEMENTATION NOTES FOR HIMEM.SYS:

-----------------------------------

-   HIMEM.SYS currently supports true AT-compatibles, 386 AT machines, IBM

PS/2s, AT&T 6300 Plus systems and Hewlett Packard Vectras.

-   If HIMEM finds that it cannot properly control the A20 line or if there

is no extended memory available when HIMEM.SYS is invoked, the driver

does not install itself.  HIMEM.SYS displays the message "High Memory

Area Unavailable" when this situation occurs.

-   If HIMEM finds that the A20 line is already enabled when it is invoked,

it will NOT change the A20 line's state.  The assumption is that whoever

enabled it knew what they were doing.  HIMEM.SYS displays the message "A20

Line Permanently Enabled" when this situation occurs.

-   HIMEM.SYS is incompatible with IBM's VDISK.SYS driver and other drivers

which use the VDISK scheme for allocating extended memory.  However,

HIMEM does attempt to detect these drivers and will not allocate the

HMA if one is found.

-   HIMEM.SYS supports the optional "/HMAMIN=" parameter.  The valid values

are decimal numbers between 0 and 63.

-   By default, HIMEM.SYS has 32 extended memory handles available for use.

This number may be adjusted with the "/NUMHANDLES=" parameter.  The

maximum value for this parameter is 128 and the minimum is 0.  Each

handle currently requires 6 bytes of resident space.

Copyright (c) 1988, Microsoft Corporation