V 0.97c

-------

- makes first drive the selected drive when rebooting the oric

- fixes the level sensed on PB0-2, because some folks program these as inputs!

- removes the weak noise heard when no sound is emitted

- allows clock frequency to go up to 99.9 MHz in configuration screen, allows

to switch joystick emulation, fixes the keyboard delay bug.

- disables keyboard switching when Keyboard.def file not present

- repairs the joystick emulation, allowing arrows keys again

V 0.97b

-------

- fixed the behavior of the programmable keyboard (a key could be depressed

too fast, thus be not detected by the oric)

- 3 speeds cursor movements in configuration screen

- alphabetic sort of directory lists

V 0.97a

-------

- repaired dump/restore mechanism

V 0.97

------

- With the addition  of an hardware  tape emulation I'm  proud of, Euphoric is

on the  path to the  final release (last  major addition  should be  digital

sound instead of FM synthesis)

Check  the option  in the configuration  screen to use  this tape simulator,

because  it's off  by default  (mostly, only true  nostalgic people will use

it,  enjoy  for example  Lone Raider 's  loading).  Previous system is still

here for fast  loading and  ease of  use (you don't  have to go  through the

configuration  screen to  select a tape,  you don't have to press the RECord

button  of  the  tape  when  CSAVEing...): it  works  by  patching  selected

locations  in Oric-1  and Atmos  roms. Some programs (few)  don't use at all

the low-level  routines  in rom, so  they require  the use  of the  hardware

emulation (roms are not patched in this mode).

Of  course, a new  tape  image format  is used in the hardware tape mode: it

is simply  a sampled (digitized)  image of real  tapes. Shannon went here so

it is a  4800 Hz sampling rate, 1  bit depth samples.  Right, this format is

harder to hack in, but  it will soon allow you  to transfer programs to your

Oric on real  tapes  (and perhaps  more useful for  most people: easier tape

transfers  from  real  oric,  ability  to  run  protected  software  without

patching them, so you can legally run your old software, ...) !

Note that you  won't be able to load  byte-format tapes in the hardware mode

(once again, format  is different: you  can CLOAD them with default mode and

CSAVE them in hardware mode if you are a real oric fundamentalist 8-)

Also, when  using  hardware  mode,  don't  forget you  *need*  to  press the

RECord  button of  the tape  before  writing to  it, don't  forget you  will

overwrite  the previous  contents of the  tape unless you  position the tape

with  the  fast  rewind  and  fast  forward   buttons  (a  counter  in  real

minutes/seconds is provided, but you will need to write program information,

location, etc.  on a piece of  paper ;-), and  don't forget  to release  the

RECord button after writing...

(that was tape day-to-day life 8-)

If this frightens  you, protect  your valuable tapes  against writing (using

file access modes  under the host). Note  also  there is  no need for a PLAY

button (it is always  pressed), since a  remote control  lead is bundled  in

the package...

Final word,  have no fears about "errors found" messages,  you won't get one

with this highly reliable DAT, and your programs won't fade with time...

- Charset redefined in configuration screen

- And... yes, Vaggelis, I know I still have to repair the dump/restore ;-)

V 0.96

------

- Added a configuration screen to swap disks, keyboards, etc.

- F6 is now the Jasmin's boot button.

V 0.95a

-------

- Last patch for the programmable keyboard 8-) I add a classical reader/writer

synchronization  problem, so the oric  sometimes caught only one of two keys

send  in two  different locations  of the  keyboard matrix (when the routine

has already scanned some of it).

V 0.95

------

- Users of the previous  national keyboard were somewhat disturbed by the fact

some software produced US  strokes, depending on the routines they used. So,

this new programmable keyboard  uses a hardware mapping  between PC keys and

oric keys,  instead of  forcing an ascii  value in the  keyboard  converting

routines of  the different oric roms  (see Keybfr.def file).  Of course, the

original  oric keyboard  is still  there, for  games  that  would  otherwise

oblige painful  positions,  and function  key F3 alternates  between the two

keyboards (help key F1 displays the current keyboard).

Moreover, the  programmable keyboard allows for *fast* typing (by depressing

oric keys when  *you*  are not  fast enough 8-) : now it's time you use your

oric for text-processing !!

- Sorry for the dump/restore I still have not repaired...

- And I've been told  Bill don't want to let me play music, and that his clock

is disturbed by my  higher interrupt frequency.  Ok, everybody will take the

right conclusion (Linuuuuuux... 8-)

V 0.94a

-------

- A fix to  the new national keyboard support introduced in 0.92 (I missed the

point that the CAPS state may be changed by software)

- No time to  fix the dump/restore  feature, broken  with 0.94 again : my hard

disk had a hard failure and its whole content is lost 8-(

- No time to explain to IBM* and Microsoft* windows users  how to benefit from

drag&drop  feature, or  associating '.dsk'  type to the  euphoric program, I

have to completely rewrite the documentation...

V 0.94

------

- Euphoric for  DOS becomes DPMI  compatible. This  means that you  can run it

under near every  DPMI  server  (currently tested:  OS/2 Warp,  Windows 3.1,

Windows 95,  Quaterdeck's QDPMI,  Qualitas' 386MAX,  CWSDPMI). There's about

no performance penalty  with  DPMI  servers under DOS  (Quaterdeck, Qualitas

and  the free CWSDPMI). Euphoric  works great  under OS/2  too.  It  suffers

from  the bad performance of Windows 3.1, it  doesn't work yet under Windows

NT (keyboard events lost). It  works with  Windows 95, but  you surely don't

want to use such a bugged down system.

V 0.93a

-------

- I have  switched to  an ELF distribution on my system, but will  continue to

link a.out euphoric  executables (so you need aout  versions of libc.so  and

libvga.so).

Soon I will get rid of svgalib (I only use it to init screen mode).

DOS executable is  still a  COFF converted  to EXE  (but  you  surely  don't

care 8-)

- Oops, as the  keyboard is  polled in the  linux version,  euphoric was stuck

when the 6502  hung. Mhh, I can restore  the old behavior  (only power reset

was active  in such a case)  but I decided  to introduce a  small difference

with the real  thing (the NMI button is now active), maybe one  day you will

thank me ;-)

- Another keyboard problem corrected, in Dos version (hum, second  time I fall

in the same trap: different scancodes under Linux and Dos)

V 0.93

------

- New display : with the -q option, you will get *rectangular*  pixels like on

your TV. DON'T use on  a LCD matrix. The vga mode  used is called mode Q (it

is  256x256x256, so a  'CUbe') and  should  work  with  every  vga card  and

monitor.

V 0.92

------

- First  support of P.A.S.E  interface  (joystick  on  parallel port)  with -p

option:

arrows on the numeric keypad for directions and 0 for fire button.

- Telestrat users have  the choice to  use the PASE  joystick as above  or the

standard telestrat's joysticks (-tj option)

- New keyboard handler allowing more flexibility and  configurability (but old

international keyboard definitions have to be changed)

- A bug  fixed in linux  version:  keyboard events  were missing in the simple

oric configuration, causing stuck keys.

- Different handling of Jasmin's interrupts, the previous trick (executing one

instruction  after a RTI,  even if an  interrupt  was  pending) caused  some

programs  to randomly fail (eg. Painter). So, now  a virtual timer allows to

reproduce  DRQ interrupts every 31,25�s (approx.), as on the real drive (250

Kbps rate)

V 0.91b

-------

- Oops, I  forgot to  update the  Sector  register  of the  1793  after a Read

Address command.

- ACIA on  the Atmos moved  to address  $031C again  (no, it's not a tango). I

have checked the  address decoding of the real  microdisc allows for such an

address.

V 0.91a

-------

- Simulates the  movement of drive heads, so it is easier to cope with strange

habits like late selecting of drives.

- Simulates the rotation of the diskettes (allowing to read sectors or sector-

ids in a circular  way) Look for example at the interleaving (1:2) of RANDOS

diskettes (track 3 and above).

V 0.91

------

- Changed the FDC 1793  emulator to  handle a new  disk image format (really a

bit  copy of a  real disk),  this will  allow  every exotic  format (eg. the

Randos  system formats the  first three tracks  with 16 sectors (256 bytes),

and the rest of the disk with 9 sectors (512 bytes)).

The  bad news is  your disk  images are  bigger (every  track is 6400 bytes,

with  datas, sector  id's, gaps,  indexes,...)  and you have  to update your

disk  images with  the oric2mfm  utility  (sorry for that),  but  this  will

soon bring great rewards for both virtual and real oric users 8-)

(anticipating,  you can already examine  M$-DOS disks with  Nibble, like  on

the real Oric: use  the dos2mfm utility  to convert a  720K M$-DOS raw image

to the  new mfm  format. M$-DOS  raw images  are easily  obtained under Unix

with the standard dd command, or under M$-DOS with the rawread utility)

V 0.9a

------

- Oops, I forgot to correct  the serial interrupts  under Linux. Hope  it's ok

now

V 0.9 (29/10/95)

-----

- Oric goes out with his girlfriend Jasmin again 8-)

(FDC 1793 simulator separated from Microdisc electronics, Jasmin electronics

added,  multiple sector read  command added,  delayed INTRQ signal simulated

differently,  artificially delayed  IRQ detection  after RTI instruction  to

allow for delayed DRQ signal)

- Setting X  register to  0 in the  patch of  the  read  tape  synchronization

routine (coming soon : hardware  simulator for tape  loading time nostalgics

;-)

V 0.8b

------

- Added  additional  cycle on  absolute  indexed  and  indirect  indexed  read

addressing modes, and branch instructions, when crossing a page boundary.

Timings should now be exactly similar than on a real 6502.

- Improved  the B flag  implementation  (still have to better implement  N and

V flags in decimal mode).

V 0.8a

------

- Oops, I forgot to modify the Real Time Clock module, sorry...

V 0.8 (386) (01/10/95)

-----

- At  last ! A cross-over of 0.7alpha and 0.7h achieving the speed of versions

anterior to the introduction of the disk controller. Owners  of fast 386 are

able to enjoy it (and now... rushing to the 286 ;-)

- PS/2 compatible

V 0.7h (486)

------

- Serial  interrupts are  in ! The 6551  ACIA emulation  is now  complete, the

communication softwares of the Telestrat run identical to the original.

V 0.7g (486)

------

- Move the IO address of the RS232 interface on the Oric 1 and Atmos.

V 0.7f (486)

------

- Patch for international keyboards on the Telestrat.

- Fix  the  RS232  interface,  it  works  in  polling  mode...(interrupts  not

implemented yet).

V 0.7e (486)

------

- Repair the broken dump/restore mechanism.

- Generously  give a  RS232  interface  to Oric1/Atmos  users (but I  have not

tested it yet), and a real-time clock extension.

V 0.7d (486)

------

- Oops, I forgot to copy files from Linux to Dos, so neither the screen aspect

nor the F7/F8 keys were corrected under Dos...

- First emulation of the real time clock extension (with Telestrat for now)

- Updates the Bios timer when exiting from the Dos version (no need for such a

hassle with unix of course. Hey, have you tried Linux ?)

V 0.7c (486)

------

- Restores the correct  aspect of the screen (during a power  reset) lost with

0.7

- Handles differently the F7 and F8 key for the Telestrat.

- Modification  of  the FDC  1793  module  to work  around a  nasty  behaviour

of Stratsed: selecting  a drive  AFTER  sending a read or  write command and

BEFORE the actual transfer (not always the case, so... a bug ?)

- First emulation of the ACIA, but still lacks interrupts handling.

V 0.7b (486)

------

- New 6502 emulator,  I have made  bencharks and  it is faster thanks to fewer

cache misses (despite additional jumps)

V 0.7a (from 0.6e)

------

- Faster clock code and keyboard reading.

- First TeleStrat  emulation ! Partial  emulation of the  second  VIA, no ACIA

emulation yet (will come quite soon... 8-)

V 0.7alpha

----------

- Divergent release...no bank  switching mechanism  (so no disk system), but a

new  6502 emulation  optimized  for 386,  a faster  keyboard  reading, and a

faster  clock code. Should be the base for a  complete release optimized for

386, slow with  bank switching (like 0.6alpha1)  but fast anywhere  else (to

do: a modified Sedoric with no programmed function keys).

V 0.6e

------

- Monochrome VGA option added (tell me if you don't like the shades)

- Linux Ctrl-C not caught by svgalib any more, thanks to Alexios Chouchoulas.

V 0.6d

------

- Can now hardcopy several screens during a single session.

- Oops, a bug fixed (introduced with 0.6a, affecting the CSAVE command)

V 0.6c

------

- Can now format virtual disks.

V 0.6b

------

- Corrected  lost interrupts with the  DOS version (when go32 switches to real

mode for disk access)

V 0.6a (19/06/95)

------

- New 6502 module with integrated paged memory mechanism.

- Optimized VIA 6522 emulation.

- New disk image format.

- Up to four drives.

- translation of some DOS scan codes to Linux corresponding ones.

- Hardcopy (PrtSc key)

V 0.6 alpha3

------------

- Fix  the deplacement  of the drive's head. Still not have time  to add track

and multiple sectors read/write, though.

V 0.6 alpha2

------------

- Faster memory switching mechanism (approximative also). Final release should

bring  the memory  scheme  in the heart of  the 6502  emulator  in order  to

optimize the whole thing. Notice also the scan codes are sometimes different

under Linux  and  Dos,  it works  ok with  Linux, but  under  Dos, AltGr  is

interpreted like the Alt key (Oric's FCT key).

V 0.6 alpha1

------------

- First floppy disk  controler  emulation, it  works... but the  memory switch

mechanism  is far  too  sloooow. Why  don't  they stop switching,  even when

waiting for user interaction ?

V 0.5f

------

- Oops, didn't notice I deleted one line. Problem with the AltGr key.

V 0.5e

------

- Oops, two bugs fixed (one in the state restore, one in national keyboard for

oric1)

V 0.5d

------

- Dump/Restore added. F9 gives the opportunity to save the state of  the oric,

and the -r parameter  on the command line  allows to restore this state. See

manual for applications.

V 0.5c

------

- Let's say  that 0.5b was so similar to the  Oric that even the tape routines

were so slow...

V 0.5b

------

- Add the behaviour of the oric when it hangs up. The only solution then is to

unplug and plug again the power...

V 0.5a

------

- Add a on/off switch for sound (F2)

V 0.5

-----

- International keyboards supported (look at the manual).

- Speed index  argument for 386 owners who don't have  the processing power to

run the emulator  at nominal speed (or  bigger cpu owners  who want to try a

speedy oric).

- ORIC environment variable to ease execution from several directories.

- Better error messages for first time Linux users.

- New documentation.

- A small bug corrected in PSG emulation, triggered by DPTLQ.

V 0.4c

------

- Change  in file  handling : it is now  possible to save a file  without name

(CSAVE"")  This is not  recommended but  some programs use this feature. The

file gets the name "________" on the host.

V 0.4b

------

- All BIOSes  don't use the  same order for Red/Green/Blue...so, don't use the

Bios anymore.

- Printer support added by default. File "Printer.out" is the result.

V 0.4a

------

- Oops, I must have been in a hurry with V0.4 : a bug in  the frequency of the

sound, and in the  noise too... I still need  to find a way to  make  square

waveforms with the adlib, and noise could be better.

- Have improved the screen mode changes by examining the real oric.

V 0.4

-----

- At last some sound ! Many thanks to Gerton Lunter who  sent me his emulation

code  of the  AY-3-8912 (the Spectrum  128 has the same  chip), from which I

have been inspired a lot.

V 0.3d

------

- Unofficial MOS 6502 opcodes added. This allows some programs to run. I don't

have time to look if they use them or if they have bugs...

V 0.3c

------

- A modification to the display code and the synchronization, so  that smaller

CPUs  (486/25 and less) have  a better screen  refresh. Animation  is not so

jerky on these CPUs now.

V 0.3b

------

- Two  gadgets added : a  help line on  top of the screen (de)activated by F1,

and a simulation of the screen aspect when powering on the oric...

V 0.3a

------

- Small detail corrected in PSG (but still no sound yet)

- Thiner emulation of  the tape, so that programs like  Lone Raider or Trouble

In Store can load.

V 0.3 (14/05/95)

----------------

- Complete  rewrite of the display code (it's mode X now). Synchronization  is

far better, the speed should be the same on every machine. Powerful machines

will just  benefit of  more screen  refresh  (up to 50 per second).  It also

allows me to get rid of GASP, so...this is a wonderful world !

- Partial rewrite of 6522 emulation : cleaner now and (almost) exhaustive...

V 0.2.2f

--------

- Change of the tape  handling for two reasons : it's easier now to use multi-

parts programs, and the emulator can read JF.Fabre emulator's format.

V 0.2.2e

--------

- Rewrite the  6502  emulator  for cpp  preprocessor. When  I change  the  VGA

code, I will throw GASP...

- Clock count of the NOP instruction (Oops, had forgot it...)

- Emulation of reset button and power on/off reset.

- For DOS  version, disable keyboard interrupt during disk access in real mode

(the ENTER key looked sometimes pressed after such access)

V 0.2.2d

--------

- Correction of the clocks count of conditional branching instructions.

V 0.2.2c

--------

- Bug fixes.  Oric1 rom was not  patched correctly to allow the CLOAD command.

BRK instruction was not correctly emulated.

V 0.2.2b

--------

- Correction  of a little bug :  stores now the oric name of the  file read by

CLOAD, in order  that the top  line displays it  correctly. I still  have to

test if CLOAD and  CSAVE work  with the Oric1 rom, since  I write the  patch

of that rom just with a listing of it.

V 0.2.2a

--------

- Near completion of the VIA emulator. A quite correct  handling of port A now

allows programs to  detect that no joystick  is connected. Timers are now as

precise as with the real oric.

V 0.2.2  (05/03/95)

-------

- Port of the V 0.2.1 to the linux i386 target. Why ?

Previous  versions have  been developed with PharLap  DOS Extender, a really

good product,  but I  would have  to pay  one more  time to  distribute  the

emulator  since I don't  have the runtime  kit, and the development  kit was

enough  expensive... So I decided  to use DJ Delorie  Dos Extender, and  his

port of  the GNU  environment. This  has the advantage  to allow a  DOS/Unix

version  quite  similar,  but  the  port has  been very  painful due  to the

lack of  support  for  assembly programmers,  and the AT&T  assembly  syntax

different  from  the  Intel  one,  not to  mention the  bugs  of  GASP,  the

assembly  preprocessor,  nor the  very  approximative  checking of  GAS.  So

bugs introduced by the port might still hide somewhere...

V 0.2.1  (25/02/95)

-------

- Speed identical to the real oric, whatever CPU you have (min 486SX/25 ?)

- BCD computations emulated

- A bug corrected with the correct handling of the IER register  (VIA). Before

that,  a timer  interrupt  rising  before  the  initialization  of  the  IRQ

sometimes happened.

- A patch to the  handling of port A in order  that  every program succeed  in

reading the keyboard, since some (incorrectly) use the acknowledged port.

V 0.2  (12/02/95)

-----

- Loads and runs an Atmos rom (in fact, my 1.3 modified one)

- patches  the rom to  allow CLOAD and  CSAVE (I still have to  test STORE and

RECALL)

- Displays  Text mode with  background and  foreground attributes,  simple and

double height,  normal and alternate charset, and reverse video (blinking is

missing).  Identical to  the real  oric, at the pixel  level, and allows the

redefinition of character shapes (but it does not affect  characters already

on screen, so Invaders* lacks some animation)

- Displays  Hires mode  with 3 text  lines (I have to figure out  what the ULA

exactly do to  allow semi-hires  modes like the one in Doggy*), and  handles

the switch to/from Text mode.

- Basic emulation of the 6522 VIA : PC timer mimics the  standard 100 Hz timer

1, basic  emulation  of  port B  and  PSG in  order to  allow  the  keyboard

simulation.

- Lacks BCD computations, so some programs don't display correctly the score.

V 0.1  (29/01/95)

-----

- A very  approximative display with 80x25 PC  standard mode, but it  works...

the rom  runs until the  Ready prompt  after having  patched the  wait for 3

interrupts to switch to text mode.

- A bug fixed  at 4 o'clock : an ADC was executed  instead of  each indirect X

indexed ROR, very annoying for the floating point computations (so you learn

that the Oric has "<58225.792 BYTES FREE" ...BTW, incredible to use floating

point instead of integer computations)

- No  emulation  of  the  VIA yet : no  timer, no  keyboard (the  PC  keyboard

interrupt  writes  the  ascii code in  the oric Basic's  keyboard  buffer. I

remember  also  another  night(mare) hunting  for  a  bug  in  this  routine

affecting the oric stack register, ouch...)

#�