A.S.P. Version 4.00 (C) 1989-1996

Amplifier Simulation Program

Written by KD9JQ

Charles H. Reichert

955 Concord Lane   Hoffman Estates, Il. 60195

All Rights Reserved

THEORY OVERVIEW

This Program is a result of finding an "easier" way to Design RF Amplifiers

by a person with little or no Engineering background or the "HomeBrewer".

Circuit Modeling for Lowest Noise Figure usually requires knowing the

devices internal physical structure, or at the least the optimum Source

Reflection Coefficient for Lowest Noise.  Unfortunately, this data is not

easily available.

The aim in developing this particular program is to use S Parameters and

then convert them to Y Parameters which are now generally available from

the Vender Data sheets.  From these Y Parameters equations for optimizing

the Source Impedance (for FET's) have been developed and tested out on

numerous test Pre-Amps with results well within expected tolerances of

components and measurement tolerances excluding board layout variations.

In the case of Bipolar devices Standard Y Parameter Design Equations are

used. The measured Un-optimized Noise Figure at some Frequency in a 50 Ohm

system must be known for Bipolar Circuit Optimization.

This data is used to calculate the Base Spreading Resistance at the

operating frequency and then solving for the optimum Source Resistance and

Noise Figure.

BACKGROUND

In the early 70's, VHF/UHF Transcievers were just coming into their own.

Unfortunately many of the transceivers then and even today lack adequate

IF gain for weak signal work.  In order to make up for this inadequacy,

many Hams added one or two preamps to the front end.  This started many a

knock down fight over the air on how an other local ham's "dirty signal"

was all over the band.

This problem of course is simply strong signal overload causing intermods

in the front end, but trying to explain this to another Ham who just plucked

down hard cash for his super duper,high gain, low noise preamp just didn't

always work!

I've noticed a number of problems associated with adding preamps to the

front end which are as follows;

Pg. 2

1. Excessive Preamp gain will overload the next stage and/or mixer causing

intermodulation distortion.  Unfortunately many commerial rigs still don't

use high signal handling front ends and mixers, along with proper impedance

terminations.

2. Many amplifiers designed for gain right at GMAX (Max unneutralized gain)

often leave very little room for mismatched load/source conditions.

3. Receiver front ends rely on a 50 ohm source for low noise source

impedance transformation, but many preamps do not act as a 50 ohm source to

the receiver, therefore instability may result.

4. On the other end, Preamps often rely on a 50 ohm resistive load for

proper operation.  Since receiver front ends seldom represent a 50 ohm load,

instability may again result.

5. Preamps are usually designed for maximum gain and minimum noise figure,

which means they are not necessarily being fully utilized for strong signal

handling.

6. The receiver AGC loop may not be adequate in some cases.  This is

evidenced by S-meter readings above S-0 not caused by instabilities.

If the AGC detector is of an RF type, then the AGC threshold may be crossed

causing premature AGC'ing.  The AGC detector AGC point should be

re-adjusted.  This is not practical except to people who like modifing

equipment or homebrewing. This in itself is not a Preamp problem but shows

an additional consequence of adding too much gain to the front end.

Designing an amplifier to meet all these conditions requires a lot of time

consuming math or luck!  Many Preamp "Contesters" have access to Noise

measuring equipment and "hand pick" devices for low noise.  Since

these amplifiers are "tweaked" to their max in a 50 ohm system, they may

or may not work properly in actual operating systems.

My goal was to work up a simple, interactive computer program to be used in

aiding, and at the same time, provide insight into amplifier design prior to

the actual fabrication of the circuit.

After researching numerous reference sources it became evident that most of

the modeling of devices was by way of its internal physical structure which

is not very practical to the average amateur.  Besides, most of the required

information is not readily available.  What is available are the

Manufacturer's Datasheets containing S-parameters and DC operating

conditions which can be utilized.

I have found that converting the S-parameters to Y-Parameters provides the

easiest way of working with Transistor Narrow Band Amplifier Design.

It is easier for the Lay Person to visualize parallel sources and loads

(Real,Imaginary) than trying to work with Stability Circles, Smith Charts,

etc.

Pg. 3

FET OPTIMUM SOURCE EQUATIONS

S to Y Parameter conversion:

Y11 = [((1+S22)(1-S11)+S12S21)/D]*1/ZO  =  (g11 + b11)       EQ.1

where D=((1+S11)(1+S22)-S12S21)  and ZO = 50 for 50 ohm system

Y12 = [(-2S12)/D]*1/ZO  =  (g12 + b12)                       EQ.2

Y21 = [(-2S21)/D]*1/ZO  =  (g21 + b21)                       EQ.3

Y22 = [((1+S11)(1-S22)+S12S21)/D]*1/ZO  =  (g22 + b22)       EQ.4

where gxx = Real part of Yxx and Bxx = Imaginary part of Yxx

As one can see, this is a time consuming procedure easily processed by a

computer, as are many of the other equations yet to be covered.  Once the

parameters are converted we can proceed with the design.

The question "where to start?" has probably entered your mind.  I found by

picking a resistive load and starting from there works best.  But what value

should I use?  Since we're looking for an amplifier capable of using its

full output at a particular bias point then one can set the real part of

the load YL to the following criteria;

GL = Pi*ID/VP^2  where ID = .18*IDSS  &  VP=.8*VDS           EQ.5

By initially setting the Imaginary portion of the Load (BL) to -B22

(assuming reactive match) we then can solve for Yin where;

Yin = Y11 - (Y12Y21)/(Y22+YL)  =  (Gin + Bin)                EQ.6

and YL =  (GL + BL)                  EQ.7

If the real part of the input admittance turns out to be negative for this

load, then the load resistance is automatically decreased in the program

until a positive real part is obtained.

The problem is determining the optimum source resistance for a particular

device.  Fortunately enough information is available to make use of

Y Parameters.

If we know Gin, then the source resistance (Rx) can be optimized for minimum

noise for this particular input admittance.  I use a technique that looks

similar to quarter wave stub matching of two impeadances.  In this case the

Impedances are the 50 ohm line and 1/Gin  (Rin).

There is an optimum transformation ratio that occurs when Rx/Rs = Rin/Rx.

This ratio occurs at a Q that minimizes input circuit losses due to the

tranformation ratio (not to be confused with the Inductor loaded Q) and is

defined by;

Q = Rx^2 - Rs^2/2*Rx*Rs = Ri^2 - Rx^2/2*Ri*Rx                EQ.8

where:  Rs = 50 ohms   &   Rin = 1/Gin

Now solving for Rx we get the familiar equation;

Rx = SQRT(Rs*Ri)                                             EQ.9

Pg. 4

Now we add the Noise Generator Resistance of the Device known as Rn.

where Rn = k/GM  &  k = .8    for MESFETS / GaAsFETs    (1 Micron Gates)

k = .67   for JFETS

k = 1.25  for 1 or 2 Channel FETS

GM = The Real part of Y21

From the model I set up the following conditions for noise contributions

(Assuming Room Temperature) with the following equation:

Let F = 1 + A + B + C                                       EQ.10

where A = (Rs+Rn)/Rx        (Loss's due to Noise Sources)

B = Rx/(Ri+Rn)        (Loss's due to Mismatch)

C = Rx/(Rp+Rx)        (Input Inductor Losses)

F = Noise Factor

and  Rs = 50 ohm Source Noise Generator

Rx = Source Transformed from Rs

Rp = Inductor loss's

Ri = Parallel Input Resistance of Device

Rn = Equivalent Device Noise Resistance

Setting F = 1 and assuming Rn << Ri, Rp --> infinity and taking the

input admittances magnitude then EQ.10 then reduces to;

(Rs+Rn)/Rx + Rx/Ri = 0  where Rx/Ri now = B (EQ.10)         EQ.11

Therefore:

|Rx/(Rs+Rn)| = |Ri/Rx|                                      EQ.12

Solving for Rx(opt) yields;

Rx(opt) = SQRT[(Rs+Rn)*Ri]                                  EQ.13

From the model we can see the Noise resistance is a portion of the input

admittance.  From Eq.11 we see that the larger Ri is to Rx and

Rx is to (Rn + Rs), the less noise added to the input.

Once the optimized Source Resistance has been found, we then set Ys to;

Ys = (1/Rx + -Bin)                                          EQ.14

Now Yout = Y22 - (Y12*Y21)/(Y11+Ys)                         EQ.15

Now   YL = (GL + -Bout)                                     EQ.16

The process is now repeated with the new YL.  I've been able to quickly

reduce this process to three iterations for approximate designs, although

it is quite tedious by hand, even with a calculator.  A faster way of course

is by using a computer program to handle the calculations.

While this is going on, the program automatically adjusts the output Shunt

resistance to provide stable operation and a low output VSWR to the next

stage.

Pg. 5

Let:    RAC = 1/GL  (Initial Condition)

where   RSH = Shunt Resistor

RPO = Output Inductor Loss's

RL' = Transformed Load from 50 ohms

RO = ABS(1 / Gout)

Therefore:

RSH = 1/((1/RAC)-(1/RO)-(2/RPO))/2                         EQ.17

RAC' = 1/((1/RL')+(1/RPO)+(1/RSH))                          EQ.18

RL' = 1/((1/RO)+(1/RSH)+(1/RPO))                           EQ.19

These equations have been the basis used in developing the program and

do not reflect all equations and optimization routines used.

This brief description mainly explains the Theory used to arrive at a

simpler method of deriving the optimum Noise for a FET device.

The equations do not account for the effect of low frequency FET operation

on noise figure due to Gate Widths.  To date no significant errors have

been noted.  This is most likely do to the fact that the S-Parameters are

measured at the operating frequency and include the Gate Width effect.

Pg. 6

REFERENCES

REF1    MICROWAVES & RF, FEBRUARY 1986

Matching Techniques Yield Optimum LNA Performance

Pgs. 87-90, By Jack D. Sifri

REF2    DESIGNING WITH FIELD-EFFECT TRANSISTORS

Siliconix Inc.  McGraw-Hill 1981    Pg. 190

REF3    MOTOROLA RF DATA MANUAL

Second Edition 1981  Pgs. 17-152 to 17-163

REF4    SOLID STATE RADIO ENGINEERING

(C) 1980 John Wiley & Sons, INC.  Authors Krauss, Bostian, Raab

REF5    HIGH FREQUENCY AMPLIFIERS

(C) 1975 John Wiley & Sons, INC.  by Ralph S. Carson

REF6    IEEE ELECTRON DEVICES, VOL. ED-26

Pgs. 1032-1037, JULY 1979

REF7    IEEE TRANS. MICROWAVE THEORY AND TECH., VOL. MTT-24

Pgs. 274-300, JUNE 1976

REF8    IEEE J. SOLID STATE CIRCUITS, VOL. SC-11

Pgs. 243-255, APRIL 1976

REF9    IEEE TRANS. MICROWAVE THEORY AND TECH., VOL. MTT-23

Pgs. 461-469, JUNE 1975