Synchronet Callback v1.10   Copyright 1993 Digital Dynamics

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Callback Verification Program for Synchronet Multinode BBS Software

Installation:

============

1. Create a directory called "SCB" off of your Synchronet XTRN directory.

Example:

MD C:\SBBS\XTRN\SCB

2. Copy all SCB files into this new directory.

Example:

COPY *.* C:\SBBS\XTRN\SCB

Note, SCB files include:

SCB.DOC 	This file

ORDER.TXT	Purchase order form

SCB.EXE 	Callback verifier external program

SCBCFG.EXE	Sysop configuration program for SCB

ALLOWED.DAT	List of allowable number prefixes

LDPREFIX.DAT	List of same-area prefixes that are long dist

SCB.MSG 	The opening screen for SCB

REFUSED.MSG	Verification refused

INSTRUCT.MSG	Instructions

NO_ONE.MSG	Long distance starting with 1 not allowed

NO_ZERO.MSG	Long distance starting with 0 not allowed

LD_TIME.MSG	Not within long distance callback time range

TOOSHORT.MSG	Phone number less than minimum number of digits

TOOLONG.MSG	Phone number more than maximum number of digits

PHONECAN.MSG	Phone number found in trash can file

VERIFIED.MSG	Displayed to users after verification

3. Run SCFG from your NODE1 directory.

4. Go to the External Programs->Online Programs menu.

5. Hit the <INSERT> key and enter "Synchronet Callback"

6. Hit <ENTER> to edit the configuration for this new program.

7. Set the "Start-up Directory" to "..\XTRN\SCB".

8. Set "Multiuser" to "Yes".

9. Set "Modify Uesr Data" to "Yes".

10. Set "BBS Drop File Type" to "Synchronet XTRN.DAT"

11. If you wish to have the SCB run automatically for new users or as a logon

event for unvalidated users, set the "Execute on Event" option to the

appropriate event type. Use "Access Requirements" if you wish SCB to be

only available to unvalidated users (i.e. "LEVEL NOT 20").

Example SCFG screen:

�[�][?]���������������������������������������������������ͻ

�		     Synchronet Callback		   �

����������������������������������������������������������͹

� �Name 		      Synchronet Callback	   �

� �Start-up Directory	      ..\XTRN\SCB		   �

� �Command Line 	      scb			   �

� �Clean-up Command Line				   �

� �Cost in Credits	      0 			   �

� �Access Requirements	      LEVEL NOT 20		   �

� �Execution Requirements				   �

� �ANSI Required	      No			   �

� �Multiuser		      Yes			   �

� �Intercept I/O Interrupts   No			   �

� �Shrink BBS		      No			   �

� �Modify User Data	      Yes			   �

� �Uses WWIV Color Codes      No			   �

� �Execute on Event	      Logon			   �

� �Menu Command Keys	      Main ( )	      Transfer ( ) �

� �BBS Drop File Type	      Synchronet      XTRN.DAT	   �

� �Place Drop File In	      Node Directory		   �

����������������������������������������������������������ͼ

Configuration:

=============

1. Go into your SCB directory and run SCBCFG.

Example:

CD \SBBS\XTRN\SCB

SCBCFG

2. If your Synchronet text directory is in a directory or drive other

than "\SBBS\TEXT", change the Phone Can and Validated Phone List options

to reflect the correct location of your PHONE.CAN file.

If you want to allow duplicate validated phone numbers, change the

Validated Phone List to another path and/or filename.

The file list(s) use the same syntax for trashcan files as Synchronet.

Appending a '~' to a line specifies that the string cannot be located

anywhere within the phone number. Appending a '^' indicates that the

phone number cannot begin with the string.

3. SCB uses the dial string specified in the modem configuration in SCFG

for each node for dialing the remote user's modem. The default dial

string is "ATDT".

4. Use the "Toggle Options" sub-menu to configure your preferences.

5. Use the "Validation Values" sub-menu to configure how you want validated

users' accounts to be modified.

Note: The DEMO version will only set new security level and add credits.

The other validation options are only available in the registered

version.

6. Use the "Edit Allowed Prefixes" sub-menu to set which prefixes will NOT

be affected by long distance restrictions (such as long distance calling

times, and numbers starting with 0 or 1).

7. Use the "Long Distance Calling Times" sub-menu to set the times on each day

when SCB is allowed to place long distance calls.

8. Hit ESC from the main SCB configuration menu and select "Yes" to save

the configuration file.

Notes:

=====

All SCB activity is logged to the file SCB.LOG in the SCB directory in

the format:

Node 7	     : Mon Sep 27 16:44:31 1993

User Name    : Digital Man

Voice Number : 714-529-6328

Modem Number : 5295313

Result	     : Verified

Results 	Description

------- 	-----------

Hung up 	User hung up before or during validation process

Refused 	User refused validation

Long Dist	User is a long distance call and long distance disallowed

Invalid #	User entered a number that is in trashcan file, less than 6

digits, or begins with a disallowed number (0 or 1)

No Init 	Modem could not be initialized

No Connect	Modem could not connect to user's modem (possibly wrong #)

Verified	User was called, modem connected, and password verified

Bad Pass	User was called, modem connected, and incorrect password

Customizations:

==============

The following files can be modified (optionally using Ctrl-A codes for color):

SCB.MSG 	The opening screen for SCB

REFUSED.MSG	Verification refused

INSTRUCT.MSG	Instructions

NO_ONE.MSG	Long distance numbers starting with 1 not allowed

NO_ZERO.MSG	Long distance numbers starting with 0 not allowed

LD_TIME.MSG	Not within allowed long distance callback time range

TOOSHORT.MSG	Phone number is less than configured minimum number of digits

PHONECAN.MSG	Phone number found in trash can file

VERIFIED.MSG	Displayed to users after verification

Registration:

============

If you wish to order a registered copy of SCB at the same time as Synchronet,

use the Synchronet ORDER.TXT file (in the SBBS\DOCS directory).

If registering SCB after already purchasing a copy of Synchronet, call

Digital Dynamics voice at 714-529-6328, Support BBS at 714-529-5313 (and use

Domain Shopper), or complete the ORDER.TXT file in the SCB directory and mail

or fax it to Digital Dynamics.

/* End of SCB.DOC */